| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Control messages not applying ASN.1 encoding (16.2.3) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Youngbin Chang, Rakesh Taori, Anil Agiwal, , Jungje Son Samsung Electronics | yb.chang@samsung.com,, anilag@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-10/0032, IEEE 802.16 Working Group Letter Ballot Recirc #31a / Topic: Control message (Section 16.2.3) |  |
| Abstract | This contribution proposes the MAC control messages not applying ASN.1 encoding |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D6. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Control messages not applying ASN.1 encoding (16.2.3)

Youngbin Chang, Rakesh Taori, Anil Agiwal, Jungje Son

Samsung Electronics

# Introduction

In the IEEE P802.16m/D5 [1], all MAC control message are encoded using ASN.1. In the implementation perspective, messages related to call flow and Software can be coded using ASN.1, but certain messages that need to be processed by DSP/Modem should not be coded ASN.1 because of absence of ASN.1 encoding library in the DSP/Modem in most commercial products. As a reference, it is noted that even in 3GPP LTE, only RRC messages are encoded using ASN.1, but RLC/MAC control signaling does not encoded by ASN.1.

In this contribution, we proposed a list of messages, related to PHY control and ARQ message, that shall not be implemented to ASN.1.

<Proposed solution :>

- Define 1 bit for indication of ASN.1 encoding and simplifying the MCEH removing SN indicator to make 2 byte aligned information

- Define messages which are applied to ASN.1 or not in the MAC control message table.

# Proposed text

[Change1: Modify the table 668, page 59 as follows]

Table 668: MCEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MCEH (){ |  |  |
| Type | 4 | MCEH Type |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Control Connection Channel ID (CCC ID) | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| ASN_1 encoding indicator | 1 | 0: payload is plain text 1: payload is encoded by ASN.1 |
| If ( SN Indicator == 0) |  |  |
| { |  |  |
| Reserved | 51 | For byte alignment |
| }else |  |  |
| { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see Table 664) |
| SN | 86 | Payload sequence number, SN is maintained per CCC ID, the ‘SN’ value increments by one (modulo 64) sequentially. |
| reserved | 2 | For byte alignment |
| } |  |  |
| } |  |  |

[Change2: In the table 670, page 61 for MAEH format, change the size of ACK_SN field to 6 bits.]

[Change 3: In the table 722, page 164 for AAI_MSG_ACK format, change the size of SN field to 6 bits.]

[Change 4: In the table 722, page 164 for AAI_MSG_ACK format, change the size of SN field to 6 bits.]

[Change 5: In the table 722, page 164 for AAI_MSG_ACK format, change the size of SN field to 6 bits.]

[Change 6: In the table 722, page 164 for AAI_MSG_ACK format, change the size of SN field to 6 bits.]

Table 679 – MAC Control Messages

| No. | Functional Areas | Message names | Message description | Security | Connection | Encoding |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | System Information | AAI_SCD | System configuration descriptor | N.A. | Broadcast | ASN.1 |
| 2 |  | AAI_SII-ADV | Service Identity Information Advertisement | N.A. | Broadcast | ASN.1 |
| 3 |  | AAI_ULPC_NI | UL Noise and Interference Level Broadcast | N.A. | Broadcast | ASN.1 |
| 4 |  | AAI-LBS-ADV | LBS Advertisement | N.A. | Broadcast | ASN.1 |
| 5 | Network Entry / Re-entry | AAI_RNG-REQ | Ranging Request | Null: during ranging procedure when there is no SA already established or pre-updated. CMAC: all other cases | Initial Ranging or Unicast | ASN.1 |
| 6 |  | AAI_RNG-RSP | Ranging Response | Null: during ranging procedure when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases in response to the AAI_RNG-REQ message | Initial Ranging or Unicast | ASN.1 |
| 7 |  | AAI_RNG-ACK | Aggregated CDMA Ranging Acknowledge | N/A:in broadcast null :in unicast when primary SA is not established Encrypted/ICV :in unicast when primary SA is established | Broadcast/ Unicast | Plain text |
| 8 |  | AAI_RNG-CFM | Ranging Confirmation | Encrypted/ICV | Unicast | ASN.1 |
| 9 |  | AAI_REG-REQ | Registration Request | Encrypted/ICV | Unicast | ASN.1 |
| 10 |  | AAI_REG-RSP | Registration Response | Encrypted/ICV | Unicast | ASN.1 |
| 11 |  | AAI_SBC-REQ | Basic Capability Request | null: during capability negotiation when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases | Unicast | ASN.1 |
| 12 |  | AAI_SBC-RSP | Basic Capability Response | null: during capability negotiation when there is no primary SA already established or pre-updated. Encrypted/ICV: all other cases | Unicast | ASN.1 |
| 13 | Network Exit | AAI_DREG-REQ | Deregistration Request | Encrypted/ICV | Unicast | ASN.1 |
| 14 |  | AAI_DREG-RSP | Deregistration Response | Encrypted/ICV | Unicast | ASN.1 |
| 15 | Connection Management | AAI_DSA-REQ | Dynamic Service Addition Request | Encrypted/ICV | Unicast | ASN.1 |
| 16 |  | AAI_DSA-RSP | Dynamic Service Addition Response | Encrypted/ICV | Unicast | ASN.1 |
| 17 |  | AAI_DSA-ACK | Dynamic Service Addition Acknowledge | Encrypted/ICV | Unicast | ASN.1 |
| 18 |  | AAI_DSC-REQ | Dynamic Service Change Request | Encrypted/ICV | Unicast | ASN.1 |
| 19 |  | AAI_DSC-RSP | Dynamic Service Change Response | Encrypted/ICV | Unicast | ASN.1 |
| 20 |  | AAI_DSC-ACK | Dynamic Service Change Acknowledge | Encrypted/ICV | Unicast | ASN.1 |
| 21 |  | AAI_DSD-REQ | Dynamic Service Deletion Request | Encrypted/ICV | Unicast | ASN.1 |
| 22 |  | AAI_DSD-RSP | Dynamic Service Deletion Response | Encrypted/ICV | Unicast | ASN.1 |
| 23 |  | AAI_GRP-CFG | Group Configuration | Encrypted/ICV | Unicast | ASN.1 |
| 24 | Security | AAI_PKM-REQ | Privacy Key Management Request | before AK is derived at network entry: NULL after AK is derived at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived at network entry and the other message is enclosed: CMAC | Unicast | ASN.1 |
| 25 |  | AAI_PKM-RSP | Privacy Key Management Response | before AK is derived at network entry: NULL at network entry and EAP-transfer message is enclosed: encryption/ICV after AK is derived after AK is derived at network entry and the other message is enclosed: CMAC | Unicast | ASN.1 |
| 26 | ARQ | AAI_ARQ-Feedback | Stand-alone ARQ Feedback | Encrypted/ICV | Unicast | Plain text |
| 27 |  | AAI_ARQ-Discard | ARQ Discard | Encrypted/ICV | Unicast | Plain text |
| 28 |  | AAI_ARQ-Reset | ARQ Reset | Encrypted/ICV | Unicast | Plain text |
| 29 | Sleep Mode | AAI_SLP-REQ | Sleep Request | Encrypted/ICV | Unicast | ASN.1 |
| 30 |  | AAI_SLP-RSP | Sleep Response | Encrypted/ICV | Unicast | ASN.1 |
| 31 |  | AAI_TRF-IND | Traffic Indication | N.A. | Broadcast | ASN.1 |
| 32 |  | AAI_TRF_IND-REQ | Traffic indication request | Encrypted/ICV | Unicast | ASN.1 |
| 33 |  | AAI_TRF_IND-RSP | Traffic indication response | Encrypted/ICV | Unicast | ASN.1 |
| 34 | Handover | AAI_HO-REQ | AMS Handover Request | Encrypted/ICV | Unicast | ASN.1 |
| 35 |  | AAI_HO-CMD | ABS Handover Command | Encrypted/ICV | Unicast | ASN.1 |
| 36 |  | AAI_HO-IND | AMS Handover Indication | Encrypted/ICV | Unicast | ASN.1 |
| 37 |  | AAI_NBR-ADV | Neighbor Advertisement | Null: in unicast N.A.: in broadcast | Unicast or broadcast | ASN.1 |
| 38 |  | AAI_NBR-REQ | Request Neighbor List | N.A. | Unicast | ASN.1 |
| 39 |  | AAI_SCN-REQ | Scanning Interval Allocation Request | Encrypted/ICV | Unicast | ASN.1 |
| 40 |  | AAI_SCN-RSP | Scanning Interval Allocation Response | Encrypted/ICV | Unicast | ASN.1 |
| 41 |  | AAI_SCN-REP | Scanning Result Report | Encrypted/ICV | Unicast | ASN.1 |
| 42 | Idle Mode | AAI_PAG-ADV | BS Paging Advertisement | N.A. | Broadcast | ASN.1 |
| 43 |  | PGID_INFO | Paging Group Advertisement | N.A. | Broadcast | ASN.1 |
| 44 | Multicarrier | AAI-MC-ADV | multicarrier Advertisement | N.A. | Broadcast | ASN.1 |
| 45 |  | AAI_MC-REQ | multicarrier Request | Encrypted/ICV | Unicast | ASN.1 |
| 46 |  | AAI_MC-RSP | multicarrier Response | Encrypted/ICV | Unicast | ASN.1 |
| 47 |  | AAI_CM-CMD | Carrier Management Command | Encrypted/ICV | Unicast | ASN.1 |
| 48 |  | AAI_CM-IND | Carrier Management Indication | Encrypted/ICV | Unicast | ASN.1 |
| 49 |  | AAI_Global-Config | Global Carrier Configuration | N.A. | Unicast | ASN.1 |
| 50 | Power Control | AAI_UL_POWER_ADJUST | Uplink TX power adjustment | Null | Unicast | Plain text |
| 51 |  | AAI_UL_PSR_Config | Uplink Power Status Reporting Configuration | Null | Unicast | Plain text |
| 52 |  | AAI_UL_PSR | Uplink Power Status Report | Null | Unicast | Plain text |
| 53 | Collocated Coexistence | AAI_CLC-REQ | Co-located coexistence request | Encrypted/ICV | Unicast | ASN.1 |
| 54 |  | AAI_CLC-RSP | Co-located coexistence response | Encrypted/ICV | Unicast | ASN.1 |
| 55 | MIMO | AAI_SingleBS_MIMO_FBK | Single-BS MIMO feedback | Null | Unicast | Plain text |
| 56 |  | AAI_MultiBS_MIMO_FBK | Multi-BS MIMO feedback | Null | Unicast | Plain text |
| 57 |  | AAI_DL_IM | Downlink interference mitigation parameter | N/A | Broadcast | Plain text |
| 58 |  | AAI_MULTI_BS_MIMO-REQ | Multi-BS MIMO Request | Null | Unicast | Plain text |
| 59 |  | AAI_MULTI_BS_MIMO-RSP | Multi-BS MIMO Response | Null | Unicast | Plain text |
| 60 |  | AAI_MultiBS_PMI_COM | Multi-BS PMI Combination | Null | Unicast | Plain text |
| 61 | FFR | AAI_FFR-CMD | FFR measurement report command | Null | Unicast | Plain text |
| 62 |  | AAI_FFR-REP | FFR measurement report | Null | Unicast | Plain text |
| 63 | SON | AAI-SON-ADV | SON Advertizement | N.A. | Broadcast | ASN.1 |
| 64 | MISC | AAI_L2-XFER | AAI L2 Transfer | Encrypted/ICV | Unicast | ASN.1 |
| 65 |  | AAI_MSG-ACK | MAC message acknowledgement | N.A. | Unicast | Plain text |
| 66 |  | AAI_E-MBS-CFG | E-MBS Configuration | Null | Broadcast | ASN.1 |
| 67 |  | AAI_RES-CMD | Reset command | Before authentication :Null After authentication : Encrypted/ICV | Unicast | ASN.1 |

# Reference

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”