| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Default Service Flow Setup during Initial Network Entry (16.2.15.6) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jungshin Park Youngkyo Baek Rakesh Taori Samsung Electronics | E-mail: shin02.park@samsung.com Phone : |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “16.2.15.6” |  |
| Abstract | This contribution proposes a procedure for setup of default service flow to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Default Service Flow Setup during Initial Network Entry (16.2.15.6)

Jungshin Park, Youngkyo Baek, Rakesh Taori

Samsung Electronics

# 1. Introduction

According to 16m/D5[1], upon successful registration, a DL FID and a UL FID are to be assigned to the AMS without using the DSA procedure in order to activate one preprovisioned service flow for each UL and DL direction which shall be used to transport upper layer signaling messages(e.g. DHCP). However, the details of how to provision the required information which should be used after the registration, is not specified clearly. This contribution proposes a method which can be used during the registration procedure and setup a default service flow without additional DSA transaction.

# 2. Solution

The service flow parameters which are required to setup the default service flow can be pre-provisioned at AMS and ABS. It is proposed that the default values for these parameters should be specified by the standard. However, the parameters values can still be set or updated through the online/offline installment by the operators when the AMS is purchased or whenever it is needed by the operators. The update of pre-provisioned parameters requires AMS and ABS to check their synchronization of parameter values before setting up the default service flow by registration. This could be resolved easily by including a new variable in the registration message, the “profile version for default service flow”.

Online update of parameters for default service flow also can be supported by the existing DSC transaction with introducing one additional parameter. The new parameter “update profile for default service flow” can be used in DSC messages to signal to MS that the QoS parameters included in DSC shall replace the current ones already stored at MS. The detailed changes can be found in the proposed texts below.

# 3. Text Proposal

Change 1: Modify the table 652 at line 1 of page 45 as follows

======================== Start of Proposed Text =====================

| Value | Description |
| --- | --- |
| 0000 | … |
| 0001 | … |
| 0010 | FID for default service flow |
| 0011-1111 | reserved |

============================== End of Proposed Text ===============

Change 2: Insert a new row at the end of the Table 688 at line 1 of page 93, and 689 at line 7 of page 100 as follows

======================== Start of Proposed Text =====================

16.2.3.7 AAI_REG-REQ

Table 688—AAI_REG-REQ message Field Descriptions

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M | Profile version for default service flow | 3 | Profile version number for default service flow, stored at AMS. Note that Service flow parameters mapped to this profile version are applied to default service flows. | N.A. |

16.2.3.8 AAI_REG-RSP

Table 689—AAI_REG-RSP message Field Descriptions

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| M | Use default service flow | 1 | 0: default service flow is not used 1: activate default service flow | N.A. |

============================== End of Proposed Text ===============

Change 3: Insert a new row at the end of the Table 744 at line 56 of page 197 as follows

======================== Start of Proposed Text =====================

Table 744—AAI_DSC-RSP

| M/O | Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | Updated profile version for default service flow | 3 | Profile version number mapped to updated default service flow parameters | It is included when updating default service flow parameters during DSC transactions for default service flow. |

============================== End of Proposed Text ===============

Change 4: Modify the specified sentences in 16.2.15.6 at line 11 of page 369 as follows

======================== Start of Proposed Text =====================

16.2.15.6 Registration

…

(start at line 50)

Upon successful registration, a DL FID and a UL FID, reserved for the default service flow, are assigned to activated at the AMS and ABS without using the DSA procedure in order to activate one preprovisioned service flow for each UL FID and DL FID direction which can be used for upper layer signaling (e.g. DHCP).

============================== End of Proposed Text ===============

Change 5: Insert the following sentences and table right after Table 784(i.e. at line 32 of page 335 ) as follows.

======================== Start of Proposed Text =====================

Table 784. Service flow/convergence sublayer parameters:

| Fields | Size(bits) | Description |
| --- | --- | --- |
| …… | ……. | …….. |
| …… | ……. | …….. |

Table xxx defines the default service flow parameters.

However, the default service flow parameters can be updated by DSC transactions or other out-of-scope methods. The default service flow parameters’ update is managed by a profile, which AAA server and AMS care for. That is, a profile defines a default service flow parameter setting. In particular the profile version of the predetermined default service flow parameters(table xxx) is set by 0b000. Whenever updating default service flow parameters, the profile version number increments.

Table xxx. Parameters for default service flow setting

| Names | Value/note |
| --- | --- |
| UL grant scheduling type | Best Effort |
| DL data delivery service type | Best Effort |
| Maximum Sustained Traffic Rate | UL: TBD DL: TBD |
| Request/Transmission Policy parameter | 1100101(see Request/Transmission Policy parameter in table 740 AAI-DSA-REQ) |
| default traffic priority | 0( default) |
| Target SAID parameter | if authorization policy supports in basic capability negotiation, Target SAID=1. Otherwise, Target SAID=0 |
| ARQ enable | 0( ARQ-disabled) |
| Packet classification rule parameter | wildcard |

16.2.12.8.1 Flow ID (FID)

The value of this parameter specifies the FID assigned by the ABS to a service flow. The ABS shall use this parameterization to assign FIDs in ABS-initiated AAI_DSA-REQ messages and in its AAI_DSA-RSP to AMS-initiated AAI_DSA-REQ messages. The FID shall be used for DSx message signaling as the identifier for a service flow.

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Mar. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.