| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on Multiprotocol flow support (5.2.6) |  |
| Date Submitted | 2010-04-30 |  |
| Source(s) | Jungshin Park Yeongmoon Son Rakesh Taori Samsung Electronics | E-mail: shin02.park@samsung.com Phone : |
| Re: | Call for LB #31a on “ P802.16m/D5”: Target topic: “5.2.6” |  |
| Abstract | This contribution proposes corrections on Multiprotocol flow to be included in the 802.16m amendment. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on Multiprotocol flow support (5.2.6)

Jungshin Park, Yeongmoon Son, Rakesh Taori

Samsung Electronics

# 1. Introduction

According to 16m/D5[1], Multiple protocols packets on the same service flow is to be supported. However, current text in the subsection 5.2.6 needs more clarification on its operation and sub-types values which it may use to identify the specific protocols of packets. For example, how we can distinguish packet with PHS and packets without PHS , what the “Raw IP” means, etc are not clear in the text.

This contribution proposes text changes for correction of these errors.

# 2. Solution

In the current CS specification, IPv4 and IPv6 packets can already be supported by the common IP type (11.13.18.1), and the ROHC or PHS packets need to be distinguished from the packets which don’t use ROHC or PHS and are sent through the same service flow. Ethernet type packets also need to be distinguished from the IP packets by the Protocol ID.

The classification of received packets at the receiver side should be corrected to be applied after uncompressing or un-suppressing the packets. Details can be found in the proposed text changes below.

# 3. Text Proposal

Modify the subsection 5.2.6 at line 38 of page 14 as follows

======================== Start of Proposed Text =====================

5.2.6 Support for multiple protocols on the same flow

In order to transport several types of protocols over the same MAC connection, the Multiprotocol flow can be used. The receiver must identify the protocol to correctly forward the SDU. For instance, if the information carried by the SDU is a RoHC packet, it should be forwarded to the RoHC decompressor. The receiver does this according to the Protocol ID field which is the first byte of a Multiprotocol flow connection as depicted in Figure 18a and Figure 18b.

…

In the transmitter side, Oonce the protocol type of an incoming packet is determined, the appropriate classification rules are applied to the packet and the correct service flow is identified. It is then optionally forwarded to the header suppression mechanism (PHS or RoHC) and then mapped MAC SAP using the format described in this section. The Protocol ID content is set by the transmitter by to the protocol type identified before by the classification rules were applied and according to the Table 2a.

The method by which the protocol of a packet introduced to the CS layer is identified is beyond the scope of this standard.

s

Table 2a—Possible values for Protocol ID fieldfor Multiprotocol flow

| Protocol ID | Meaning |
| --- | --- |
| 1 | Raw IP |
| 2 | IPv4 |
| 3 | RoHC |
| 4 | IPv6 |
| 5-255 | Reserved |

| Protocol ID | Meaning |
| --- | --- |
| 1 | IP (without RoHC and PHS) |
| 2 | IP with RoHC |
| 3 | IP with PHS |
| 4 | Ethernet |
| 5 | Ehternet with PHS |
| 6-255 | Reserved |

============================== End of Proposed Text ===============

# 4. References

- IEEE P802.16m/D5. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, Mar. 2010.

- IEEE 802.16m-08/003r9a. The Draft IEEE 802.16m System Description Document, May 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.