| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Rank 1 MIMO Support inside OL Region |  |
| Date Submitted | 2010-03-16 |  |
| Source(s) | Kiran Kuchi Klutto Milleth Padmanabhan M S Bhaskar Ramamurthi | kkuchi@cewit.org.in klutto@cewit.org.in padmanabhan@cewit.org.in |
| Re: | Letter Ballot #31 on the Draft Amendment (IEEE P802.16m/D4) |  |
| Abstract | Introduce Rank1 MIMO mode inside OL region |  |
| Purpose | To be discussed and adopted by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Rank1 MIMO Support inside OL Region

Kiran Kuchi

Klutto Milleth

Padmanabhan M S

Bhaskar Ramamurthi

Abstract

Rank1 MIMO mode inside OL region gives significant system capacity gain for both NLRU and SLRU modes. This mode can be introduced in D5, without increasing control channel overhead. This mode can be introduced by using existing MIMO mode, MIMO feedback modes and feedback formats., without affecting other features. Rank-1 mode is  introduced as Type 3 OL region.

Start of Text

…………………….. …………………………………………………..

Table 865—Codebook subsets used for non-adaptive precoding in DL DLRU and NLRU

| MEF | RU with Mt pilot MIMO streams outside OL region | RU in OL region with MaxMt MIMO streams |
| --- | --- | --- |
| SFBC | Nt=2: CDL,OL,SU (2, Mt, 1), Mt=2 Nt=4: CDL,OL,SU (4, Mt, 4), Mt=2 Nt=8: CDL,OL,SU (8, Mt, 4), Mt=2 | Nt=2: CDL,OL,SU (2,MaxMt, 1), MaxMt=2 Nt=4: CDL,OL,SU (4,MaxMt, 4), MaxMt=2 Nt=8: CDL,OL,SU (8,MaxMt, 4), MaxMt=2 |
| VE | Nt=2: CDL,OL,SU (2, Mt, Nw), Mt=1,2 Nt=4: CDL,OL,SU (4, Mt, Nw), Mt=1,2,3,4 Nt=8: CDL,OL,SU (8, Mt, Nw), Mt=1,2,3,4 Nw depends on Mt | Nt=1: CDL,OL,SU (2,MaxMt, 1), MaxMt=1 Nt=2: CDL,OL,SU (2,MaxMt, 1), MaxMt=2 Nt=4: CDL,OL,SU (4,MaxMt, 4), MaxMt=2 Nt=8: CDL,OL,SU (8,MaxMt, 4), MaxMt=2 |
| ME | n.a | n.a |
| CDR | n.a | Nt=2: CDL,OL,SU (2, MaxMt, 2), MaxMt=1 Nt=4: CDL,OL,SU (4, MaxMt, 4), MaxMt=1 Nt=8: CDL,OL,SU (8, MaxMt, 8), MaxMt=1 |

Table 870—DL MIMO parameters

|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 0 | 2 | 1 | 2 | 2 | 1 |
|  | 4 | 1 | 2 | 2 | 1 |
|  | 8 | 1 | 2 | 2 | 1 |
| MIMO mode 1 and MIMO mode 2 | 2 | 1 | 1 | 1 | 1 |
|  | 2 | 2 | 2 | 1 | 1 |
|  | 4 | 1 | 1 | 1 | 1 |
|  | 4 | 2 | 2 | 1 | 1 |
|  | 4 | 3 | 3 | 1 | 1 |
|  | 4 | 4 | 4 | 1 | 1 |
|  | 8 | 1 | 1 | 1 | 1 |
|  | 8 | 2 | 2 | 1 | 1 |
|  | 8 | 3 | 3 | 1 | 1 |
|  | 8 | 4 | 4 | 1 | 1 |
|  | 8 | 5 | 5 | 1 | 1 |
|  | 8 | 6 | 6 | 1 | 1 |
|  | 8 | 7 | 7 | 1 | 1 |
|  | 8 | 8 | 8 | 1 | 1 |

Table 870—DL MIMO parameters

|  | Number of transmit antennas | STC rate per MIMO layer | Number of MIMO streams | Number of subcarriers | Number of MIMO layers |
| --- | --- | --- | --- | --- | --- |
|  | Nt | R | Mt | NF | L |
| MIMO mode 3 and MIMO mode 4 | 2 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 2 | 1 | 2 |
|  | 4 | 1 | 3 | 1 | 3 |
|  | 4 | 1 | 4 | 1 | 4 |
|  | 8 | 1 | 2 | 1 | 2 |
|  | 8 | 1 | 3 | 1 | 3 |
|  | 8 | 1 | 4 | 1 | 4 |
| MIMO mode 4 | 4 | 2 and 1* | 3 | 1 | 2 |
|  | 4 | 2 and 1** | 4 | 1 | 3 |
|  | 4 | 2 | 4 | 1 | 2 |
|  | 8 | 2 and 1* | 3 | 1 | 2 |
|  | 8 | 2 and 1** | 4 | 1 | 3 |
|  | 8 | 2 | 4 | 1 | 2 |
| MIMO mode 5 | 2 | ½ | 1 | 2 | 1 |
|  | 4 | ½ | 1 | 2 | 1 |
|  | 8 | ½ | 1 | 2 | 1 |

Table 872—Supported Permutation for each DL MIMO mode in the OL region

|  | DLRU | NLRU | SLRU |
| --- | --- | --- | --- |
| MIMO mode 0 | Yes, with MaxMt = 2 | No | No |
| MIMO mode 1 | Yes, with MaxMt = 2 | Yes, with MaxMt = 1 | Yes, with MaxMt = 1 and 2 |
| MIMO mode 2 | No | No | No |
| MIMO mode 3 | No | No | Yes, with MaxMt = 2 |
| MIMO mode 4 | No | No | No |
| MIMO mode 5 | No | Yes, with MaxMt = 1 | Yes, with MaxMt = 1 |

Table 873—Types of open-loop regions

|  | MaxMt | MIMO mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL region type 0 | 2 MIMO streams | MIMO mode 0 MIMO mode 1 (Mt = 2 MIMO streams) | MFM 0 | DLRU |

Table 873—Types of open-loop regions

|  | MaxMt | MIMO mode | MIMO feedback mode | Supported permutation |
| --- | --- | --- | --- | --- |
| OL region type 1 | 1 MIMO stream | MIMO mode 5 (Mt = 1 MIMO stream) | MFM 1 | NLRU |
|  |  |  | MFM 2 | SLRU |
| OL region type 2 | 2 MIMO streams | MIMO mode 1 (Mt = 2 MIMO streams for one AMS) MIMO mode 3 (Mt = 2 total number of MIMO streams for two AMSs) | MFM 5 | SLRU |
| OL region type 3 | 1 MIMO stream | MIMO mode 1 (Mt = 1 MIMO stream) | MFM 1 | NLRU |
|  |  |  | MFM 2 | SLRU |

Table 874—MIMO feedback modes

| MIMO Feedback Mode | Description and type of RU | Feedback content | Supported MIMO transmission mode outside the OL region (when Measurement Method Indication = 0b0) | Supported MIMO transmission mode inside the OL region (when Measurement Method Indication = 0b1) |
| --- | --- | --- | --- | --- |
| 0 | OL SU MIMO SFBC/SM (Diversity: DLRU, NLRU) Sounding based CL SU and MU MIMO | 1. STC Rate 2. Wideband CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI 2 <= STC Rate <= 4: SM CQI In DLRU: Mt=2 for SM. In NLRU: Mt >= 2 for SM For sounding based CL SU MU MIMO, STC Rate = 1: SFBC CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate = 2: SM CQI In DLRU only. |
| 1 | OL SU MIMO CDR (Diversity: NLRU) | 1. Wideband CQI | NA. | MIMO mode 5,STC Rate = ½ and MIMO mode 1, STC Rate=1 |
| 2 | OL SU MIMO SM (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband Selection | MIMO mode 1 1 <= STC Rate <= 8 | MIMO mode 5,STC Rate = ½ and MIMO mode 1, STC Rate = 1 |
| 3 | CL SU MIMO (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband PMI 4.Subband selection 5. Wideband correlation matrix | MIMO mode 2 1 <= STC Rate <= 8 | n.a |
| 4 | CL SU MIMO (Diversity: NLRU) | 1. STC Rate 2. Wideband CQI 3. Wideband PMI 4. Wideband correlation matrix | MIMO mode 2 (Mt≤4) | n.a |
| 5 | OL MU MIMO (localized: SLRU) | 1. Subband CQI 2. Subband Selection 3. MIMO stream indicator | MIMO mode 3 | MIMO mode 3 |

Table 874—MIMO feedback modes

| MIMO Feedback Mode | Description and type of RU | Feedback content | Supported MIMO transmission mode outside the OL region (when Measurement Method Indication = 0b0) | Supported MIMO transmission mode inside the OL region (when Measurement Method Indication = 0b1) |
| --- | --- | --- | --- | --- |
| 6 | CL MU MIMO (localized: SLRU) | 1. Subband CQI 2. Subband PMI 3. Subband Selection 4. Wideband correlation matrix | MIMO mode 4 | n.a |
| 7 | CL MU MIMO (Diversity: NLRU) | 1. Wideband CQI 2. Wideband PMI 3. Wideband correlation matrix | MIMO mode 4 | n.a |

……………………………….End of Text………………………………………………………………………………………..