| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Legacy ASN support capability |  |
| Date Submitted |  |  |
| Source(s) | Guanghui Fan, Yang Tang, Li Li Huawei Technologies | E-mail: fanguanghui@huawei.com lily08@huawei.com |
| Re: | Call for LB #31a on “ P802.16m/D5”: |  |
| Abstract | This contribution proposes about Legacy ASN support capability |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 31a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Legacy ASN support capability

Guanghui Fan, Yang Tang, Li Li

Huawei Technologies

Introduction

In the WiMAX network deployment scenario, one ABS may be connected to more than one ASN GWs.  If the legacy ASN GW is deployed in 16m network, there may be such a scenario that the ABS is connected to both legacy ASN GW and advance ASN GW. The ABS should broad such a configuration status to the AMSs and to neighbor ABSs.

The AMS may support either both legacy ASN and advanced ASN or advanced ASN only. This capability should inform the ASN network for the purpose of connection mode selection or handover decision.

Text proposal for inclusion in the P802.16m/D5

========================== Start of Proposed Text ==============================

[Editor’s Note 1 : modify the following field description in “Table 838—S-SFH SP2 IE format” in page 532]

Table 838—S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Network Configuration | 1 2 | Indicates configuration of the ABS network 0b0: AAI network configuration 0b1: WirelessMAN-OFDMA network configuration 0b00: AAI network configuration 0b01: WirelessMAN-OFDMA network configuration 0b10: Both AAI network and WirelessMAN-OFDMA network configuration 0b11: Reserved |

[Editor’s Note 2: modify the following field description in “Table 680—AAI_RNG-REQ message Field Descriptions” in page 76]

| M/O | Attributes/ Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | AMSID or MAC address included | 1 | This bit indicates which identifier is followed. 0：AMSID is followed 1：MAC address is followed | N.A. |
| O | AMSID*/MAC address | 48 | It's the hash value of AMSID in order to protect AMS privacy, which is used for ABS to distinguish AMSs when more than one AMS send AAI_RNG-REQ message at the same time. In the legacy network mode, where the ABS is connected to the legacy network, the AMS provides its actual MAC address instead. | It shall be included when the AMS is attempting network entry without its STID/DID which the ABS/Paging Controller assigns. |
| O | MAC version | 8 | See 11.3 |  |
| O | WirelessMAN-OFDMA network configuration support | 1 | 0: support WirelessMAN-OFDMA network configuration 1: not support WirelessMAN-OFDMA network configuration | It shall be included when the AMS using AMSID* to entry/reentry the network. |

========================== End of Proposed Text ==============================