| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Reply comment for Ranging Abort Timer (16.2.3.2) |  |
| Date Submitted | 2010-05-12 |  |
| Source(s) | Hyunjeong Kang, Jaejeong Shim Samsung Electronics Co., Ltd. | hyunjeong.kang@samsung.com +82-31-279-5084 |
| Re: | IEEE 802.16-10/0032: “IEEE 802.16 Working Group Letter Ballot #31a: Announcement” |  |
| Abstract | The contribution proposes a reply comment as a remedy for comment #94 |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Reply comment for Ranging Abort Timer

Hyunjeong Kang, Jaejeong Shim

Samsung Electronics Co., Ltd.

- Introduction

An access of an AMS may be failed during ranging i.e. AAI_RNG-REQ and AAI_RNG-RSP transaction due to admission control, authentication failure, context retrieval failure or internal error. The access failure is indicated through a parameter Ranging Status=0b01 (abort) in the AAI_RNG-RSP message. In this case, Ranging Abort Timer should be provided to prohibit the AMS from attempting network access to the ABS during a specific time or provided not to try its access to the ABS again.

- Text proposal

-------------------------------  Text Start  ---------------------------------------------------

[Remedy: Insert a new parameter Ranging Abort Timer at line 14 on page 80 in Table 681 as follows:]

| O | Ranging Abort Timer | 16 | Timer defined by an ABS to prohibit the AMS from attempting network entry at this ABS, for a specific time duration. Value: 0 (Do not try ranging again at the ABS.) Value: 1–65535, In units of seconds | It shall be included when Ranging Abort is set to 1. |
| --- | --- | --- | --- | --- |

-------------------------------  Text End  ---------------------------------------------------