| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarifications for Update of S-SFH (16.2.24) |  |
| Date Submitted | 2010-05-10 |  |
| Source(s) | Hyunkyu Yu, Hyunjeong Kang and Jiyun Seol Samsung Electronics Co., Ltd. |  |
| Re: | Call for Ballot#31a” Target topic: “IEEE P802.16m/D5, section 16.2.24”. |  |
| Abstract | The contribution provides the clean up text for update of S-SFH |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m/D6 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarifications for Update of S-SFH (16.2.24)

Hyunkyu Yu, Hyunjeong Kang and Jiyun Seol

Samsung Electronics Co., Ltd.

- Introduction

This contribution provides the clarification on the update of S-SFH. Also, the proposed texts include some modification to the applying time in order to know in advance when new S-SFH contents will be applied. For example, considering migration path, from mixed mode to 16m only mode, it causes to change the frame configuration information carried in S-SFH SP1 IE. In this case, the location of SFH will be changed whenever DL/UL ratio at 16m region is changed. So, AMS trying to access to ABS (i.e., doesn’t know the periodicity of each SP, rate of change of S-SFH) cannot acquire the SFH location when ABS changes the frame configuration. To avoid this situation, the applying time should be set to 0 in advance.

Removed text

Added text

- Proposed Text Changes

[Remedy #1, Modify the text in line 40, page 408, section 16.2.24 as follows,]

Every superframe, the ABS transmits P-SFH IE containing the S-SFH Scheduling information bitmap, S-SFH change count, S-SFH SP change bitmap, and S-SFH applying offset. The S-SFH change count shall remain unchanged as long as all of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH SP IEs remain unchanged. The S-SFH change count shall be incremented by 1 modulo 16 whenever any of the values (except MSB of superframe number in S-SFH SP1 IE) of S-SFH IEs changes. The changed S-SFH SP IE(s) shall be transmitted at the scheduled superframes of period corresponding to each S-SFH SP IE.

[Remedy #1, Modify the text in line 20, page 409, section 16.2.24 as follows,]

The S-SFH applying offset in the P-SFH IE is used to indicate which S-SFH change count shall be considered for applying the S-SFH contents in next superframe. the system parameters in the S-SFH SPx IEs. According to the S-SFH applying offset, if the S-SFH applying offset is set to 0, the AMS shall use the S-SFH contents system parameters of S-SFH SPx IEs associated with the current S-SFH change count in next superframe. Otherwise, if the S-SFH applying offset is 1, the AMS shall store the changed S-SFH contents parameters of S-SFH SP IEs associated with the current S-SFH change count, but use the S-SFH contents parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16) in next superframe. If the S-SFH contents are not available, the AMS shall acquire the relevant S-SFH contents by decoding the S-SFHs carried in their corresponding scheduled superframes.

If the AMS determines that S-SFH change count has not changed and S-SFH applying offset is set to 0, then the AMS shall use the S-SFH contents associated with current S-SFH change count in current superframe.

The ABS shall set the S-SFH applying offset to 1 for a number of superframes after the S-SFH change count is incremented by 1. To guarantee enough time for AMSs to receive the changed S-SFH SPx IE(s), S-SFH applying offset shall be set to 1 until the changed S-SFH SP1 IE and the changed S-SFH SP2 IE are transmitted 2 times and the changed S-SFH SP3 IE is transmitted once i.e, S-SFH applying offset shall be set to 0 when the changed S-SFH SP1 IE and the changed S-SFH SP2 IE are transmitted 2 times and the changed S-SFH SP3 IE is transmitted once.

For all S-SFH SPx IEs indicated in the S-SFH change bitmap, their changed contents shall be applied at the same timesuperframe.

[Remedy #2. Insert the following text and figure after line 37 on page 409, section 16.2.24 as follows]

Figure xxx shows the examples of S-SFH update procedure. This figure considers the following assumptions:

- The periodicity of SP1 IE, SP2 IE and SP3 IE are 40ms, 80 ms and 160 ms, respectively

- Rate of change of S-SFH info is 16, i.e., S-SFH change cycle is 32.

- ‘No’ means that no SPx IE is scheduled.

(a) SP1 IE is changed

(b) SP1 IE and SP3 IE are changed

(c) SP3 IE is changed

Figure xxx. Illustrations of S-SFH update procedure

[Remedy #3, Modify the text in line 52, page 525, section 16.3.6.5.1.1 as follows,]

Table 835 – P-SFH IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| P-SFH IE format () { |  |  |
| ….. |  |  |
| S-SFH applying offset | 1 | Indicate the associated S-SFH change count in this next superfame 0b0: Use S-SFH SPx IEs associated with the current S-SFH change count 0b1: Use S-SFH SPx IEs associated with (the current S-SFH change count - 1) modulo 16 |
| …… |  |  |

….

S-SFH applying offset

A one bit flag that indicates AMSs which S-SFH change count shall be considered for applying the S-SFH contents in next superframe the system parameters in the S-SFH SPx IEs. If the S-SFH applying offset is set to '0', the AMS shall use the S-SFH contents the system parameters of S-SFH SPx IEs associated with the current S-SFH change count in next superframe. Otherwise, if the S-SFH applying offset is '1', the AMS shall use the S-SFH contents the parameters associated with previous S-SFH change count(= (the current S-SFH change count -1) modulo 16) in next superframe. If the S-SFH contents associated with previous S-SFH change count is are not available, the AMS shall acquire the relevant S-SFH contents by decoding the S-SFH IEs carried in their corresponding scheduled superframes