| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text related to trigger descriptions for MC and IM operations (16.2.3.30) |  |
| Date Submitted | 2010-05-11 |  |
| Source(s) | Jaehyuk Jang, Hyunjeong Kang Samsung Electronics | E-mail: jack.jang@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #31a |  |
| Abstract | The contribution proposes the text of trigger descriptions for MC and IM operations. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text related to trigger descriptions for MC and IM operations (16.2.3.30)

Jaehyuk Jang, Hyunjeong Kang

Samsung Electronics

- Introduction

In this contribution, we propose the text of trigger descriptions for MC and IM operations.

- Discussions

We suggest defining two new broadcasting trigger tables for ‘MC’ and ‘IM’ as part of the AAI_SCD message. To include these two tables does not impact to the ABS which does not support MC and IM, and overhead problem can also be solved.

- References

[1] IEEE P802.16m/D5, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------------------Start of the Text------------------------------------------------------------

[Add following fields to Table 717 – AAI_SCD message in page 159, line 65 as follows:]

16.2.3.30 AAI_System Configuration Descriptor (SCD) Message

Table 717—AAI_SCD Message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| O | MC Trigger definitions | Variable | Refer to Table x1 defined in section 16.2.8.2.11.1.1 | Present if MC specific trigger needs to be defined |
| O | IM Trigger definitions | Variable | Refer to Table y1 defined in section 16.4.11.1 | Present if IM is initiated |

[Insert the section after line 65 on page 311 as follows:]

16.2.8.2.11.1.1 Multicarrier-specific Trigger condition definitions

MC-specific Trigger definitions are encoded using the description in Table x1.

Table x1 —MC-specific Trigger Description

| Name | Legnth (bits) | Value |
| --- | --- | --- |
| Type/Function/Action | 4 | See Table x2 for description |
| Trigger Value | 8 | Trigger value is the value used in comparing measured metric for determining a trigger condition. |
| Trigger averaging parameter | 8 | The averaging parameter used for averaging this trigger metric according to equation (1) defined in section 16.2.6.2 for target ABS 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |

Table x2—MC-specific Trigger; Type/Function/Action Description

| Name | Size (bits) | Value |
| --- | --- | --- |
| Type | 1 | Trigger metric type: 0x0: CINR metric 0x1: RSSI metric |
| Function | 2 | Computation defining trigger condition: 0x0: Metric of primary carrier is greater than absolute value 0x1: Metric of primary carrier is less than absolute value 0x2-3: Reserved |
| Action | 1 | Action performed upon reaching trigger condition: 0x0: Respond on trigger with AAI_SCN-REQ 0x1: Respond on trigger with AAI_SCN-REP |

[Insert the section after line 42 on page 779 as follows:]

16.4.11.1 Interference Mitigation Trigger condition definitions

IM-specific Trigger definitions are encoded using the description in Table y1.

Table y1—IM-specific Trigger; Type/Function/Action Description

| Name | Legnth (bits) | Value |
| --- | --- | --- |
| Type/Function/Action | 3 | See Table y2 for description |
| Trigger Value | 8 | Trigger value is the value used in comparing measured metric for determining a trigger condition. |
| Trigger averaging parameter | 8 | The averaging parameter used for averaging this trigger metric according to equation (1) defined in section 16.2.6.2 for target ABS. 0x0: 1 0x1: 1/2 0x2: 1/4 0x3: 1/8 0x4: 1/16 0x5: 1/32 0x6: 1/64 0x7: 1/128 0x8: 1/256 0x9: 1/512 0xA to 0xFF: reserved |

Table y2—IM-specific Trigger; Type/Function/Action Description

| Name | Size (bits) | Value |
| --- | --- | --- |
| Type | 1 | Trigger metric type: 0x0: RSSI metric 0x1: Reserved |
| Function | 1 | Computation defining trigger condition: 0x0: Metric of neighbor ABS is less than absolute value 0x1: Reserved |
| Action | 1 | 0x0: Respond on trigger with AAI_SCN-REP to terminate IM. 0x1: Reserved |

--------------------------------------------------------------------End of the Text-------------------------------------------------------------------------