| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text to move FFR resource metric to AAI_SCD (16.2.21) |  |
| Date Submitted | 2010-5-12 |  |
| Source(s) | Yi Hsuan, Hujun Yin, Yuan Zhu Intel | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 16.2.21 (Interference Mitigation Mechanism) “Comments on AWD 16.2.21 Interference Mitigation Mechanism” |  |
| Abstract | The contribution proposes text to move FFR resource metric to AAI_SCD message |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text to move FFR resource metric to AAI_SCD (16.2.21)

Yi Hsuan, Hujun Yin, Yuan Zhu

Intel Corporation

- Introduction

In IEEE P802.16m/D5, the FFR resource metric can be in either S-SFH SP3 or AAI_DL_IM. However there is no need to transmit the same information in different places. We are proposing to use AAI_SCD to transmit the resource metric because 1) it doesn’t need to be always transmitted like S-SFH SP3 and 2) it can be transmitted at reasonable frequency if the feature of resource metric based FFR is activated.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal 1 (Section 16.2.21.1.1, page 401, line 62) ]

AAI_SCD AAI_DL_IM message or S-SFH SP3 includes resource metric information, as shown in Table 796, which is needed for AMS to select DL frequency partition in case of distributed permutation, or to select subband in case of localized Permutation.

- [ Recommended AWD Text Proposal 2 (Section 16.2.3.30, page 159, line 65). Add the following entry at the end of Table 717. ]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | resourceMetricFP2 | 4 | Resource_Metric_FP2 Resource Metric of the first power de-boosted frequency partition which is defined in Table 796. | resourceMetricFP2 is transmitted if FPS2 >0 as described in Table 810 to 812. |
| O | resourceMetricFP3 | 4 | Resource_Metric_FP3 Resource Metric of the second power de-boosted frequency partition which is defined in Table 796. | resourceMetricFP3 is transmitted if FPS3 >0 as described in Table 810 to 812. |

- [ Recommended AWD Text Proposal 3 (Section 16.2.3.34, page 162, line 46). ]

| O | resourceMetricFP2 | 4 | Resource_Metric_FP2 Resource Metric of the first power de-boosted frequency partition which is defined in Table 761. There is no this field if FPCT=1 which is defined in Table 777 to Table 781. | resourceMetricFP2 is transmitted if FPS2 >0 as described in Table 810 to 812. |
| --- | --- | --- | --- | --- |
| O | resourceMetricFP3 | 4 | Resource_Metric_FP3 Resource Metric of the second power de-boosted frequency partition which is defined in Table 796. There is no this field if FPCT=1 which is defined in Table 777 to Table 781. | resourceMetricFP3 is transmitted if FPS3 >0 as described in Table 810 to 812. |

- [ Recommended AWD Text Proposal 4 (Section 16.3.6.5.1.2, page 533, line 42). ]

| FFR partition resource metrics | 8 | As defined in Table 772 |
| --- | --- | --- |

-------------------------------------- End of Text Proposal -------------------------------------------