| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup AAI_MC-REQ and AAI_MC-RSP messages |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Joey Chou Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix MAC control message issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Cleanup AAI_MC-REQ and AAI_MC-RSP messages

- Joey Chou,

- Intel

- Introduction

This contribution Cleanup AAI_MC-REQ and AAI_MC-RSP messages that include redundant tables, and imcomplete attribute definitions.

- Proposed text

------------------------------------------------- Start of proposed text I--------------------------------------------------

II.1 Proposed text 1

16.2.3.52 AAI_MC-REQ (multicarrier Request) Message

[Delete Table 755—AAI_MC-REQ message format]

Table 755—Contents of AAI_MC-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Support | 1 | Indicates whether the AMS can process all carriers supported in ABS simultaneously 0b0: The AMS can only process a subset of the carriers supported in ABS 0b1: The AMS can process all carriers supported in ABS |  |
| O | Number of candidate combinations [0..15] | Variable 4 | Indicates the number of carrier combination MS can support | Shall be present when Global Support is zero |
| O | A) Number of candidate assigned carrier [0..7] | Variable 3 | Indicates the number of carriers MS can support | Shall be present when the number of candidate combinations is not zero |
| O | A.1) Physical carrier index | 6 | Indicates the carriers MS can simultaneously support | Shall be present when the number of candidate assigned carrier is not zero |
| O | DU/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. | Shall be present when the number of candidate combinations is not zero |
| M | Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |

------------------------------------------------- End of proposed text I --------------------------------------------------

------------------------------------------------- Start of proposed text II--------------------------------------------------

II.2 Proposed text 2

16.2.3.53 AAI_MC-RSP (multicarrier Response) Message

[Delete Table 757—AAI_MC-RSP message format]

Table 758—Contents of AAI_MC-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Global Support | 1 | Indicates whether the ABS assigns all the carriers requested by AMS through AAI_MC-REQ 0b0: The ABS assigns a subset of the carriers requested by AMS 0b1: The ABS assigns all the carriers requested by AMS |  |
| O | Number of assigned carriers [0..7] | Variable 3 | Indicates the number of carriers to be assigned to the MS | Shall be present when the Global Assign is zero |
| O | A) Physical carrier index | 6 | Indicates the carriers to be assigned to the MS. Logical carrier index is assigned implicitly in the order of assigned physical carrier index | Shall be present when the Global Assign is zero |
| M | B) Support of data transmission over guard sub-carrier | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A |
| O | C) DU/UL indicator | 1 | Indicates whether the carrier is assigned for both DL and UL 0b0: The carrier is assigned for both DL and UL 0b1: The carrier is assigned for DL only. | Shall be present when the Global Assign is zero |

------------------------------------------------- Start of proposed text II --------------------------------------------------