| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up capability negotiation parameters during Network Entry/Re-entry Sections: 16.2.3 |  |
| Date Submitted | 2010-07-xx |  |
| Source(s) | Shaocheng Wang Sassan Ahmadi Xiangying Yang Muthaiah Venkatachalam Intel Corporation | E-mail: shaocheng.wang@intel.com; sassan.ahmadi@intel.com; xiangying.yang@intel.com; muthaiah.venkatachalam@intel.com; |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “section 16.2.3” |  |
| Abstract | This contribution proposes a new structure for capability negotiation parameters during network entry/re-entry to minimize the use of radio resources for transmission of unnecessary parameters that otherwise must be supported by default. We further categorize the parameters to be negotiated based on pre-authentication or post-authentication to ensure encryption and protection of those parameters that are directly pertained to the user or device identity and credentials. |  |
| Purpose | Adopt the proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clean up capability negotiation parameters during Network Entry/Re-entry

Shaocheng Wang, Sassan Ahmadi, Muthaiah Venkatachalam, Xiangying Yang

Intel Corporation

- Introduction

We propose to remove the following parameters in current D6 NE procedures that lack technical justifications to be included in capability negotiation.

- 3-step BR:

Green field 16m’s BR channel is designed around 3-step BR with 5-step fallback since early 16m standard stages. There is no good technical reason to not consider 3-step BR as mandatory.

For AMS, there is no added cost to put the 4-bit quick access message, the space is already there. In fact, if AMS opts for 5-step directly, this 4-bit space is a waste! For ABS, there is also minimum or almost none added cost to process this 4-bit, and if this 4-bit info is successfully decoded, it saves further BR from AMS, which is good thing ABS wants to see!

- Dynamic ranging for HO:

There is no logical need to negotiate this capability during NE. If any AMS doesn’t support dynamic ranging, it will just ignore the extra ranging opportunities. If any ABS doesn’t support dynamic ranging, it will just not provide such opportunities.

- Resource retain timer:

The issue is, this may as well be a per BS configuration. If this is made into capability, how do one update it when MS performs handover from say a macro to a femto?

Also this would increase BS complexity since BS needs to remember possible different timer for each different MS. So it’s a good idea to keep this timer in HO-CMD, and not pushing it to NE level.

- Redirection info:

First it is not clear whether this is used for rev.2 load balancing or Femto.

If this is used for rev.2 load balancing, it should be irrelevant from the MSID. It is meant for BS to perform load balancing, i.e. regardless what the MSID is, MSID* or MSID.

If this is used for femto case when MS does not supply CSGIDs, then I am not sure current NWG performs Femto CSG access control during authentication or after authentication. Need to check on this.

- Frame configuration to support legacy(5MHz & 10MHz):

these two configurations are mandatory per latest WMF twg decisions.

- OFDMA system support:

No clear definition.

- Text Proposal

--------------------------------Start of the proposed text-----------------------------------------

[Remedy #1]

[Note to Editor] In section 16.2.3.7 under AAI_REG-REQ modify Table 687 and Table 688 with following table (entries that are crossed out are existing parameters that we suggest to remove)

Table 687—AAI_REG-REQ message Field Descriptions

| O |  | frame configuration to support legacy(5MHz) | 1 | If Bit#0 =1,it supports(TDD only) |  |
| --- | --- | --- | --- | --- | --- |
| O |  | frame configuration to support legacy(5MHz) | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support | 1 | If Bit#0 =1,it supports |  |
| O |  | 3-step BR | 1 | If Bit#0 =1,it supports |  |
| O |  | dynamic ranging for HO | 1 | If Bit#0 =1,it supports |  |
| O |  | Resource_Retain_Time | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |

Table 688—AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | Convergence sublayer capabilities | Resource_Retain_Time | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |
| O | Redirection Info | ABSID, preamble index and center frequency for one or more neighbor ABS |  | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |

--------------------------------End of the proposed text-----------------------------------------