| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up AAI_DSX-REQ/RSP/ACK attributes |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Joey Chou Jie Hui Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_DSX-REQ/RSP/ACK attribute issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clean up AAI_DSX-REQ/RSP/ACK attributes

- Joey Chou, Jie Hui

- Intel

- Introduction

This contribution is intended to clean up AAI_DSX-REQ, AAI_DSX-RSP, and AAI_DSX-ACK attributesthat are missing definitions, not properly defined, or using wrong format.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.46.1 AAI_DSA-REQ

Table 742—AAI_DSA-REQ

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_DSA-REQ | N.A. |
| O | SFIF | 32 | Service flow identifier | Present when an ABS initiates AAI_DSA-REQ |
| O | FID | 4 | Flow identifier | Present when an ABS initiates AAI_DSA-REQ |
| O | Global Service Class Name field | variable (see 6.3.14.4.1) 84 | Variable: Refer to Table 186 of 6.3.14.4.1 Name of global service classes (see Table 783 in section 16.2.12.8) | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| O | S) Quick Access Predefined BR Index Array [1..15] Parameter | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions (see Table 789) | Present for UL |
| O | S.1) Predefined BR index | 4 | Predefined BR index |  |
| O | S.2) BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved |  |
| O | S.3) BR size | 11 | Number of bytes requested by the predefined BR index | Present if BR action = 0b10 |
| O | U) Vendor Specific QoS Parameter | variable | Vendor Specific QoS Parameter | Present if needed |
| O | U.1) Vendor ID | 24 | vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. |  |
| O | ARQ parameters | variable |  | Present if needed |
| O | G) ARQ_ERROR_DETECTION_TIMEOUT | 16 | 0 = Infinite 1-6553500 s (100 s granularity) |  |
| O | H) ARQ_FEEDBACK_POLL_RETRY_TIMEOUT | 16 | 0 = Infinite 1-6553500 s (100 s granularity) |  |
| O | Packet Classification Rule parameter | variable |  | Present if needed |
| O | I) Vendor Specific classification Parameter | variable | Compound | Present if needed |
| O | I.1) Vendor ID | 24 | vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. |  |
| O | N) PHS rule field | variable |  | Present if needed |
| O | N.6) Vendor-specific PHS parameters | variable | Compound | Present if needed |
| O | ROHC Parameter | variable |  | Present if needed |
| O | fullNumE-MBSIdArr[1..8] | Variable |  | Present if needed |
| M | C) Number of E-MBS ID_FID Mappings (M) | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to add | N.A. |
| M | D) fullEMBSIdFidMappingArr[01...156] | variable | Mapping of E-MBS ID and FID. |  |
| M | D.1) E-MBS ID and FID | Variable(16xN) 12 | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings E-MBS identifier | N.A. |
| M | D.2) FID | 4 | Flow ID |  |
| O | Group Parameter Create/Change | variable |  | Present if needed |
| O | D) Group FID Parameters List | variable |  |  |
| O | D.2) Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
| O | Coupled Group Create/Change | variable |  |  |
| O | A) Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | D) Coupled FID Parameter List | variable |  |  |
| O | D.2) Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.3.46.2 AAI_DSA-RSP

Table 743—AAI_DSA-RSP

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |
| O | fullNumE-MBSIdArr[1..8] | Variable |  |  |
| M | C) Number of E-MBS ID_FID Mappings (M) | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to add | N.A. |
| M | D) fullEMBSIdFidMappingArr[01...156] | variable | Mapping of E-MBS ID and FID. |  |
| M | D.1) E-MBS ID and FID | Variable(16xN) 12 | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings E-MBS identifier | N.A. |
| M | D.2) FID | 4 | Flow ID |  |

------------------------------------------------- End of proposed text II --------------------------------------------------

II.4 Proposed text 3

------------------------------------------------- Start of proposed text III--------------------------------------------------

16.2.3.46.3 AAI_DSA-ACK

Table 744—AAI_DSA-ACK

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |

------------------------------------------------- End of proposed text III --------------------------------------------------

II.3 Proposed text 4

------------------------------------------------- Start of proposed text IV--------------------------------------------------

16.2.3.46.4 AAI_DSC-REQ

Table 745— AAI_DSC-REQ

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 | AAI_DSC-REQ | N.A. |
| O | SFIF | 32 | Service flow identifier | Present when an ABS initiates AAI_DSC-REQ |
| O | FID | 4 | Flow identifier | Present when an ABS initiates AAI_DSC-REQ |
| O | Global Service Class Name field | variable (see 6.3.14.4.1) 84 | Variable: Refer to Table 186 of 6.3.14.4.1 Name of global service classes (see Table 783 in section 16.2.12.8) | Present when a predefined BS service configuration to be used for this service flow and are synchronized among all BS. |
| O | Q) Quick Access Predefined BR Index Array [1..15] Parameter | Variable | The mapping of predefined BR index used in quick access message to BR size and BR actions (see Table 789) | Present for UL |
| O | Q.1) Predefined BR index | 4 | Predefined BR index |  |
| O | Q.2) BR action | 2 | 0b00: ertPS service flow requests to resume to maximum sustained rate 0b01: aGP service flow requests to switch to Primary QoS parameters 0b10: BR 0b11: reserved |  |
| O | Q.3) BR size | 11 | Number of bytes requested by the predefined BR index | Present if BR action = 0b10 |
| O | S) Vendor Specific QoS Parameter | variable | Vendor Specific QoS Parameter | Present if needed |
| O | S.1) Vendor ID | 24 | vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. |  |
| O | Packet Classification Rule parameter | variable |  | Present if needed |
| O | I) Vendor Specific classification Parameter | variable | Compound | Present if needed |
| O | I.1) Vendor ID | 24 | vendor identification specified by the 3-byte, vendor-specific organizationally unique identifier of the SS or BS MAC address. |  |
| O | N) PHS rule field | variable |  | Present if needed |
| O | N.6) Vendor-specific PHS parameters | variable | Compound | Present if needed |
| O | ROHC Parameter | variable |  | Present if needed |
| O | fullNumE-MBSIdtoUpdateArr[1..8] | Variable |  | Present only if Num of E-MBS Zone ID to update > 0 |
| O | E) Service Flow Update Bitmap | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. | Presented only if Service_flow_update_indicator is set to 0b0. |
| O | F) fullServiceFlowUpdateBitmapArr[0°¶161..15] | variable | For each bit in the Service Flow Update Bitmap = 1, an enrey is present to indicate the corresponding service flow is updated | Presented only if Service_flow_update_indicator is set to 0b0, and Service Flow Update Bitmap > 1. |
| O | F.1) New_E-MBS ID and FID | Variable (16xN) 12 | New E-MBS IDs and FIDs corresponding to the Service Flow Update Bitmap to update where N is the number ones in the Service Flow Update Bitmap. New E-MBS identifier |  |
| M | F.2) FID | 4 | Flow ID |  |
| O | G) Number of E-MBS ID_FID Mappings (M) | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to update | Presented only if Service_flow_update_indicator is set to 0b1 |
| O | H) fullEMBSIdFidMappingArr[0...161..15] | variable | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. | Presented only if Service_flow_update_indicator is set to 0b1. |
| O | H.1) Current_E-MBS ID and FID and New_E-MBS ID and FID | Variable (32xM) 12 | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update. Based on the value of Number of E-MBS ID_FID Mappings Current E-MBS identifier | Presented only if Service_flow_update_indicator is set to 0b1. |
| O | H.2) Current FID | 4 | Current Flow ID |  |
| O | H.3) New_E-MBS ID | 12 | New E-MBS identifier |  |
| O | H.4) New FID | 4 | Current Flow ID |  |
| O | Group Parameter Create/Change | variable |  | Present if needed |
| O | D) Group FID Parameters List | variable |  |  |
| O | D.2) Non-common for Group Create/Change | variable | Non-common service flow encodings that are specific to individual service flows specified in Group FID Parameter List |  |
| O | Coupled Group Create/Change | variable |  |  |
| O | A) Common for Coupled Group | variable | Common service flow encodings that are common to all service flows specified in Coupled Group Create/Change parameter. |  |
| O | D) Coupled FID Parameter List | variable |  |  |
| O | D.2) Non-common for Coupled Group | variable | Non-common service flow encodings that are specific to individual service flows specified in Coupled FID Parameter List |  |

------------------------------------------------ End of proposed text IV --------------------------------------------------

II.4 Proposed text 5

------------------------------------------------- Start of proposed text V--------------------------------------------------

16.2.3.46.6 AAI_DSA-RSP

Table 746—AAI_DSC-RSP

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |
| O | SFID] | 32 | Service flow identifier |  |
| O | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure 0 = request was successful. 1 = request was failure |  |

------------------------------------------------- End of proposed text V --------------------------------------------------

II.6 Proposed text 6

------------------------------------------------- Start of proposed text VI--------------------------------------------------

16.2.3.46.6 AAI_DSC-ACK

Table 746—AAI_DSC-ACK

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |
| O | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure 0 = request was successful. 1 = request was failure |  |

------------------------------------------------- End of proposed text VI --------------------------------------------------

II.7 Proposed text 7

------------------------------------------------- Start of proposed text VII--------------------------------------------------

16.2.3.46.7 AAI_DSD-REQ

Table 747—AAI_DSD-REQ

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |
| O | Number of E-MBS ID_FID Mappings | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to delete | Present when ABS initiates AAI_DSD-REQ |
| O | fullEMBSIdFidArr[0..161..15] | Variable | Mapping of E-MBS ID and FID. | Present when ABS initiates AAI_DSD-REQ |
| M | A) E-MBS ID and FID | Variable (16xN) 12 | Mapping of E-MBS ID and FID. Based on the value of Number of E-MBS ID_FID Mappings E-MBS identifier |  |
| M | B) FID | 4 | Flow identifier |  |

------------------------------------------------- End of proposed text VII --------------------------------------------------

II.8 Proposed text 8

------------------------------------------------- Start of proposed text VIII--------------------------------------------------

16.2.3.46.8 AAI_DSD-RSP

Table 748—AAI_DSD-RSP

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Control Message Type | 8 |  | N.A. |
| O | Confirmation Code | 1 | Zero indicates the request was successful. Nonzero indicates failure 0 = request was successful. 1 = request was failure |  |

------------------------------------------------- End of proposed text VIII --------------------------------------------------