| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Delta Information Attributes in AAI_NBR-ADV and AAI_MC-ADV messages |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Joey Chou Xiangying Yang Shantidev Mohanty Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix Delta Information Attributes |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Delta Information Attributes in AAI_NBR-ADV and

- AAI_MC-ADV messages

- Joey Chou, Xiangying Yang, Shantidev Mohanty

- Intel

- Introduction

The Delta Information attributes in AAI_NBR-ADV and AAI_MC-ADV messages have the following issues:

- They are encoded as a sequence of TLVs that are not valid, as AAI_NBR-ADV and AAI_MC-ADV messages are encoded in ASN.1. The type is not needed for ASN.1 encoding.

- It indicated that parameters from Type 60 to Type 91 shall only be included in AAI_MC-ADV. But, the title of table 693 showa that the types are for delta SFH encoding. This is not true, since it contains type 60 to 91 that are derived from AAI_SCD message.

- Table 693 lists the delta encoding for SFH. But, it is not consistent with the SFH SP1, SP2, and SP3 formats as defined in Table 840, Table 841, and table 842, respectively. It is a maintenance issue that the update in one table is not reflected in another table.

To resolve the above issues, this contribution proposes to have the delta information attributes refer to SFH SP1, SP2, and SP3, and AAI_SCD messages, instead of having a separate table.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.12 AAI_NBR-ADV

[Page 117, line 6]

Table 692—AAI_NBR-ADV parameters

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Delta information | Variable. | Indicates the dDelta encoding, w.r.t. between the reference carrier and the current carrier where transmitting this message is transmitted if SFH_encoding format = 01, or between the reference carrier and the preceding carrier if SFH_encoding_format = 11) Delta information contains SFH SP1, SP2, and SP3 attributes as defined in Table 840, Table 841, and table 842, respectively. Each delta information attribute is optional. | Shall be included for each neighbor BS if SFH_encoding_format is set to 0b01 or 0b11 |

[Page 117, line 39]

Delta information is encoded as a sequence of TLVs. The encoding of the one-byte type field for S-SFH elements can be found in Table 693—. Parameters from Type 60 to Type 91 shall only be included in AAI_MC-ADV. The length field should equal the element length as specified in the S-SFH, rounded up to an integer number of bytes. The delta information field should be padded so its length will be an integer number of bytes

Table 693 –Delta Information Type values for delta SFH encoidng in AAI_NBR-ADV and AAI_MC-ADV

| Type | Names |
| --- | --- |
| 1 | LSBs of 48 bit ABS MAC ID |
| 2 | Number of UL ACK/NACK channels per HARQ feedback region |
| 3 | Number of DL ACK/NACK channels per HF-A-MAP region |
| 4 | Power control channel resource size indicator |
| 5 | DL Primary frequency partition location |
| 6 | A-A-MAP MCS selection |
| 7 | DCASSB0 |
| 8 | DCASMB0 |
| 9 | DCASi |
| 10 | Frame configuration index |
| 11 | WirelessMAN-OFDMA support |
| 12 | Allocation periodicity of ranging channel |
| 13 | Subframe offset of ranging channel |
| 14 | Start code information of ranging channel |
| 15 | Ranging preamble code partition information |
| 16 | UL_Permbase |
| 17 | Allocation periodicity of ranging channel for synchronized AMSs |
| 18 | Start code information of ranging channel for synchronized AMSs |
| 19 | Allocation periodicity of ranging channel for non-synchronized AMSs |
| 20 | Start code information of ranging channel for non-synchronized AMSs |
| 21 | Ranging preamble code partition information for non-synchronized AMSs |
| 22 | Number of cyclic shifted ranging preamble codes per root index for non-synchronized AMSs |
| 23 | Ranging channel formats for non-synchronized AMSs |
| 24 | UCASSB0 |
| 25 | UCASMB0 |
| 26 | UCASi |
| 27 | Uplink AAI subframes for sounding |
| 28 | ABS EIRP |
| 29 | Cell bar information |
| 30 | UL_N_MAX_ReTx |
| 31 | DL_N_MAX_ReTx |
| 32 | TUL_Rx_Processing |
| 33 | UL carrier frequency |
| 34 | UL bandwidth |
| 35 | MSB bytes of 48 bit ABS MAC ID |
| 36 | MAC protocol revision |
| 37 | DSAC |
| 38 | DFPC |
| 39 | DFPSC |
| 40 | USAC |
| 41 | UFPC |
| 42 | UFPSC |
| 43 | AMS Transmit Power Limitation Level |
| 44 | EIRxPIR,min |
| 45 | Rate of change of S-SFH info |
| 46 | SA-Preamble sequence soft partitioning information |
| 47 | FFR partition resource metrics |
| 48 | IoT correction value for UL power control |
| 49 | Number of Distributed LRUs for UL feedback channel per a UL AAI subframe |
| 50 | # Tx antenna |
| 51 | SP scheduling periodicity information |
| 52 | HO Ranging backoff start |
| 53 | HO Ranging backoff end |
| 54 | Initial ranging backoff start |
| 55 | Initial ranging backoff end |
| 56 | UL BW REQ channel information |
| 57 | Bandwidth request backoff start |
| 58 | Bandwidth request backoff end |
| 59 | fpPowerConfig |
| 60 | BS_Restart_Count |
| 61 | SA_PreamblePartitionforBStype |
| 62 | Trigger encoding |
| 63 | DefaultTrigger_Averagingparameter |
| 64 | NormalizedCINR |
| 65 | OL-Region-ON |
| 66 | OL-Rank1-Configuration |
| 67 | SB-OL-Region-2Size |
| 68 | periodicityOfRngChSync |
| 69 | cntlStartCodeOfRngChSync |
| 70 | rangingPreambleCodeSync |
| 71 | GAMMA_IOT_FP0 |
| 72 | GAMMA_IOT_FP1 |
| 73 | GAMMA_IOT_FP2 |
| 74 | GAMMA_IOT_FP3 |
| 75 | Alpha (a) |
| 76 | Beta (??) |
| 77 | SINRmin_Data |
| 78 | SINRmax_Data |
| 79 | SINR_Target_HARQ |
| 80 | SINR_Target_SyncRanging |
| 81 | SINR_Target_PFBCH |
| 82 | SINR_Target_SFBCH_BASE |
| 83 | SINR_Target_SFBCH_Delta |
| 84 | SINR_Target_BWRequest |
| 85 | GAMMA_IOT_SOUNDING |
| 86 | SINRmin_SOUNDING |
| 87 | T_ReTx_Interval |
| 88 | BR_Channel Configuration MIN Access Class |
| 89 | SoundingMultiplexingType |
| 90 | shiftValueUForSoundingSymbol |
| 91 | 16m_Relay_zone_AMS_allocation_indicator |

------------------------------------------------- End of proposed text I--------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II --------------------------------------------------

16.2.3.56 AAI_MC-ADV

[Page 222, line 15]

Table 761—AAI_MC-ADV parameters

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Delta information | Variable | Indicates dDelta encoding w.r.t. between the reference carrier and (the current carrier where transmitting this message is transmitted if SFH_encoding_format = 0b01, or between the reference carrier and the preceding carrier if SFH_encoding_format = 0b10) Delta information may contain SFH SP1, SP2, and SP3 attributes as defined in Table 840, Table 841, and table 842, respectively. It may contain the following AAI_SCD attributes: - BS_Restart_Count - SA_PreamblePartitionforBStype - Trigger definitions - NormalizedCINR - OL-Region-Type0-ON - OL-Region-Type2-SLRU-Size - periodicityOfRngChSync - cntlStartCodeOfRngChSync - rangingPreambleCodeSync - UlpcDataChannelSet - gammaIotFp0 - gammaIotFp1 - gammaIotFp2 - gammaIotFp3 - Alpha - Beta - dataSinrMin - dataSinrMax - targetHarqSinr - targetSyncRangingSinr - targetPfbchSinr - targetSfbchBaseSinr - targetSfbchDeltaSinr - targetBwRequestSinr - gammaIotSounding - soundingSinrMin - T_ReTx_Interval - BR_Channel Configuration MIN Access Class of the (i+0)-th frame - BR_Channel Configuration MIN Access Class of the (i+1)-th frame - BR_Channel Configuration MIN Access Class of the (i+2)-th frame - BR_Channel Configuration MIN Access Class of the (i+3)-th frame - SoundingMultiplexingType - decimationValueD - maxCyclicShiftIndexP - shiftValueUForSoundingSymbol - 16m_Relay_zone_AMS_allocation_indicator Each delta information attribute is optional. | May be present when SFH encoding format is set to 0b01 or 0b10 |

------------------------------------------------- End of proposed text II --------------------------------------------------