| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up AAI_REG-REQ and AAI_REG-RSP attributes |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Joey Chou Shaocheng Wang Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_REG-REQ and AAI_REG-RSP issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clean up AAI_REG-REQ and AAI_REG-RSP attributes

- Joey Chou, Shaocheng Wang

- Intel

- Introduction

This contribution is intended to clean up AAI_REG-REQ and AAI_REG-RSP ttributes.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.7 AAI_REQ-REQ

An AAI_REG-REQ message is transmitted by AMS to negotiate general AMS capabilities and do registration during network entry.

The AAI_REG-REQ message shall be encrypted and not contain CMAC Tuple if authentication has been completed.

The following parameters may be included in AMS capability negotiation parameters of AAI_REG-REQ.

•AMS initiated aGP Service Adaptation Capability:

0b0: no support

0b1: support

Table 687—AAI_REG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS | 10 | the number of unique SN values |  |
| O |  | ARQ Parameters | ARQ_WINDOW_SIZE | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks |  |
| O |  | ARQ Parameters | ARQ_SUB_BLOCK_SIZE |  | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  | ARQ Parameters | ARQ_BLOCK_LIFETIME | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |  |
| O |  | ARQ Parameters | ARQ_RX_PURGE_TIMEOUT | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START |  |
| O |  | ARQ parameters | MAXIMUM_ ARQ_BUFFER_SIZE |  | The maximum ARQ buffer size Depend on AMS’s memory |  |
| O |  | Multicarrier capabilities |  | 3 | 0b000: No MC modes 0b001: Basic MC mode 0b010: Multicarrier Aggregation 0b011: Multicarrier Switching 0b100: Both Multicarrier Aggregation and Multicarrier Switching |  |
|  |  | Capabilities for Zone switching mode | Zone Switch Mode Support |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported |  |
|  |  | Capabilities for Zone switching mode | Reserved |  |  |  |
| O |  | LBS Capabilities | Capability for supporting A-GPS method | 1 | 0b0: no support |  |
| O |  | Capabilities for interference mitigation support | DL PMI coordination capability | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable |  |
|  |  |  | DL collaborative multi-BS MIMO capability | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable |  |
|  |  |  | DL closed-loop multi-BS macro diversity capability | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable |  |
| O |  | Capabilities for interference mitigation support | UL PMI combination capability | 1 | 0: AMS is not UL PMI combination capable |  |
| O |  | E-MBS capabilities |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O |  | Channel BW and Cyclic prefix |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) |  |
| O |  | frame configuration to support legacy(5MHz) |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | frame configuration to support legacy(10MHz) |  | 1 | If Bit#0 =1,it supports(TDD only) |  |
| O |  | OFDMA system support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | MAX Tx Power |  | 24 | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. |  |
| O |  | 3-step BR |  | 1 | If Bit#0 =1,it supports |  |
| O |  | dynamic ranging for HO |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Persistent Allocation support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Group Resource Allocation support |  | 1 | If Bit#0 =1,it supports |  |
| O |  | Co-located coexistence capability support |  | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class |  |
| O |  | HO trigger metric |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports |  |
| O |  | Capabilities for Handover EBB mode | EBB support | 1 | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported |  |
| O |  | Capabilities for Handover EBB mode | Minimal HO_Reentry_Interleaving_Interval |  | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1 |  |
| O | AMS capability negotiation parameters |  |  | Variable |  | Present if needed |
| O | A) ARQ parameters |  |  | Variable |  | Present if needed |
| O | A.1) ARQ_SN_MODULUS |  |  | 10 | the number of unique SN values | Present if needed |
| O | A.2) ARQ_WINDOW_SIZE |  |  | 5 | the maximum number of ARQ blocks with consecutive SN in the sliding window of ARQ blocks | Present if needed |
| O | A.3) ARQ_SUB_BLOCK_SIZE |  |  | 3 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.4) ARQ_BLOCK_LIFETIME |  |  | 16 | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement | Present if needed |
| O | A.5) ARQ_RX_PURGE_TIMEOUT |  |  | 16 | the time interval the receiver shall wait after successful reception of a block that does not result in advancement of ARQ_RX_WINDOW_START, before advancing ARQ_RX_WINDOW_START | Present if needed |
| O | A.6) MAXIMUM_ARQ_BUFFER_SIZE |  |  | 2048 x 8 | The maximum ARQ buffer size allowed by the system. The actual maximum ARQ buffer size depend on AMS’s memory, and shall be below 2048 bytes. | Present if needed |
| O | B) Multicarrier capabilities |  |  | 3 | 0b000: No MC modes 0b001: Basic MC mode 0b010: Multicarrier Aggregation 0b011: Multicarrier Switching 0b100: Both Multicarrier Aggregation and Multicarrier Switching | Present if needed |
| O | C) Capabilities for Zone switching mode |  |  |  |  | Present if needed |
| O | C.1) Zone Switch Mode Support |  |  | 1 | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported | Present if needed |
| O | D) LBS Capabilities |  |  |  | Indicates whether the AMS has a capability to maintain its data communication with the ABS in LZone while performing network reentry in MZone 0b0: Not supported 0b1: Supported | Present if needed |
| O | D.1) Capability for supporting A-GPS method |  |  | 1 | 0b0: no support 0b1: support | Present if needed |
| O | E) Capabilities for interference mitigation support |  |  |  |  | Present if needed |
| O | E.1) DL PMI coordination capability |  |  | 1 | 0: ABS is not DL PMI coordination capable 1: ABS is DL PMI coordination capable | Present if needed |
| O | E.2) DL collaborative multi-BS MIMO capability |  |  | 1 | 0: ABS is not DL collaborative multi-BS MIMO capable 1: ABS is DL collaborative multi-BS MIMO capable | Present if needed |
| O | E.3) DL closed-loop multi-BS macro diversity capability |  |  | 1 | 0: ABS is not DL closed-loop multi-BS macro diversity capable 1: ABS is DL closed-loop multi-BS macro diversity capable | Present if needed |
| O | E.4) UL PMI combination capability |  |  | 1 | 0: ABS is not UL PMI combination capable 1: ABS is UL PMI combination capable | Present if needed |
| O | F) E-MBS capabilities |  |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0~Bit#2 are set to 0, it indicates no E-MBS is supported. | Present if needed |
| O | G) Channel BW and Cyclic prefix |  |  | 15 | If Bit#0 =1, 5MHz supports(1/16 CP) If Bit#1 =1, 5MHz supports(1/8 CP) If Bit#2 =1, 5MHz supports(1/4 CP) If Bit#3 =1, 10MHz supports(1/16 CP) If Bit#4 =1, 10MHz supports(1/8 CP) If Bit#5 =1, 10MHz supports(1/4 CP) If Bit#6 =1, 20MHz supports(1/16 CP) If Bit#7 =1, 20MHz supports(1/8 CP) If Bit#8 =1, 20MHz supports(1/4 CP) If Bit#9 =1, 8.75MHz supports(1/16 CP) If Bit#10 =1, 8.75MHz supports(1/8 CP) If Bit#11 =1, 8.75MHz supports(1/4 CP) If Bit#12 =1, 7MHz supports(1/16 CP) If Bit#13 =1, 7MHz supports(1/8 CP) If Bit#14 =1, 7MHz supports(1/4 CP) | Present if needed |
| O | H) frame configuration to support legacy(5MHz) |  |  | 1 | If Bit#0 =1,it supports(TDD only) | Present if needed |
| O | I) frame configuration to support legacy(10MHz) |  |  | 1 | If Bit#0 =1,it supports(TDD only) | Present if needed |
| O | J) OFDMA system support |  |  | 1 | If Bit#0 =1,it supports | Present if needed |
| O | K) MAX Tx Power |  |  | 24 | Bit 0-7: Maximum transmitted power for QPSK. Bit 8-15: Maximum transmitted power for 16-QAM Bit 15-23: Maximum transmitted power for 64-QAM. | Present if needed |
| O | L) 3-step BR |  |  | 1 | If Bit#0 =1,it supports | Present if needed |
| O | M) dynamic ranging for HO |  |  | 1 | If Bit#0 =1,it supports | Present if needed |
| O | N) Persistent Allocation support |  |  | 1 | If Bit#0 =1,it supports | Present if needed |
| O | O) Group Resource Allocation support |  |  | 1 | If Bit#0 =1,it supports | Present if needed |
| O | P) Co-located coexistence capability support |  |  | 5 | If Bit#0 =1, Type I CLC class support If Bit#1 =1, Type II CLC class subtype 1 If Bit#2 =1, Type II CLC class subtype 2 If Bit#3 =1, Type II CLC class subtype 3 If Bit#4 =1, Type III CLC class | Present if needed |
| O | Q) HO trigger metric |  |  | 4 | If Bit#0 =1, BS CINR mean supports If Bit#1 =1, BS RSSI mean supports If Bit#2 =1, Relative delay supports If Bit#3 =1, BS RTD supports | Present if needed |
| O | O) Capabilities for Handover EBB mode |  |  |  |  | Present if needed |
| O | O.1) EBB support |  |  | 1 | Indicates whether the AMS has a capability to support EBB operation during handover 0b0: Not supported 0b1: Supported | Present if needed |
| O | R) Capabilities for Handover EBB mode |  |  |  |  | Present if needed |
|  | R.1) Minimal HO Reentry Interleaving Interval |  |  | ? | The minimal HO_Reentry_Interleaving_Interval measured in frames. For MC HO capable AMS, this value shall be 0. Shall be included only if EBB support is set to 1 | Present if needed |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  |  | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O |  | Maximum number of classification rules |  |  | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | IP-Service-Type IE |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved |  |
| O | Convergence sublayer capabilities |  |  |  |  | Present if needed |
| O | A) CS type Classification/PHS options and SDU encapsulation support |  |  | 32 | A bit set to “1” Indicates which CS Type the AMS supports Bit #0: GPCS (Generic Packet Convergence Sublayer) Bit #1: Packet, IPv4 Bit #2: Packet, IPv6 Bit #3: Packet, IEEE 802.3/Etherneta Bit #4: Reserved Bit #5: Packet, IPv4 over IEEE 802.3/Etherneta Bit #6: Packet, IPv6 over IEEE 802.3/Etherneta Bit #7: Reserved Bit #8: Reserved Bit #9: ATM Bit #10: Reserved Bit #11: Reserved Bit #12: Reserved Bit #13: Reserved Bit #14: Packet, IPb Bit #15: Multiprotocol flow Bit #16 - Bit #31 Reserved | Present if needed |
| O | B) Maximum number of classification rules |  |  | 16 | Maximum number of simultaneous admitted classification rules that a service flow supports. | Present if needed |
| O | C) ROHC support |  |  | 1 | 0: not support 1: support | Present if needed |
| O | D) PHS support |  |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if needed |
| O | E) IP-Service-Type IE |  |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | Present if needed |
| O | Host configuration capabilities and parameters | Host-Configuration-Capability-Indicator IE |  | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. |  |
| O | Host configuration capabilities and parameters | Requested-Host-Configurations IE |  | 1 | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. |  |
| O | Host configuration capabilities and its parameters |  |  | Variable |  | Present if needed |
| O | A) Host-Configuration-Capability-Indicator IE |  |  | 1 | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) This shall be omitted if Requested-Host-Configurations IE is included in the message. | Present if needed |
| O | B) Requested-Host-Configurations IE |  |  | 1 | Includes requested host configuration options in DHCP Options format. If included, this IE indicates that the AMS supports host configuration using AAI_REG-RSP message, and Host-Configuration-Capability-Indicator IE shall be omitted. | Present if needed |
| O | Global carrier configuration change count |  |  | 3 | Indicates the AMS's last received value of Global carrier configuration change count of the network. If set to 0, it implies that AMS never received a Global carrier configuration information. | Present if needed |
| O | E-MBS capabilities |  |  | 3 | If Bit#0 is set to 1, it indicates E-MBS in Serving ABS only is supported If Bit#1 is set to 1, it indicates macro-diversity multi ABS E-MBS is supported. If Bit#2 is set to 1, it indicates non-macro-diversity multi ABS E-MBS is supported. If all Bit#0-Bit#2 are set to 0, it indicates no E-MBS is supported. |  |
| O | AMS initiated aGP Service Adaptation Capability: |  |  | 1 | 0b0: no support 0b1: support | N.A. |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.3.7 AAI_REQ-RSP

Table 688—AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| O | Convergence sublayer capabilities | Classification/PHS options and SDU encapsulation support |  |  | Indicates which classification/PHS options and SDU encapsulation the AMS supports |  |
| O |  | Maximum number of classification rules |  |  | Maximum number of simultaneous admitted classification rules that a service flow supports. |  |
| O |  | ROHC support |  | 1 | 0: not support 1: support |  |
| O |  | PHS support |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS |  |
| O |  | Resource_Retain_Time |  | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context |  |
| O |  | IP-Service-Type IE |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved |  |
| O | Convergence sublayer capabilities |  |  |  |  | Present if needed |
| O | A) CS type Classification/PHS options and SDU encapsulation support |  |  | 32 | A bit set to “1” Indicates which CS Type the AMS supports Bit #0: GPCS (Generic Packet Convergence Sublayer) Bit #1: Packet, IPv4 Bit #2: Packet, IPv6 Bit #3: Packet, IEEE 802.3/Etherneta Bit #4: Reserved Bit #5: Packet, IPv4 over IEEE 802.3/Etherneta Bit #6: Packet, IPv6 over IEEE 802.3/Etherneta Bit #7: Reserved Bit #8: Reserved Bit #9: ATM Bit #10: Reserved Bit #11: Reserved Bit #12: Reserved Bit #13: Reserved Bit #14: Packet, IPb Bit #15: Multiprotocol flow Bit #16 - Bit #31 Reserved | Present if needed |
| O | B) Maximum number of classification rules |  |  | 16 | Maximum number of simultaneous admitted classification rules that a service flow supports. | Present if needed |
| O | C) ROHC support |  |  | 1 | 0: not support 1: support | Present if needed |
| O | D) PHS support |  |  | 1 | Indicates the level of PHS support. 0: Reserved 1: Packet PHS | Present if needed |
| O | E) Resource_Retain_Time |  |  | 16 | 0-65535: In units of 100 milliseconds Duration that the serving ABS shall retain the AMS context | Present if needed |
| O | F) IP-Service-Type IE |  |  | 2 | Indicates which IP service will be used, the value is one of following: 0b00: IPv4-Service 0b01:IPv6-Service 0b10:IPv4/IPv6-Dual-Mode-Service 0b11:reserved | Present if needed |
| O | Host configuration capabilities and parameters |  |  | Variable |  |  |
| O | A) IPv4-Host-Address IE |  |  | 32 | The allocated IPv4 Host Address for the AMS. | Present if needed |
| O | B) IPv6-Home-Network-Prefix IE |  |  | 64 | The allocated IPv6 Home Network Prefix for the AMS. | Present if needed |
| O | C) Additional-Host-Configurations IE |  |  | ??? | Includes additional host configuration options in DHCP Options format | Present if needed |
| O | Redirection Info |  | ABSID, preamble index and center frequency for one or more neighbor ABS |  | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. |  |
| O | Redirection Info |  |  |  |  | Present if needed |
| O | A) ABSID, preamble index and center frequency for one or more neighbor ABS |  |  | ??? | Sent by serving ABS to aid cell reselection in the event the serving ABS is not able to allow the AMS to perform entry. | Present if needed |
| O | Global carrier configuration change count |  |  | 3 | Indicates the current value of Global carrier configuration change count of the network | Present if needed |
| O | AMS initiated aGP Service Adaptation Capability: |  |  | 1 | 0b0: no support 0b1: support | N.A. |

------------------------------------------------- End of proposed text II--------------------------------------------------