| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up AAI_RNG-REQ and AAI_RNG-RSP attributes |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Joey Chou Shaocheng Wang Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_RNG-REQ and AAI_RNG-RSP issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clean up AAI_RNG-REQ and AAI_RNG-RSP attributes

- Joey Chou, Shaocheng Wang

- Intel

- Introduction

This contribution is intended to clean up AAI_RNG-REQ and AAI_RNG-RSP ttributes.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.1 AAI_RNG-REQ

Table 679—AAI_RNG-REQ message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | CSGID(s) |  |  | One or more identifiers for one or more CSG(s) | Sent by AMS to aid quick CSG membership detection by Femto ABS as well as to aid ABS reselection |
| O | csgIdArray [1..15] |  | Variable | List of CSG ID | Present if sent by AMS for quick CSG membership detection or ABS reselection assistance |
| M | A) CSGID |  | 8 | CSG identifier |  |
|  | CMAC tuple | PMK SN | 4 | The sequence number of PMK used to derive the current AK. | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication. |
|  |  | CMAC_PN | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
|  |  | CMAC value | 64 | The CMAC value derived from the whole message. |  |
| O | List of FIDs, and Change Count of each FID |  |  | List of FIDs which the AMS currently has, and Change Count of each FID perEachFID SEQUENCE { fid INTEGER (0 .. 15), fidchangecount INTEGER (0 .. 15) } | Shall be included if Ranging Purpose Indication value is set to 0b0000 and Paging Controller ID is absent, |
| O | fidArray [1..15] |  | Variable | List of FID | Present if Ranging Purpose Indication value is set to 0b0000 and Paging Controller ID is absent, |
|  | A) FID |  | 4 | Flow identifier |  |
|  | B) FID change count |  | 4 | FID change count |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.3.2 AAI_RNG-RSP

Table 680—AAI_RNG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | FID_update | SFID | 32 | SFID which need to update existing flow. | It shall be included if the ABS needs to update AMS's existing flows. FIDs which are not appeared in this field shall be regarded as guranteed by the ABS. |
|  |  | Update or delete | 1 | 0: delete the flow |  |
|  |  | Updated QoS Info | Note: refer to DSA message | All the rules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsulated in this field. |  |
| O | FIG update |  | Variable |  | It shall be included if the ABS needs to update AMS's existing flows. FIDs which are not appeared in this field shall be regarded as guranteed by the ABS. |
| O | A) SFID |  | 32 | SFID which need to update existing flow. |  |
| O | B) Update or delete |  | 1 | 0: delete the flow |  |
| O | C) Updated QoS Info |  | Note: refer to DSA message | All the rules and settings that apply to the parameters when used in the DSC-RSP message apply to the contents encapsulated in this field. |  |
| O | CLC-INFO Response |  | Note: refer to CLC messages Variable | The information of co-located coexistence response (as defined in Table 693 701) | It shall be included when the AMS is attempting network reentry after HO if the AMS has any Type I or II CLC class active before handover. The Information Type field of CLC-INFO shall be set to 2 to indicate it provides CLC response information. |
| O | InvalidFidArray [1..15] |  | Variable | List of invalid FID | Shall be included if 'Invalidity indication for FIDs' is set to 1. ABS and AMS may exchange the AAI_DSx-messages for invalid FIDs |
| O | A) FID |  | 4 | Flow identifier |  |
| O | fullNumE-MBSIdtoUpdateArr[1..8] |  | Variable |  |  |
| O | E) Service Flow Update Bitmap |  | 16 | E-MBS Service Flow update bitmap where the E-MBS ID+FID is sorted with increasing order between ABS and AMS. A bit = 1, means the corresponding service flow, identified by E-MBS ID and FID, is updated. | Presented only if Service_flow_update_indicator is set to 0b0. |
| O | F) fullServiceFlowUpdateBitmapArr[0°¶161..15] |  | variable | For each bit in the Service Flow Update Bitmap = 1, an enrey is present to indicate the corresponding service flow is updated | Presented only if Service_flow_update_indicator is set to 0b0, and Service Flow Update Bitmap > 1. |
| O | F.1) New_E-MBS ID and FID |  | Variable (16xN) 12 | New E-MBS IDs and FIDs corresponding to the Service Flow Update Bitmap to update where N is the number ones in the Service Flow Update Bitmap. New E-MBS identifier |  |
| O | F.2) FID |  | 4 | Flow ID |  |
| O | G) Number of E-MBS ID_FID Mappings (M) |  | 4 | Number of E-MBS ID_FID Mappings in the E-MBS_Zone_ID to update | Presented only if Service_flow_update_indicator is set to 0b1 |
| O | H) fullEMBSIdFidMappingArr[0...161..15] |  | variable | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update the service flow. | Presented only if Service_flow_update_indicator is set to 0b1. |
| O | H.1) Current_E-MBS ID and FID and New_E-MBS ID and FID |  | Variable (32xM) 12 | Mapping of current E-MBS ID and FID and new E-MBS ID and FID to update. Based on the value of Number of E-MBS ID_FID Mappings Current E-MBS identifier | Presented only if Service_flow_update_indicator is set to 0b1. |
| O | H.2) Current FID |  | 4 | Current Flow ID |  |
| O | H.3) New_E-MBS ID |  | 12 | New E-MBS identifier |  |
| O | H.4) New FID |  | 4 | Current Flow ID |  |

------------------------------------------------- End of proposed text II--------------------------------------------------