| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Disabling Subframes to Mitigate Interference between Femto ABSs (16.4.11) |  |
| Date Submitted | 2010-07-01 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.4.11 (Interference Avoidance and Interference Mitigation) |  |
| Abstract | This contribution proposes changes to the Section 16.4.11 of IEEE P802.16m/D6 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Disabling Subframes to Mitigate Interference between Femto ABSs (16.4.11)

Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu

ITRI

- Introduction

In current D6, to mitigate the interference between Femto ABSs, it is said that “Femto ABS may disable some of its subframes and announce the disabled subframes via AAI_SON-ADV”. However, there is no parameter in the AAI_SON-ADV message enabling this function. In this contribution, we propose a new parameter, “Disabled subframe index”, in the AAI_SON-ADV message to enable this function. We also provide the mappings between the values of this index and the subframe status, these mappings fully meet the HARQ timing criteria.

- Proposed Text for IEEE P802.16m/D6

[Accept the following modifications marked in blue]

============================ Start of Proposed Text ==================================

[Insert the blue text into line 27, page 94 (16.2.3.6)]

#### 16.2.3.6 AAI_SON-ADV message

Table 686—Parameters for AAI_SON-ADV message

| M/O | Attributes/Array of attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action type | 2 | Used to indicate the purpose of this message 0b00: ABS Reconfiguration 0b01: ABS Restart 0b10: ABS Scanning 0b11: ABS Reliability | mandatory |
| O | Unavailable start time (UST) |  | Start of unavailable time | If Action type = 0b00, 0b01, 0b10), it is mandatory. If Action type =0b11, it is optional. |
| O | Unavailable Time Interval (UTI) |  | Interval of unavailable time |  |
| O | Reason |  | 0b000: Power down 0b001: Power reduction 0b010: FA change 0b011: Backhaul link down 0b100: Disable subframes | If Action type=0b11, it is mandatory, otherwise it will not exist. |
| O | Tx power reduction |  | dB value of Tx power reduction | If Reason = 0b001, it is mandatory. |
| O | FA index |  | FA index | If Reason = 0b010, it is mandatory. |
| O | Disabled subframes index |  | The mapping between the value of this index and subframe status is listed in Table XXX. | If Reason = 0b100, it is mandatory. |
| O | Time of power down |  | Expected power down time or resource adjustment time or current FA downtime | If Action type = 0b11), it is mandatory. |
| O | Expected uptime or new FA uptime |  |  | If Action type = 0b11), it is optional. |
| O | New IDCell |  | New IDCell that the ABS will use after the reconfiguration process. | If Action type=0b00, it is optional, otherwise, it will not exist. |
| O | Recommended BSID list |  | Recommended BSID list | It is optional to help AMS to HO. |

[Insert the blue text into line 58, page 798 (16.4.11)]

#### 16.4.11 Interference Avoidance and Interference Mitigation

The interference between Femto and/or macro can be mitigated by static or semi-static radio resource reservation and resource sharing using FDM and/or TDM manner and/or DL power control. While using the TDM manner, Femto ABS may disable some of its subframes and periodically announce the disabled subframes via “Disabled subframes index” in the AAI_SON-ADV message. Femto ABS shall keep the midambles, preambles, and common pilots in the disabled subframes. The mappings between the index values and the subframes status are listed in Table XXX.

Table XXX – The mappings between the values of Disable subframes index and the subframe status.

| No. | Number of subframes per frame | Duplex & D:U ratio | Disabled subframes index | Subframe Status |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | #0 | #1 | #2 | #3 | #4 | #5 | #6 | #7 |
| 1 | 8 | TDD (6:2) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |
| 2 | 8 | TDD (6:2) | 1 | Enabled | Enabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled |
| 3 | 8 | TDD (6:2) | 2 | Disabled | Disabled | Disabled | Enabled | Enabled | Enabled | Disabled | Enabled |
| 4 | 8 | TDD (5:3) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |
| 5 | 8 | TDD (5:3) | 1 | Enabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled | Disabled |
| 6 | 8 | TDD (5:3) | 2 | Disabled | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled |
| 7 | 8 | TDD (5:3) | 3 | Disabled | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled | Enabled |
| 8 | 8 | TDD (4:4) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |
| 9 | 8 | TDD (4:4) | 1 | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled | Disabled | Disabled |
| 10 | 8 | TDD (4:4) | 2 | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled | Disabled |
| 11 | 8 | TDD (4:4) | 3 | Disabled | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled |
| 12 | 8 | TDD (4:4) | 4 | Disabled | Disabled | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled |
| 13 | 8 | TDD (3:5) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |
| 14 | 8 | TDD (3:5) | 1 | Enabled | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled | Disabled |
| 15 | 8 | TDD (3:5) | 2 | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled | Disabled |
| 16 | 8 | TDD (3:5) | 3 | Disabled | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled | Enabled |
| 17 | 8 | FDD (DL) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |
| 18 | 8 | FDD (DL) | 1 | Enabled | Enabled | Disabled | Disabled | Disabled | Disabled | Disabled | Disabled |
| 19 | 8 | FDD (DL) | 2 | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled | Disabled | Disabled |
| 20 | 8 | FDD (DL) | 3 | Disabled | Disabled | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled |
| 21 | 8 | FDD (DL) | 4 | Disabled | Disabled | Disabled | Disabled | Disabled | Disabled | Enabled | Enabled |
| 22 | 7 | TDD (5:2) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |
| 23 | 7 | TDD (5:2) | 1 | Enabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled |  |
| 24 | 7 | TDD (5:2) | 2 | Disabled | Disabled | Enabled | Enabled | Enabled | Disabled | Enabled |  |
| 25 | 7 | TDD (4:3) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |
| 26 | 7 | TDD (4:3) | 1 | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled | Disabled |  |
| 27 | 7 | TDD (4:3) | 2 | Disabled | Enabled | Disabled | Disabled | Not used | Enabled | Disabled |  |
| 28 | 7 | TDD (4:3) | 3 | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled | Enabled |  |
| 29 | 7 | TDD (3:4) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |
| 30 | 7 | TDD (3:4) | 1 | Enabled | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled |  |
| 31 | 7 | TDD (3:4) | 2 | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled | Disabled |  |
| 32 | 7 | TDD (3:4) | 3 | Disabled | Disabled | Enabled | Disabled | Disabled | Disabled | Enabled |  |
| 33 | 7 | FDD (DL) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |
| 34 | 7 | FDD (DL) | 1 | Enabled | Enabled | Disabled | Disabled | Disabled | Disabled | Disabled |  |
| 35 | 7 | FDD (DL) | 2 | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled | Disabled |  |
| 36 | 7 | FDD (DL) | 3 | Disabled | Disabled | Disabled | Disabled | Enabled | Enabled | Disabled |  |
| 37 | 7 | FDD (DL) | 4 | Disabled | Disabled | Disabled | Disabled | Disabled | Disabled | Enabled |  |
| 38 | 6 | TDD (4:2) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |
| 39 | 6 | TDD (4:2) | 1 | Enabled | Enabled | Disabled | Disabled | Enabled | Disabled |  |  |
| 40 | 6 | TDD (4:2) | 2 | Disabled | Disabled | Enabled | Enabled | Disabled | Enabled |  |  |
| 41 | 6 | TDD (3:3) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |
| 42 | 6 | TDD (3:3) | 1 | Enabled | Disabled | Disabled | Enabled | Disabled | Disabled |  |  |
| 43 | 6 | TDD (3:3) | 2 | Disabled | Enabled | Disabled | Disabled | Enabled | Disabled |  |  |
| 44 | 6 | TDD (3:3) | 3 | Disabled | Disabled | Enabled | Disabled | Disabled | Enabled |  |  |
| 45 | 6 | TDD (2:4) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |
| 46 | 6 | TDD (2:4) | 1 | Enabled | Disabled | Enabled | Enabled | Disabled | Disabled |  |  |
| 47 | 6 | TDD (2:4) | 2 | Disabled | Enabled | Disabled | Disabled | Enabled | Enabled |  |  |
| 48 | 6 | FDD (DL) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |
| 49 | 6 | FDD (DL) | 1 | Enabled | Enabled | Disabled | Disabled | Disabled | Disabled |  |  |
| 50 | 6 | FDD (DL) | 2 | Disabled | Disabled | Enabled | Enabled | Disabled | Disabled |  |  |
| 51 | 6 | FDD (DL) | 3 | Disabled | Disabled | Disabled | Disabled | Enabled | Enabled |  |  |
| 52 | 5 | TDD (3:2) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |  |
| 53 | 5 | TDD (3:2) | 1 | Enabled | Disabled | Disabled | Enabled | Disabled |  |  |  |
| 54 | 5 | TDD (3:2) | 2 | Disabled | Enabled | Enabled | Disabled | Enabled |  |  |  |
| 55 | 5 | TDD (2:3) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |  |
| 56 | 5 | TDD (2:3) | 1 | Enabled | Disabled | Enabled | Enabled | Disabled |  |  |  |
| 57 | 5 | TDD (2:3) | 2 | Disabled | Enabled | Disabled | Disabled | Enabled |  |  |  |
| 58 | 5 | FDD (DL) | 0 | Enabled | Enabled | Enabled | Enabled | Enabled |  |  |  |
| 59 | 5 | FDD (DL) | 1 | Enabled | Enabled | Disabled | Disabled | Disabled |  |  |  |
| 60 | 5 | FDD (DL) | 2 | Disabled | Disabled | Enabled | Enabled | Disabled |  |  |  |
| 61 | 5 | FDD (DL) | 3 | Disabled | Disabled | Disabled | Disabled | Enabled |  |  |  |

============================== End of Proposed Text ================================

References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, May 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.