| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Initiating DCR Mode from ABS Side (16.2.19) |  |
| Date Submitted | 2010-07-01 |  |
| Source(s) | Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu ITRI | Voice: +886-3-5914629 E-mail: MHTao@itri.org.tw |
| Re: | Category: SB_16m / Area: Chapter 16.2.19 (DCR mode) |  |
| Abstract | This contribution proposes changes to the Section 16.2.19 of IEEE P802.16m/D6 |  |
| Purpose | To be discussed and adopted by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Initiating DCR Mode from ABS Side (16.2.19)

Ming-Hung Tao, Ying-Chuan Hsiao, Yu-Min Chiu

ITRI

- Introduction

In current D6, DCR mode can be only initiated by the AMS, there is no way for the ABS to trigger the DCR Mode. In this contribution, we add 0x10 to the action code in AAI_DREG-RSP to allow the ABS-initiated procedure for DCR Mode. The ABS-initiated DCR Mode is useful for the following case: When an AMS is almost running out its battery (Battery Level < 5%), the ABS may request the AMS to enter DCR mode. By applying this use case, the AMS can return to the network within a very short time after renewing its battery.

- Proposed Text for IEEE P802.16m/D6

============================== Start of proposed text=================================

[Accept the following modifications marked in blue from line 49 on page 140.]

16.2.3.21 AAI_DREG-RSP message

Table 707—AAI_DREG-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Action code | 4 | Used to indicate the purpose of this message 0x00: AMS shall immediately terminate service with the ABS and should attempt network entry at another ABS 0x01: AMS shall listen to the current ABS but shall not transmit until a RES-CMD message or AAI_DREG-RSP message with action code 0x02 or 0x03 is received. 0x02: AMS shall listen to the current ABS but only transmit on the control connection. 0x03: AMS shall return to normal operation and may transmit on any of its active connections. 0x04: This option is valid in response to a AAI_DREG-REQ message with De-Registration Request Code=0x00. The AMS shall terminate current Normal Operation with the ABS. 0x05: AMS shall begin idle mode initiation: a) to signal AMS to begin idle mode in unsolicited manner or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x06: This option is valid only in response to a AAI_DREG-REQ message with De-Registration Code 0x01: a) to reject AMS-initiated idle mode request or b) to allow AMS to transmit AMS-initiated idle mode request at the REQ-Duration expiration 0x07: This option is valid in response to a AAI_DREG-REQ message with De-registration-request-code= 0x01. 0x08: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to allow retention of the AMS's connection information 0x09: This option is valid only in response to an AAI_DREG-REQ message with De-Registration Request Code 0x04 to reject retention of the AMS's connection information. 0x10: The AMS shall deregister from the current ABS, the context of the AMS will be kept in the network. 0x101-0x15: reserved | N.A. |

[Accept the following modifications marked in blue from line 55 on page 404.]

16.2.19 Deregistration with content retention (DCR) mode

16.2.19.1 DCR initiation in connected state

When ABS decides to reject AMS’s DCR mode request, the ABS shall send an AAI_DREQ-RSP with action code 0x09 in response to the AAI_DREG-REQ message. The ABS may also send an AAI_DREG-RSP with action 0x10 to initiate DCR Mode for an AMS.

=============================== End of proposed text==================================

References

- IEEE P802.16m/D6. DRAFT Amendment to IEEE Standard for Local and metropolitan area networks—Part 16: Air Interface for Broadband Wireless Access Systems—Advanced Air Interface, May 2010.

- IEEE 802.16m-08/0034r2. IEEE 802.16m System Description Document, Sep 2009.

- IEEE 802.16m-07/002r9. IEEE 802.16m System Requirements Document, Sep 2009.