| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Use of Identifiers in AAI_RNG-RSP message in response to AAI_RNG-REQ (16.2.3.2) |  |
| Date Submitted | 2010-7-14 |  |
| Source(s) | Feng Xie, Yang Liu, Nan Li, Lei Zhang, Lin Chen, Zhaohua Lu ZTE Corporation | E-mail: {xie.feng2, liu.yang8, li.nan25, zhang.lei13, chen.lin23, lu.zhaohua }@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.2” |  |
| Abstract | This contribution proposes the texts for clarification on AAI_RNG-REQ message (16.2.3.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Use of Identifiers in AAI_RNG-RSP message in response to AAI_RNG-REQ (16.2.3.2)

Feng Xie, Yang Liu, Nan Li, Lei Zhang, Lin Chen, Zhaohua Lu

ZTE Corporation

# 1. Introduction

AAI_RNG-RSP message needs clean-up.

# 2. Text Proposal

In the P802.16m/D6, page 79, modify Table 680 as follows:

======================== Start of Proposed Text =====================

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Ranging Abort | 1 | Set to 1 when an ABS rejects the AMS | Present when an ABS rejects an AMS |
| … | … | … | … | … |
| O | Emergency Service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | It shall be included when an ABS sends an AAI_RNG-RSP in response to an AAI_RNG-REQ with Ranging Purpose Indication set to code 0b0010 when an AMS performs initial ranging. |
| O | Location Update Response | 4 | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved 0x03=Success of location update and DL traffic pending 0x04 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x05 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x06~0xF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| O | New Paging Cycle | 4 | New Paging Cycle of the AMS 0b0000 - 0b1111 | It shall be included only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed |
| O | New Paging Group ID | 16 | New PGID of the AMS |  |
| O | New Paging Offset | 6 | New PAGING_OFFSET of AMS |  |
| O | New Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and Paging Controller ID has changed |
| O | Deregistration Identifier (DID) | 12 | The new DID which the AMS shall maintain in idle mode. |  |
| O | Paging carrier information | 6 | physical carrier index | It may be included only if the Location Update Response is set to 0x00(Success of Idle Mode Location Update) |
| O | HO Reentry Process Optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines. Identifies reentry process MAC control messages that may be omitted during the current HO attempt due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC control message responses as indicated in HO process optimization. | It shall be included when the AMS is attempting to perform network reentry, or HO, or zone switch and the target ABS wishes to identify reentry process MAC control messages that may be omitted during the current HO attempt |
| … | … | … | … | … |

============================== End of Proposed Text ===============