| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AAI_HO-CMD message and HO Procedures (16.2.3.11 & 16.2.6.3.5.2) |  |
| Date Submitted | 2010-7-9 |  |
| Source(s) | Feng Xie, Nan Li, Yuqin Chen, Yang Liu, Zhaohua Lu, Lin Chen, Jianjian Zhu ZTE Corporation | E-mail: {xie.feng2, li.nan25, chen.yuqin, liu.yang8, lu.zhaohua, chen.lin23 }@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.11 & 16.2.6.3.5.2” |  |
| Abstract | This contribution proposes the texts for clarification on AAI_HO-CMD message (16.2.3.11 & 16.2.3.5.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AAI_HO-CMD message and HO Procedures (16.2.3.11 & 16.2.6.3.5.2)

Feng Xie, Nan Li, Yuqin Chen, Yang Liu, Zhaohua Lu, Lin Chen, Jianjian Zhu

ZTE Corporation

# 1. Introduction

AAI_HO-CMD message（16.2.3.11）and operations during EBB mode and zone switch from MZone to LZone need clarification.

1) Some of the text in AAI_HO-CMD message does not comply with the text from line 1 to 12 in page 283 (16.2.6.3.3).

=============cited from line 1 to 12 in page 283==================

If HO_Reentry_Mode is set to 1, the serving ABS shall negotiate with the target ABS the EBB HO parameters. In the single carrier handover case, the EBB HO parameters include HO_Reentry_Interleaving_Interval, HO_Reentry_Interval and HO_Reentry_Iteration for the AMS to communicate with the serving ABS during network reentry, in which case HO_Reentry_Interleaving_Interval and HO_Reentry_Interval must be no less than the minimal values defined in AMS capability. The HO_Reentry_Interval defines the period during which an AMS performs network re-entry at the target ABS. Whereas the HO_Reentry_Interleaving_Interval defines the period during which an AMS performs normal data communication at the serving ABS after the HO_Reentry_Interval. In the multicarrier handover case, the EBB_HO parameters include the carrier information in the target ABS for the AMS performing network reentry while continuing communication with the serving ABS concurrently.

===============end of citation============================

According to the description, EBB HO parameters include three parameters named as HO_Reentry_Interleaving_Interval, HO_Reentry_Interval and HO_Reentry_Iteration. However, only two of them, namely HO_Reentry_Interleaving_Interval and HO_Reentry_Iteration are included in AAI_HO-CMD message. So HO_Reentry_Interval is missed from the message, which defines the period during which an AMS performs network re-entry at the target ABS.

2) FA index should be included in AAI_HO-CMD message when AMS handover to neighbor R1 BS or when zone switch from MZone and LZone if MZone and target LZone are located in different carriers. This may happen in two scenarios. a) mixed-mode ABS applies FDM mode for deploying LZone and MZone. b) mixed-mode ABS applies TDM mode for deploying LZone and MZone in one carrier and the ABS support multi-carrier. Then MZone and target  LZone may be located in different carriers.

# 2. Text Proposal

Remedy 1:

In the P802.16m/D6, page 108, modify Table 691 as follows:

======================== Start of Proposed Text =====================

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Mode | 2 | 0b00: HO command; 0b01: Zone switch command from MZone to LZone; 0b10: AMS HO request rejected (ABS in list unavailable). In this case, AAI_HO-CMD message shall not include any target ABS. | N.A. |
| O | HO Reentry Mode | 1 | 0: the AMS disconnects from the Serving ABS before performing network reentry with the Target ABS (Mode=0b00); or the AMS disconnects from the MZone before performing network reentry with the LZone (Mode=0b01). 1: the AMS maintains communication with Serving ABS while performing network reentry with the Target ABS (Mode=0b00); or the AMS maintains communication with MZone while performing network reentry with LZone in the same frame or on another carrier (Mode=0b01). | Shall be included when Mode = 0b00 or 0b10 Shall be included when Mode = 0b01. Shall not be set to 1 if AMS does not support EBB (according to AAI_REG-REQ). |
| O | HO Reentry Interleaving Interval | 8 | If HO Reentry Interleaving Interval > 0, the AMS performs network reentry to the target ABS within the HO Reentry Interleaving Interval and continues data transmission with the Serving ABS during HO Reentry Interval in the remaining time. If HO Reentry Interleaving Interval =0, the AMS performs multi-carrier EBB (Established Before Break) HO procedure per 16.2.8.2.9.2.2 | Shall be included when HO Reentry Mode is set to 1 |
| O | HO Reentry Interval | 8 | AMS performs network re-entry at the target ABS during HO Reentry Interval. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | HO Reentry Iteration | 3 | The requested number of iterating HO Reentry Intervals by an AMS. | Shall be included when HO Reentry Interleaving Interval > 0 |
| O | Disconnect time offset | 8 | Difference between Disconnect time and Action time in units of frames. The value of disconnect time shall be calculated for each T-ABS by adding/ subtracting this value with the value of Action time specified for this T-ABS. For HO_Reentry_Mode = 0, Disconnect time will be (Action time - Disconnect time offset); For HO_Reentry_Mode = 1, Disconnect time will be (Action time + Disconnect time offset). | Shall be included when Mode = 0b00. |
| O | Resource_Retain_Time | 8 | The time when the serving BS discards MS's context. | Shall be included when Mode = 0b00 or 0b01. |
| O | targetBSID | 48 | - | Shall be included for each target legacy BS or ABS when Mode=0b00 |
| O | Emergency Service FID | 4 | A FID number assigned within the Transport FID numbers 0010-1111 according to Table 652 | It shall be included when an ABS sends an AAI_RNG-RSP in response to an AAI_RNG-REQ with Ranging Purpose Indication set to code 0b0010 when an AMS performs initial ranging. |
| O | Location Update Response | 4 | 0x00= Success of Location Update 0x01= Failure of Location Update 0x02 = Reserved 0x03=Success of location update and DL traffic pending 0x04 = Allow AMS's DCR mode initiation request or DCR mode extension request 0x05 = Reject AMS's DCR mode initiation request or DCR mode extension request 0x06~0xF: Reserved | It shall be included when an ABS sends an AAI_RNG-RSP message in response to an AAI_ RNG-REQ message used to perform location update or DCR mode initiation from Idle Mode or DCR mode extension. |
| O | SA-Preamble Index | 10 |  | Shall be included for each target ABS when Mode=0b00 |
| O | FA index | 8 |  | Shall be included for each target ABS legacy BS when Mode=0b00 or for target LZONE located in a different carrier when Mode=0b01 |
| O | Action Time | 8 | Mode=0b00 Action Time included in this message is the absolute frame number at the serving ABS. When CDMA_RNG_FLAG is set to 1, it shall be set to the frame where either a normal or dynamic ranging channel is present. Mode=0b01 This value is defined as the frame number that AMS starts zone switch. Action Time included in this message is indicated by frame number | Shall be included for each target ABS when Mode=0b00. Shall be included when Mode = 0b01. |
| O | CDMA_RNG_FLAG | 1 | If CDMA_RNG_FLAG=0, CDMA based ranging shall be skipped. Otherwise, if set to 1, it indicates that ranging shall be done. This parameter shall be included for each target ABS when Mode=0b00.; It shall be included when Mode = 0b01. If CDMA_RNG_FLAG is set to 0, pre-assigned STID at the target BS shall be assigned. | Shall be included for each target ABS when Mode=0b00 |
| O | offsetData | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | Presents when CDMA_RNG_FLAG=0 |
| O | offsetControl | 7 | It represents the value among -31.5 to 32 dB with 0.5 dB step | Presents when CDMA_RNG_FLAG=0 |
| O | Dedicated Ranging Code Flag | 1 | If set to 0, it indicates that no dedicated ranging code is provided. If set to 1, it indicates that no dedicated ranging code is provided. | May be included for each target ABS when CDMA_RNG_FLAG=1 |
| O | Dedicated CDMA ranging code | 5 | Indicates the dedicated ranging code. | Shall be included when Dedicated Ranging Code Flag =1 |
| O | Ranging opportunity index | 3 | Indicates the index of the allocated ranging opportunity. | Shall be included when Action Time refers to a frame where a dynamic ranging channel is allocated |
| O | Subframe index | 2 | Indicates the subframe index of the allocated ranging opportunity | Shall be included when Action Time refers to a frame where a dynamic ranging channel is allocated |
| O | HO process optimization | 4 | For each bit location, a value of 0 indicates the associated reentry MAC control messages shall be required, a value of 1 indicates the reentry MAC control message may be omitted. Bit #0: Omit AAI_SBC-REQ/RSP MAC control messages during reentry processing Bit #1: Omit PKM Authentication phase Bit #2: Omit AAI_REG-REQ/RSP message and higher layer protocol triggering (for IP address refresh) during reentry processing. Bit #3: If Bit #3 = 1, Full service and operational state transfer or sharing between Serving BS and Target BS (All static and dynamic context, e.g., ARQ windows, state machines | Shall be included for each target ABS when Mode=0b00 When Bit #2 is set to 0 and IP address refresh is needed, the AAI_REG-REQ/RSP includes Host configuration capabilities and its parameters to provide IP address information. |
| O | Seamless HO | 1 | Indicates whether seamless HO is supported | Shall be included for each target ABS when Mode=0b00. In case of Seamless HO and CDMA_RNG_FLAG set to 1, Dedicated Ranging Code Flag shall be set to 1. |
| O | Ranging initiation deadline | 8 | An AMS shall send the AAI_RNG-REQ message during HO until Ranging initiation deadline | Shall be included for each target ABS when Mode=0b00 |
| O | Pre-assigned STID | 12 | STID assigned to the AMS by the target ABS | Should be included for each target ABS when Mode = 0b00 |
| O | Service level prediction | 2 | Indicates the level of service the AMS can expect from this ABS. The following encodings apply: 0 = No service possible for this AMS 1 = Some service is available for one or several service flows authorized for the AMS 2 = For each authorized service flow, a MAC connection can be established with QoS specified by the AuthorizedQoSParamSet. 3 = No service level prediction available. | Shall be included for each target ABS when Mode=0b00 |
| O | Physical_Carrier_Index | 6 | Physical carrier index of the recommended Target ABS | Shall be included for each target ABS when Mode = 0b00 and target ABS is not included in AAI_NBR-ADV message or is multi-carrier ABS |
| O | Channel bandwidth | 8 | Channel bandwidth of recommended target ABS in units of 125 KHz. | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | CP length | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each target ABS when Mode=0b00 and target ABS is not included in AAI_NBR-ADV message |
| O | Carrier Preassignment Indication | 1 | 0: no pre-assigned secondary carrier 1: information of pre-assigned secondary carriers by the target ABS is included in this message; | Shall be included for an AMS with multi-carrier capability when Mode = 0b00 |
| O | N_Preassigned_Carriers | 3 | Number of pre-assigned secondary carriers at the target ABS. A value of 0 means no secondary is pre-assigned. | Shall be included when Carrier Preassignment Indication = 1 |
| O | Pre-assigned secondary carrier information |  | Carrier information of each pre-assigned secondary carrier, which includes: Carrier Status Bitmap: indicating whether this pre-assigned carrier will be activated immediately after HO procedure is done; Physical carrier index of pre-assigned secondary carriers; Logical carrier index, which is assigned implicitly in the order of Carrier information of each pre-assigned secondary carriers. | Shall be included when N_Preassigned_Carriers > 0 |
| O | LZone Preambile Preamble Index | 7 | LZone Preamble Index for AMS to switch from MZone to LZone | Shall be included when Mode = 0b01. |
| O | Pre allocated Basic CID | 16 | Used by the AMS to derive its own primary CID and transport CID in target legacy BS or LZONE. | May be included when HO to switch from MZone to LZone or HO to a legacy BS |
| O | SFH delta information | Variable | Delta encoding with reference to the serving ABS. | May be included for each target ABS when Mode = 0b00 and SFH delta information is not included in the AAI_NBR-ADV message for the target ABS |
| O | S-SFH change count | 4 | S-SFH change count of the reference for the included SFH delta information.. | Shall be included for each target ABS when SFH delta information is included |
| O | Physical carrier index at the serving ABS | 6 | Recommended carrier index of the AMS to be used for network reentry to the target ABS | May be included when AMS's multicarrier capability= 0b10 , HO Reentry Mode = 1 and the target carrier is different from the serving carrier |

============================== End of Proposed Text ===============

Remedy 2:

In the P802.16m/D6, Section 16.2.6.3.5.2, page 286, modify the text from line 60 as follows:

======================== Start of Proposed Text =====================

If HO_Reentry_Mode is set to 1, the AMS performs network reentry with the target ABS at Action Time while continuously communicating with the serving ABS. However, the AMS stops communication with serving ABS after network reentry at target ABS is completed. In addition, AMS does not exchange data with target ABS prior to completion of network reentry. If HO_Reentry_Interleaving_Interval > 0, the AMS communicates with the target ABS during HO_Reenry_Interleaving_Interval, and with the serving ABS during HO_Reentry_Interval using the remaining communication opportunity. If HO_Reentry_Interleaving_Interval = 0, AMS performs Multi-carrier EBB HO reentry procedure per 16.2.8.2.9.2. ABS setting HO_Reentry_Interleaving_Interval = 0 is allowed only for AMS with multicarrier aggregation capabilities. Upon completion of network reentry, the target ABS informs the serving ABS to stop allocating resources to the AMS and release AMS context.

============================== End of Proposed Text ===============