| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Cleanup on AAI_NBR-ADV message (16.2.3.12) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Feng Xie, Yang Liu, Lin Chen, Zhaohua Lu, Nan Li ZTE Corporation | E-mail: {xie.feng2, liu.yang8}@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.12” |  |
| Abstract | This contribution proposes the texts for AAI_NBR-ADV (16.2.3.12) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Cleanup on AAI_NBR-ADV message (16.2.3.12)

Feng Xie, Yang Liu, Lin Chen, Zhaohua Lu, Nan Li

ZTE Corporation

# 1. Introduction

# The parameters in AAI_NBR-ADV message should be consistent with the message descriptions in 16.2.3.12.

1) “IDcell” in the message format should be changed to “SA-preamble index” for consistency and convention. The name “Preamble index” is used in the 16e/16m MAC procedures. And the name “IDcell” or Cell_ID is used in the 16e/16m PHY procedures.

2) Prefix of BSID should be included in the message format for consistency with the message descriptions.

# 2. Text Proposal

Accept the following Modifications to 16.2.3.12 AAI_NBR-ADV:

In the P802.16m/D5, page 111, line 1, replace Table 693 to the following table:

--------------------------------------------------  Text Start  -----------------------------------------------------------------

Table 693 — AAI_NBR-ADV parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Count | 3 | NBR-ADV Change Count | NA |
| M | Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101-0b111: reserved | NA |
| M | Total Number of AAI_NBR-ADV Segments | 4 | Total number of segments of AAI_NBR-ADV for this cell type | NA |
| M | AAI_NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type | NA |
| M | BS number M | 8 | Total number of BSs to be included in this AAI_NBR-ADV segment | NA |
| M | Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index. | NA |
| O | Prefix of BSID | variable | Prefix of BSID | For ABS type of shared configuration information |
| O | List of PHY carrier indices | 6x64 (maximum 64 phy indices) | Refer to the physical carrier index in AAI_Global-Config message | For ABS type of shared configuration information whose system info are not included in AAI_NBR-ADV |
| O | IDCell SA-preamble index range start | 10 | Lower bound of SA-preamble index range | For ABS type of shared configuration information whose system info are not included in AAI_NBR-ADV |
| O | IDCell SA-preamble index range end | 10 | Upper bound of SA-preamble index range | For ABS type of shared configuration information whose system info are not included in AAI_NBR-ADV |
| MO | LSB of BSID | 48variable | Remained BSID bits not included in Prefix of BSID | Shall be included for each neighbor BS |
| M | MAC protocol versions | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 10 defined for 16m. | Shall be included for each neighbor BS |
| M | CP time | 2 | CP time of the BS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each neighbor BS |
| M | Number of carriers | 3 | Number of carriers of the BS | Shall be included for each neighbor BS |
| M | SA-PREAMBLE index | 10 | - | Shall be included for each carrier of each neighbor BS |
| M | A-PREAMBLE transmit power | 8 | - | Shall be included for each carrier of each neighbor BS |
| M | Physical carrier index | 6 | Refer to the physical carrier index in AAI_Global-Config message | Shall be included for each carrier of each neighbor BS |

============================== End of Proposed Text ===============