| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Request of Neighbor Femto ABS Information with AAI_NBR-REQ Message (16.2.3.36 & 16.4.8.1.2) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Feng Xie, Lin Chen, Yuqin Chen, Yang Liu, Zhaohua Lu, ZTE Corporation Namgi Kim Kyonggi University | E-mail: {xie.feng2, liu.yang8}@zte.com.cn E-mail: ngkim@kyonggi.ac.kr |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.36 & 16.4.8.1.2” |  |
| Abstract | This contribution proposes the texts for AAI_NBR-REQ (16.2.3.36) and AMS scanning of neighbor Femto ABS (16.4.8.1.2) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Request of Neighbor Femto ABS Information with AAI_NBR-REQ Message (16.4.8.1.2 & 16.2.3.36)

Feng Xie, Lin Chen, Yuqin Chen, Yang Liu, Zhaohua Lu

ZTE Corporation

Namgi Kim

Kyonggi University

# 1. Introduction

# How AMS requests the neighbor information with AAI_NBR-REQ message needs clarification. In section 16.4.8.1.2, it reads

“If the AMS decides to perform HO to any Femto ABS, it may request more detailed system information of the detected neighbor Femto ABSs to the serving ABS using an AAI_NBR-REQ message.

AMS may include the CSGID(s) of the subscribed CSG femto BSs in the AAI_NBR-REQ message.”

1) If an AMS request more detailed system information of the detected neighbor Femto ABSs to the serving ABS using an AAI_NBR-REQ message. The AMS shall include the information of the detected Femto ABS such as SA-preamble index or LSB of BSID. Note that LSB of BSID in S-SFH SP1 IE is 12-bits length instead of 16-bits.

# 2. Text Proposal

Remedy 1:

Accept the following Modifications to 16.2.3.36 AAI_NBR-REQ:

In the P802.16m/D6, page 167, line 16,

Modify the text as follows:

--------------------------------------------------  Text Start  -------------------------------------------------------------

16.2.3.36 AAI_NBR-REQ

An AMS may send the AAI_NBR-REQ message to request the unicast AAI_NBR-ADV message. The request should not contain the BSIDs for which full information is already provided in the AAI_NBR-ADV message. The AAI_NBR-REQ message may contain the request type indication to request the neighbor BS information with the specified type.

The AAI_NBR_REQ message shall include following parameters:

•LSB of Requested BS ID: 16 bits

—16-bit LSB of unique 48-bit identifier of neighbor BS

- Request BS Type: 2bits

- 00: CSG Femto ABS

- 01: OSG Femto ABS

- 10-11: reserved

Table 722 — AAI_NBR-REQ parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | BSID | 1612 | 16-bit LSB of unique 48-bit identifier of neighbor BS 12-bit LSB of unique 48-bit identifier of neighbor BS | NA Present if AMS requests detailed neighbor information with LSB of BSID |
| M | Request BS type | 2 | 00: CSG Femto ABS 01: OSG Femto ABS 10-11: reserved | NA |

============================== End of Proposed Text ===============

Remedy 2:

Accept the following Modifications to 16.4.8.1.2 AMS scanning of neighbor Femto ABS:

In the P802.16m/D6, page 796, line 5,

Modify the text as follows:

--------------------------------------------------  Text Start  -------------------------------------------------------------

If the AMS decides to perform HO to any Femto ABS, it may request more detailed system information of the detected neighbor Femto ABSs to the serving ABS using an AAI_NBR-REQ message. For that purpose, AAI_NBR-REQ includes the detected 12 bit LSB of BSID of neighbor Femto ABS(s).

AMS may include the CSGID(s) of the subscribed CSG femto BSs in the AAI_NBR-REQ message.

============================== End of Proposed Text ===============