| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on Zone Switch TLV in RNG-RSP message (16.10.2.1) |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Feng Xie, Yang Liu, Yuqin Chen, Nan Li, Lei Zhang, Lin Chen, Zhaohua Lu ZTE Corporation | E-mail: {xie.feng2, liu.yang8}@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.10.2.1” |  |
| Abstract | This contribution proposes the texts for zone switch TLV (16.10.2.1) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on Zone Switch TLV in RNG-RSP message (16.10.2.1)

Feng Xie, Yang Liu, Yuqin Chen, Nan Li, Lei Zhang, Lin Chen, Zhaohua Lu

ZTE Corporation

# 1. Introduction

# Zone Switch TLV in RNG-RSP message needs clarification and clean-up.

1) NONCE_BS is no longer used and should be deleted from zone switch TLV.

2) FA index should be included in Zone Switch TLV if LZone and target MZone are located in different carriers. This may happen in two scenarios. a) mixed-mode ABS applies FDM mode for deploying LZone and MZone. b) mixed-mode ABS applies TDM mode for deploying LZone and MZone in one carrier and the ABS support multi-carrier. Then LZone and target MZone may be located in different carriers.

# 2. Text Proposal

Accept the following Modifications to 16.10.2.1 RNG-RSP management message encodings:

In the P802.16m/D6, page 854, line 6,

Modify Table 982 as follows:

--------------------------------------------------  Text Start  -------------------------------------------------------------

Table 982 —OFDMA-specific RNG-RSP message encodings

| Name | Type(1 byte) | Length | Value |
| --- | --- | --- | --- |
| MZone SA-Preamble index | 41 | 1 2 |  |
| Time offset | 42 | 1 | Time offset between LZone and MZone |
| Action Time | 43 | 1 | Action time of zone switch from LZone to MZone. AMS performs zone switch at Action Time. If HO_Reentry_Mode Zone Switch Mode=0, ABS stops all resource allocation for the AMS at LZone. |
| Zone Switch Mode | 44 | 1 | 0x01: AMS maintains its data communication with the ABS in LZone while performing network reentry in MZone; 0x00: AMS breaks data communication in LZone before performing network reentry in MZone. |
| NONCE_BS | 45 | 4 | For AMSID* derivation. PMK is shared between LZone and MZone during zone switch. |
| Temporary STID | 46 | 1 | Temporary STID for being used in MZone |
| Ranging initiation deadline | 47 | 1 | Valid time for Temporary STID. Shall be included if Temporary STID is included. |

============================== End of Proposed Text ===============