| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Paging listening interval mapping for Legacy ASN support (16.2.18.2.3) |  |
| Date Submitted | 2009-07-15 |  |
| Source(s) | Lin Chen, Feng Xie, Yang Liu, Lei Zhang, Zhaohua Lu, ZTE Corporation | E-mail: {chen.lin23, xie.feng2, liu.yang8, zhang.lei13, lu.zhaohua, }@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.18.2.3” |  |
| Abstract | This contribution proposes the texts for the paging listening interval mapping to support the legacy ASN. (16.2.18.2.3) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Paging listening interval mapping for legacy ASN support (16.2.18.2.3)

Lin Chen, Feng Xie, Yang Liu, Lei Zhang, Zhaohua Lu

ZTE Corporation

# Introduction

Due to the deployment requirement, the smooth evolution from 16e to 16m network SHOULD be supported. However, the first release of Wimax profile supporting 16m will be lack of network support. That is, the Rel1.x ASN-GW is used for not only the 16e BS/MS but also the 16m ABS/AMS.

Under this deployment scenario, The ABS/AMS has to bridge the gap between the legacy ASN-GW and the 16m AMS/ABS. For example, the paging listening interval relevant parameters are quite different between 16e and 16m as shown in Table 1. The paging cycle and paging offset in 16m and 16e are in terms of superframe and frame respectively. In 16m, the only accepted paging cycle is the 8, 16, 32, 64, 128, 256 and 512 superframe whereas the paging offset in 16e could be between 0~64Kframe. Also the length of paging offset in 16m and 16e are 6 bit and 16 bit respectively. Last but not the least, the frame that the AMS listens for paging message in 16m is determined by DID and time hash parameter m. In legacy ASN scenario, the DID could not be assigned, so the method for the derivation of Nframe has to be modified.

Table 1 Comparison of the paging parameter between 16e and 16m

| Item | 16e | 16m |
| --- | --- | --- |
| Paging cycle | 16 bit, maximal 64K frame | 4 bit, The paging cycle could be 8,16,32,64,128,256 and 512 superframes. |
| Paging offset | 16 bit, maximal 64K frame | 6 bit |
| Time duration of paging listening interval | 8 bit paging interval length carried in DREG-CMD message, the value shall be between 1~5 frames (default =2). | Default to be one superframe |
| Paging listening interval | The paging listening interval shall begin with the Nframe that meets the following condition: Nframe modulo PAGING_CYCLE == PAGING_OFFSET | The paging listening interval shall begin with the Nsuperframe that meets the following condition: Nsuperframe modulo PAGING_CYCLE == PAGING_OFFSET The frame whose paging message may contain the AMS is determined by: Nframe = AMS’s DID/m, m=1,2,3,4 The value of m is obtained through the PGID_Info message in the first frame of Nsuperframe. |

Based on the aforementioned analysis, the following solution for the paging listening interval relevant parameter mapping is proposed.

# 2. Proposed Solution

In legacy ASN, to support the paging parameter in legacy PC, the paging cycle and paging offset should be extended to 16 bit to support the legacy PC. That is, the management messages that contain the paging cycle and paging offset field should be modified correspondingly. These management messages include the DREG-REQ, DREG-RSP, PAG-ADV, RNG-REQ and RNG-RSP message.

Even though the paging cycle and paging offset are returned to the 16e format, the time duration of paging listening interval in legacy ASN is one superframe per paging cycle similar to the 16m specification. The start of the AMS’s paging listening interval could be determined by the following fomula:

Nsuperframe=floorof[(pagingcycle∗n+pagingoffset)4],

where n could be determined by the following fomula:

n=floorofNcurrent_superframe∗4−pagingoffsetpagingcycle+1modceilof212∗4−pagingoffsetpagingcycle;

Here theNsuperframe represents the superframe number of next paging listening interval; pagingcycle and pagingoffset represent the paging cycle and paging offset in terms of frame; Ncurrent_superframe represent the current superframe number that the AMS detects.

The specific frame within theNsuperframe that contain the PAG-ADV message for this AMS is determined by:

Npagingframe=MACaddresshashmodm, m=1, 2, 3, 4,

Where m is the time hash parameter indicated by the PGID_Info message.

Both the ABS and AMS should use the aforementioned formula to calculate the Nsuperframe and Npagingframe to decide the paging listening interval and paging frame for a specific AMS.

# 2. Proposed text with change marks

Add the following Modifications at the end of the introduction, above section 16.2.18.1 in page 392, line 37:

If the Network Configuration indicates one ABS is attached to a legacy network, the paging cycle and paging offset in terms of frame and a MAC Address Hash result of one AMS shall be transmitted to the AMS which is performing idle mode initiation. Both the ABS and the AMS shall use the paging cycle, paging offset in terms of frame, and the MAC Address Hash result to determine the paging frame and the paging listening interval.

16.2.18.1 Idle mode initiation

Accept the following Modifications for section 16.2.3.22 in page 143, line 23:

16.2.3.22 AAI_PAG-ADV (paging advertisement) Message

Table 708—AAI_PAG-ADV message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Paged MS Info[1..32] | Paging Cycle | 4/16 | Used to indicate Paging cycle for the AMS to be paged If the Network Configuration indicates ABS is attached to the advanced network: 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03: 64 superframes 0x04: 128 superframes 0x05: 256 superframes 0x06: 512 superframes 0x07-0x15: reserved | Shall be present unless Num_AMSs is 0. If the Network Configuration indicates ABS is attached to the legacy network, 16 bit paging cycle shall be used, otherwise, 4 bit paging cycle shall be used. |

Accept the following Modifications for section 16.2.3.21 in page 140, line 52:

16.2.3.21 AAI_DREG-RSP Message

Table 707—AAI_DREG-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging cycle | 4/16 | Used to indicate Paging cycle for the AMS If the Network Configuration indicates ABS is attached to the advanced network: 0x00: 8 superframes 0x01: 16 superframes 0x02: 32 superframes 0x03: 64 superframes 0x04: 128 superframes 0x05: 256 superframes 0x06: 512 superframes 0x07-0x15: reserved | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07. If the Network Configuration indicates ABS is attached to the legacy network, 16 bit paging cycle shall be used, otherwise, 4 bit paging cycle shall be used. |
| O | Paging offset | 6/16 | Used to indicate Paging offset for the AMS | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07. If the Network Configuration indicates ABS is attached to the legacy network, 16 bit paging offset shall be used, otherwise, 6 bit paging offset shall be used. |

Accept the following Modifications for section 16.2.3.20 in page 138, line 27:

16.2.3.20 AAI_DREG-REQ message

Table 706—AAI_DREG-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging cycle request | 4/16 | Used to indicate Paging cycle recommend by MS 0b0000-0x1111: reserved | Shall be present when Deregistration_Request_Code of AAI_DREG-REQ message is set to 0x01. If the Network Configuration indicates ABS is attached to the legacy network, 16 bit paging cycle shall be used, otherwise, 4 bit paging cycle shall be used. |

Accept the following Modifications for section 16.2.3.2 in page 81, line 30:

16.2.3.2 AAI_RNG-RSP message

Table 680—AAI_RNG-RSP message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging Cycle | 4/16 | New Paging Cycle for the AMS 0b0000-0x1111 | It shall be present only if the Location Update Response is set to 0x00 (Success of Idle Mode Location Update) and the Paging Information has changed. If the Network Configuration indicates ABS is attached to the legacy network, 16 bit paging cycle and paging offset shall be used, otherwise, 4 bit paging cycle and 6 bit paging offset shall be used. |
| O | Paging Offset | 6/16 | New PAGING_OFFSET of AMS |  |

Accept the following Modifications for section 16.2.3.1 in page 78, line 25:

16.2.3.1 AAI_RNG-REQ message

Table 679—AAI_RNG-REQ message

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Paging Cycle Change | 4/16 | PAGING_CYCLE requested by the AMS | It shall be included when the AMS in Idle Mode is attempting to change Paging Cycle during Location Update. If the Network Configuration indicates ABS is attached to the legacy network, 16 bit paging cycle change shall be used, otherwise, 4 bit paging cycle change shall be used. |