| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Idle mode message cleanup to support legacy ASN |  |
| Date Submitted | 2009-07-09 |  |
| Source(s) | Lin Chen, Feng Xie, Yang Liu, Lei Zhang, Zhaohua Lu ZTE Corporation | E-mail: {chen.lin23, xie.feng2, liu.yang8, zhang.lei13, lu.zhaohua}@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.2” |  |
| Abstract | This contribution proposes the texts for the idle mode management message clarification to support the legacy ASN. |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Idle mode message cleanup to support legacy ASN

Lin Chen, Feng Xie, Yang Liu, Lei Zhang, Zhaohua Lu

ZTE Corporation

# 1. Introduction

In this legacy ASN scenario, the idle mode is impacted and has to be modified. The DID is replaced by the MAC address hash which shall be used as the paging identifier. In D6, the usage of MAC address hash has been adopted. However, the related management messages do not give clear condition of when should the MAC address hash shall be used and when the DID shall be used. So in this contribution, we give the clarification of these idle mode related management messages.

# 3. Proposed text with change marks

Adopt the following modification in section 16.2.3.2 “AAI_RNG-RSP”, page 81, line 38:

Table 680—AAI_RNG-RSP message Field Descriptions

| O | Paging Controller ID | 48 | The new Paging Controller ID which the AMS shall maintain in idle mode. | It shall be included only if the Location Update Response = 0x00 (Success of Idle Mode Location Update) and Paging Controller ID has changed. The DID is included only when the Network Configuration indicates ABS is attached to the advanced network. |
| --- | --- | --- | --- | --- |
| O | Deregistration Identifier (DID) | 12 | The new DID which the AMS shall maintain in idle mode. |  |

Adopt the following modification in section 16.2.3.21 “AAI_DREG-RSP”, page 141, line 7:

Table 707—AAI_DREG-RSP message

| O | Paging offset | 6 | Used to indicate Paging offset for the AMS | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| --- | --- | --- | --- | --- |
| O | Paging controller ID | 48 | Used to indicate Paging controller which manages and retains the MS's idle mode information 0..2481– | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Paging group ID | 16 | Used to indicate Paging group which the MS is located in 0..216 – 1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 |
| O | Deregistration ID | 12 | Used to indicate Deregistration ID used to identify the MS in idle mode 0..212 – 1 | Shall be present when Action code in AAI_DREG-RSP message is set to 0x05 or 0x07 and when the Network Configuration indicates ABS is attached to the advanced network. |

Adopt the following modification in section 16.2.3.22 AAI_PAG-ADV (paging advertisement) Message, page 143, line 13:

Table 708 –AAI_PAG-ADV message

| MO | Paged MS Info[1..32] | Deregistration Identifier | 12 | Used to indicate Deregistration ID for the MS to be paged (Deregistration Identifier and Paging Cycle are used to identify each paged AMS) 0..212 – 1 | Shall be present if the Network Configuration indicates ABS is attached to the advanced network and unless Num_AMSs is 0 |
| --- | --- | --- | --- | --- | --- |
| MO | Paged MS Info[1..32] | MAC Address Hash | 24 | used to identify the MS to be paged. | Shall be present if the Network Configuration indicates ABS is attached to the legacy network and unless Num_AMSs is 0 |
| O | MAC Address Hash |  | 24 | See 6.3.2.3.51 | Present only if the Network Configuration indicates that ABS is attached to the legacy network. |