| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The cleanup of AAI_MC-REQ message (16.2.3.52) |  |
| Date Submitted | 2009-07-09 |  |
| Source(s) | Lin Chen, Feng Xie, Zhaohua Lu Yang Liu ZTE Corporation | E-mail: {chen.lin23, xie.feng2, lu.zhaohua, liu.yang8}@zte.com.cn |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.3.52” |  |
| Abstract | This contribution proposes the texts for the AAI_MC-REQ message format cleanup (16.2.3.52) |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The cleanup of AAI_MC-REQ message (16.2.3.52)

Lin Chen, Feng Xie, Zhaohua Lu, Yang Liu

ZTE Corporation

# Introduction

In D6, the contents in Table 755 and Table 756 are duplicated. It’s better to remove one of them. The format of Table 756 is aligned with most of the MAC control messages. However, the array attribute format is not presented. So in this contribution, we remove Table 755 and clarified the content of Table 756.

# 2. Proposed text with change marks

Adopt the following modification in section 16.2.3.52 “AAI_MC-REQ Message”, page 215, line 4:

16.2.3.52 AAI_MC-REQ (multicarrier Request) Message

Table 755—AAI_MC-REQ message format

| Field | Size (bits) | Notes |  |
| --- | --- | --- | --- |
| Global_Support | 1 | Indicates whether the AMS can process allcarriers supported in ABS simultaneously 0b0: The AMS can only process a subset ofthe carriers supported in ABS 0b1: The AMS can process all carriers supported in ABS |  |
| If (Global_Support==0){ |  |  |  |
| Number of Candidate Combinations (N) | 4 |  |  |
| For (i=0, i<N; i++) { |  |  |  |
| Number of Candidate Assigned Carrier (Nc) | 3 |  |  |
| For (j=0, j<Nc; j++) { |  |  |  |
| Physical Carrier Index | 6 | The carriers AMS can simultaneously support |  |
| } |  |  |  |
| DL/UL indicator | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. |  |
| } |  |  |  |
| } |  |  |  |
| Support of data transmission over guard sub-carrier | 1 | 0b0 = not supported 0b1=support |  |

Table 756 – Contents of AAI_MC-REQ message

| M/O | Attributes / Array of attributes |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- |
| M | Global Support |  |  | 1 | Indicates whether the AMS can process all carriers supported in ABS simultaneously 0b0: The AMS can only process a subset of the carriers supported in ABS 0b1: The AMS can process all carriers supported in ABS | M |
| O | Number of candidate combinations (N) |  |  | 4 | Indicates the number of carrier combination MS can support | Shall be present when Global Support is zero |
| O | Carrier combination info [0..N] | Number of candidate assigned carrier(Nc) |  | 3 | Indicates the number of carriers MS can support | Shall be present when the number of candidate combinations is not zero |
|  |  | Carrier index info[0..Nc] | Physical carrier index | 6 | Indicates the carriers MS can simultaneously support |  |
|  |  | DU/UL indicator |  | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. |  |
| O | Number of candidate assigned carrier(Nc) |  |  | 3 | Indicates the number of carriers MS can support | Shall be present when the number of candidate combinations is not zero |
| O | Physical carrier index |  |  | 6 | Indicates the carriers MS can simultaneously support | Shall be present when the number of candidate assigned carrier is not zero |
| O | DU/UL indicator |  |  | 1 | Indicates whether the AMS supports the combination with UL transmission. 0b0: The combination can be applied to both DL and UL 0b1: The combination can be applied to DL only. | Shall be present when the number of candidate combinations is not zero |
| M | Support of data transmission over guard sub-carrier |  |  | 1 | Indicates the support of guard carrier for data 0b0: not supported 0b1: supported | N.A. |