| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on the Dynamic Allocation of NS-RCH (16.3.6.5.2.4.13 & 16.3.9.2.4.1) |  |
| Date Submitted | 2010-07-13 |  |
| Source(s) | Xianming Chen, Feng Xie, Yanfeng Guan, Zhaohua Lu, Huiying Fang ZTE Corporation | E-mail: {xie.feng2, chen.xianming1, guan.yanfeng, lu.zhaohua, fang.huiying }@zte.com.cn |
| Re: | Call for SB_16m on “ P802.16m/D6”: Target topic: “16.3.6.5.2.4.13&16.3.9.2.4.1” |  |
| Abstract | This contribution proposes the texts about the Dynamic Allocation of NS-RCH (16.3.6.5.2.4.13 &16.3.9.2.4.1). |  |
| Purpose | To be discussed and adopted |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on the Dynamic Allocation of NS-RCH

Xianming Chen, Feng Xie, Yanfeng Guan, Zhaohua Lu, Huiying Fang

ZTE Corporation

# 1. Introduction

This contribution proposes the texts about the dynamic allocation of NS-RCHs. Dynamic NS-RCHs can be allocated by Broadcast Assignment A-MAP IE. But the allocation number for dynamic NS-RCHs in one frame and the purpose for using dynamic NS-RCHs are not clarified. Therefore I suggest the proposed text to clarify it.

- Text proposal in the P802.16m/D6

Proposed text has been underlined in blue and deleted text has been struck through in red. Existing draft text is shown in black.

------------------------------------------------   Text Start  -----------------------------------------------------------

[Remedy #1]

[Modify the text in line 39 of page 606 as below:]

16.3.6.5.2.4.13 Broadcast Assignment A-MAP IE

Broadcast assignment A-MAP allocates resource for a broadcast burst. A broadcast burst contains one or more broadcast MAC control messages. Table 865 describes the fields in a broadcast assignment A-MAP IE.

Table 865 – Broadcast Assignment A-MAP IE*

| Syntax | Size (bit) | Description/Notes |
| --- | --- | --- |
| Broadcast_Assignment_A-MAP _IE() { |  |  |
| A-MAP IE Type | 4 | Broadcast Assignment A-MAP IE |
| Function Index | 1 | 0: This IE carries broadcast assignment information 1: This IE carriers ranging channel dynamic allocation information of NS-RCH. |
| If (Function Index == 0) { |  |  |
| Burst Size | 6 | Burst size as indicated in the first 39 entries in Table 959 |
| Resource Index | 11 | 5MHz: 0 in first 2 MSB bits + 9 bits for resource index 10MHz: 11 bits for resource index 20MHz: 11 bits for resource index Resource index includes location and allocation size. |
| Long TTI indicator | 1 | Indicates number of AAI subframes spanned by the allocated resource. 0b0:1 AAI subframe (default) 0b1:4 UL AAI subframes for FDD or all UL subframes for TDD If number of DL AAI subframes, D, is less than number of UL AAI subframes, U, Long TTI Indicator = 0b1. |
| Transmission Format | 1 | 0b0: no time domain repetition 0b1: with time domain repetition |
| If (Transmission Format == 0b1){ |  |  |
| Repetition | 2 | 0b00: no more repetition of the same burst 0b01: the same burst will be transmitted one more time 0b10: the same burst will be transmitted two more time 0b11: the same burst will be transmitted three more time |
| Reserved | 14 | Reserved bits |
| }else{ |  |  |
| Reserved | 16 | Reserved bits |
| } |  |  |
| } else if (Function Index == 1) { |  |  |
| Number of Ranging Opportunities (N) | 21 | 0: one NS-RCH 1: two NS-RCHs |
| for (i=0; i<N; i++) { |  |  |
| Subframe Index | 3 |  |
| } |  |  |
| Reserved | TBD | Reserved bits |
| } |  |  |
| } |  |  |

[Remedy #2]

[Modify the text in line 44 of page 710 as below:]

The NS-RCH for handover ranging can also be allocated by Broadcast Assignment A-MAP based on ABS scheduling decision in any AAI subframe, except the AAI subframe that has already been used for a regular allocation.

-----------------------------------------------   Text End  -----------------------------------------------------------