| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | CRID update procedure (16.2.1) |  |
| Date Submitted | 2010-07-06 |  |
| Source(s) | Xiangying Yang Intel | Xiangying.yang@intel.com |
| Re: | Commets on P802.16m/D6 for SB |  |
| Abstract | The contribution provides the proposed text of SA-Preamble partitioning for ABS type to be included in IEEE 802.16m amendment document. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

CRID update procedure (16.2.1)

Xiangying Yang

Intel

- Introduction

CRID is associated with network entity. The network, depending on NWG specifications, may choose to update CRID during mobility and other possible cases (offloading for example). A unified air interface hook to enable CRID update is needed.

We propose the use AAI_REG-RSP as the unified air interface method for assigning/updating CRID. The benefits are

- Simplicity: a single message to handle all cases

- Security: it is desirable for the network to authenticate the AMS first, before setting up network context and associate the context with the ID. In addition, AAI_REG-RSP is ALWAYS encrypted and integrity protected. Therefore AAI_REG-RSP seems to be a better choice than AAI_RNG-RSP.

- Overhead: Note that network may choose to update CRID at will (e.g. a possible case is anchor relocation in the network, which however is agnostic to the AMS). This may not necessarily trigger re-authentication. Therefore, AAI_REG-RSP is a better choice than AAI_PKM-RSP.

- References

[1] IEEE 802.16m-07/0034r2, “802.16m System Requirements”

[2] IEEE 802.16m/D6, “Advanced Air Interface (Draft)”

[3] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m amendment

Modify P802.16m/D6 as following

-------------------------------------------------------  Text Start  --------------------------------------------------------------

[Note for Editor: Page 48, line 29]

16.2.1.2.4 Context Retention Identifier (CRID)

The network shall assign a 72bit CRID to each AMS during network entry or zone switch to Mzone in AAI_REG-RSP. The AMS is identified by the CRID in coverage loss recovery and DCR mode, where the CRID allows the network to retrieve AMS context. The network may assign the AMS a new CRID in unsolicited AAI_REG-RSP if necessary. When CRID field presents in AAI_REG-RSP, the polling bit shall be set to “1” in MCEH and the AMS shall acknowledge the reception of AAI_REG-RSP.

When the network wants to update the CRID for a idle mode AMS, the AMS shall perform network reentry to connected mode in order to receive the update.

[Note for Editor: Page 80, line 49]

16.2.3.2 AAI_RNG-RSP

Table 680—AAI_RNG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| o | CRID | 72 | AMS identifier which the AMS has been assigned for coverage loss or DCR mode and are currently maintained | It shall be included when the AMS is attempting to perform network reentry from coverage loss or DCR mode or to perform network reentry/location update/zone-switch with assigning a new CRID or it shall be included when the AMS detects the restart count in AAI_SCD message different from old one save in AMS. |
| … |  |  |  |  |

[Note for Editor: Page 80, line 49]

16.2.3.8 AAI_REG-RSP

Table 688—AAI_REG-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … |  |  |  |  |
| Mo | CRID | 72 | AMS identifier, of type BITSTRING, which the AMS has been assigned for coverage loss or DCR mode | When CRID is initially assigned to the MS during network entry/reentry or CRID is updated by the network |
| … |  |  |  |  |

[Note for Editor: Page 862, line 53]

-- Ranging Response

AAI-RNG-RSP ::= SEQUENCE {

[…]

stid	 		STID	 		OPTIONAL,

crid 			CRID 			OPTIONAL,

locationUpdateResp 	LocationUpdateResponse	OPTIONAL,

[…]

}

-------------------------------------------------------  Text End  ----------------------------------------------------------------