| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | The Clarification of the NS-RCH in the IEEE P802.16m/D6 (16.3.9.2.4.1) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | HyunWoo Lee and Jin Sam Kwak LG Electronics Chiwoo Lim and Hwasun Yoo Samsung Electronics Zheng Yan-Xiu, Chung-Lien Ho, Chang-Lan Tsai, and Yu-Chuan Fang ITRI Xinrong Wang and Yang-Seok Choi Intel Corporation Pei-Kai Liao and Paul Cheng MediaTek Inc. | Voice: +82-31-450-1916 E-mail: hyunwoolee.lee@lge.com, jinsam.kwak@lge.com chiwoo.lim@samsung.com zhengyanxiu@itri.org.tw xinrong.wang@intel.com, yang-seok.choi@intel.com pk.liao@mediatek.com |
| Re: | sb_16m : Sponsor Ballot Comments for IEEE 802.16m P802.16m/D6 : 16.3.9.2.4.1 |  |
| Abstract | This contribution proposes the modified text of ranging channel section to be included in the IEEE P802.16m Draft. |  |
| Purpose | To be discussed and adopted by TGm for the IEEE P802.16m Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

The Clarification of the NS-RCH in the IEEE P802.16m/D6 (16.3.9.2.4.1)

HyunWoo Lee and Jin Sam Kwak

LG Electronics

Chiwoo Lim and Hwasun Yoo

Samsung Electronics

Zheng Yan-Xiu, Chung-Lien Ho, Chang-Lan Tsai, and Yu-Chuan Fang

ITRI

Xinrong Wang and Yang-Seok Choi

Intel Corporation

Pei-Kai Liao and Paul Cheng

MediaTek Inc.

# 1. Introduction

This contribution proposes the modified text of ranging channel section for further clarification in the IEEE P802.16m/D6 [1]. The Koffset of equation (284) should be modified to align it in the center of ranging bandwidth.

# Reference

[1] IEEE P802.16m/D6, “Part 16: Air Interface for Broadband Wireless Access Systems : Advanced Air Interface,” May 2010.

- Text proposal for inclusion in the 802.16m amendment

----------------------------------------------------------------------------------------------------------------------------------------------------------------

Black text: text in the subclause 16.3.9.2.4.1 of P802.16m/D6

Red Strike through Text: Deleted

Blue text: new text

[Bracketed Italic text]: Informative

------------------------------------------------------------------------- Text Start --------------------------------------------------------------------------

-----------------	Remedy  : 	16.3.9.2.4.1 Ranging channel for non-synchronized AMSs	------------------

---------------			Page 710/Line 49 ~ Page 711/Line 31				------------------

[Modify the following sentences]

# Ranging signal transmission

Equation (284) specifies the transmitted signal voltage to the antenna, as a function of time, during the NS-RCH format 0 or 1.

(284)

where

t		is the elapsed time since the beginning of the subject NS-RCH, with 0<t<(TRCP+2×TRP) for Format 0 or 0<t<(TRCP+TRP) for Format 1.

NRP		is the length of the RP code in frequency domain.

xp(n)		is the p-th RP code with length NRP.

Koffset		is the parameter related to the frequency position and is defined by

NPRU	is the total number of PRUs as defined 16.3.8.2.1.

k0	is the lowest PRU index of the assigned NS-RCH.

Psc	is the number of the consecutive subcarriers within a PRU in frequency domain as defined in 16.3.8.1.

is the ranging subcarrier spacing.

------------------------------------------------------------------------ Text End ---------------------------------------------------------------------------