| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on unsolicited Bandwidth indication during network entry (16.2.15) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak LG Electronics, Inc. | +82-31-450-1879 jin1.lee@lge.com kiseon.ryu@lge.com ronnyyoungho.kim@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract | This contribution is to clarify unsolicited bandwidth indication during network entry |  |
| Purpose | Update P802.16m/D6 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Unsolicited bandwidth indication during network entry

Jin Lee, Kiseon Ryu, Ronny Yongho Kim, JinSam Kwak

LG Electronics, Inc.

- Introduction

- According to D6, ABS is able to allocate UL bandwidth to transmit and retransmit subsequent MAC messages during network entry by setting unsolicited bandwidth indicator in AAI_RNG-RSP. And if the unsolicited bandwidth indicator is set, AMS should monitor A-MAP IE during the predefined time duration to see if there is resource allocated. However, the predefined time duration is not clear. Therefore, this contribution defines BR grant timer to represent the predefined time period in D6.

- Proposed Texts

---------------------------------------------------------------Text Start ----------------------------------------------------------

Remedy 1 :

[Modify texts as suggested below: Page 377, Line 54]

During network entry, an ABS may allocate an UL bandwidth for transmission or retransmission of MAC messages, the size of BR without STID header, without a contention-based bandwidth request from the AMS by setting the Unsolicited bandwidth grant indicator in an AAI_RNG-RSP message to the AMS. If the unsolicited bandwidth indicator is enabled, the ABS should allocate UL bandwidth within the predefined BR grant time duration in order to transmit or retransmit subsequent MAC messages during network entry.

An AMS should monitor the A-MAP IE during the predefined BR grant time duration for possible bandwidth allocation without performing any bandwidth request. If the AMS fails to identify allocated bandwidth within the predefined BR grant time duration, the AMS may perform contention based bandwidth request. When the allocated resource is not sufficient for transmission of MAC messages, the AMS shall send a piggyback bandwidth request.

The BR grant timer in ABS is started upon ABS receiving MAC messages from AMS during network entry. The AMS also starts the BR grant timer when it receives the MAC messages from ABS successfully.

Remedy 2 :

[Add the following rows at the end of the table 554:]

| System | Name | Time Reference | Minimum Value | Default Value | Maximum Value |
| --- | --- | --- | --- | --- | --- |
| ABS, AMS | BR grant timer | UL allocation waiting time for BR header during network entry | - | - | - |

Remedy 3 :

[Modify Table 680 as indicated below, P83 L17:]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Unsolicited bandwidth indicator | 1 | 1: an ABS should allocate UL bandwidth for transmission of BR header or retransmission of MAC messages without request from AMS during network entry | It shall be included when AMS is attempting network entry |

---------------------------------------------------------------Text End -----------------------------------------------------------

- Reference

[1] IEEE P802.16m/D6, “Part 16: Air Interface for Broadband Wireless Access Systems: Advanced Air Interface,”.