| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Change for UL 2Tx Codebook (16.3.10.3.1) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Wookbong Lee LG Electronics | wookbong.lee@lge.com |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Change for UL 2Tx Codebook (16.3.10.3.1)

Wookbong Lee

LG Electronics

- Introduction

Currently IEEE 802.16m uplink 2Tx codebook contains antenna selection codewords (index 1000 and 1001) [1]. This antenna selection codewords may have some problem. So, we propose to change these antenna selection codewords to antenna turn off codewords.

- Problem of antenna selection codewords

The following is one example of uplink data transmission procedure in case of codebook based closed loop MIMO.

  - BS finds PMI for transmission.

  - BS calculates suitable MCS level according to power status report (PSR) and MS’s sounding signal.

  - MS calculates its power level based on equation (291) in D6 [1].

  - MS compares its calculated power level and its maximum power level.

  - If the calculated power level is larger than maximum power, then AMS reduce its power based on concurrent transmission rule (see section 16.3.9.4.6).

Problem comes when the selected PMI is one of the antenna selection codewords, Index 1000 and 1001 in uplink 2Tx codebook. (As mentioned in [2], power imbalance between two different antennas comes from e.g. user hand-gripping. In this case, transmitting using one antenna will be better. In other words, BS will select one of the antenna selection codewords.)

Since it requires transmitting different power level for different antennas, MS needs to do step 4 for each antennas. This requires more complex algorithm than comparing sum power for all antennas.

And if the maximum power is larger than it has for each transmitting antenna, then MS will reduce its power for that antenna. Since quantized power status is reported by periodic or event-driven manner, there will be a gap between reported and using. This will increase BLER when there is power shortage.

In most cases, if MS’s maximum power is 23dBm, then it wants to employ 20dBm power amp. for each transmit antennas to reduce cost of MS. However, to eliminate above issue, MS needs to employ 23dBm power amp. for each antennas even though it reports its maximum power as 23dBm.

- Proposed Solution

To resolve above issue, we propose to use 1/sqrt(2) instead of 1 for antenna selection codewords. In other words, MS doesn’t need to do step 4 for each antenna but can do as a normal codeword, which has constant modulus property, as well as it can safely use low cost 20dBm without performance degradation.

MS can turn off one of its antenna when it does not help to improve performance much. This will reduce MS power consumption.

- References

[1] Draft Amendment, P802.16m/D6

[2] “IEEE 802.16m Amendment Text Proposal for Sounding Antenna Selection,” C80216m-09/1449r1

Remedy :page 756,  Modify table 955 as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

Table 955 – Cbase,UL(2,1,4)

| Binary Index | m | Cbase,UL(2,1,4,m)=[c1;c2] |  |
| --- | --- | --- | --- |
|  |  | c1 | c2 |
| … | … | … | … |
| 1000 | 8 | 1 0.7071 | 0 |
| 1001 | 9 | 0 | 1 0.7071 |
| 1010-1111 | 10-15 | - | - |

-------------------------------------------------Text Change END -----------------------------------------------------