| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Resource Mapping of Assignment A-MAP (16.3.6) |  |
| Date Submitted | 2010-07-06 |  |
| Source(s) | Lei Huang Panasonic Singapore Laboratories Isamu Yoshii Panasonic Corporation | Email: lei.huang@sg.panasonic.com |
| Re: | IEEE 802.16m Sponsor Ballot: P802.16m/D6 |  |
| Abstract | The contribution clarifies the resource mapping of assignment A-MAP |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Resource Mapping of Assignment A-MAP

Lei Huang

Panasonic Singapore Laboratories

Isamu Yoshii

Panasonic Corporation

- Introduction

According to [1], in primary frequency partition, an A-MAP region consists of HARQ Feedback A-MAP, Power Control A-MAP, Non-user specific A-MAP and Assignment A-MAP. In non-primary frequency partition, an A-MAP region consists of Assignment A-MAP only.

In [1], the resource mapping of HARQ Feedback A-MAP, Power Control A-MAP, and Non-user specific A-MAP in an A-MAP region has been described clearly. However, there is no any description about the resource mapping of Assignment A-MAP in an A-MAP region. In this contribution, we clarify the resource mapping of Assignment A-MAP in an A-MAP region.

- Reference

- [1]	IEEE P802.16m/D6, DRAFT Amendment to IEEE Standard for local and metropolitan area networks - Part 16: Air Interface for Broadband Wireless Access Systems - Advanced Air Interface, June 2010

- Proposed Modifications

- Editorial notes

- Current text in P802.16m/D6

Remove text from P802.16m/D6

- Add new text to P802.16m/D6

- -------------------------------------------------------------------------------------------------------------------

- To insert the following paragraphs after line 7 in Page 538

- In the primary frequency partition, for each Tx antenna, symbols at the output of MIMO encoder, denoted by SA[0] to SA[LA-1], are mapped to tone-pairs from RMP[(LHF + LPC + LNUS)/2] to RMP[(LHF + LPC + LNUS+ LA)/2 -1], where RMP refers to the renumbered A-MAP tone-pairs described in 16.3.5.3.4 and LHF is the number of tones required to transmit the entire HARQ feedback A-MAP; LPC is the number of tones required to transmit the entire power control A-MAP; LNUS is the number of tones required to transmit the non-user specific-A-MAP; and LA is the number of tones required to transmit the assignment A-MAP.

- In the non-primary frequency partition, for each Tx antenna, symbols at the output of MIMO encoder, denoted by SA[0] to SA[LA-1], are mapped to tone-pairs from RMP[0] to RMP[LA/2 -1], where RMP refers to the renumbered A-MAP tone-pairs described in 16.3.5.3.4 and LA is the number of tones required to transmit the assignment A-MAP.

- -------------------------------------------------------------------------------------------------------------------