| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | AAI_SCN-RSP message clean up for IEEE802.16m (sections 16.2.3.14) |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak LG Electronics, Inc. | +82-31-450-7188 inuk.jung@lge.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract | This contribution is to propose text changes for 16.2.3.14. |  |
| Purpose | Update P802.16m/D6 by discussion and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

AAI_SCN-RSP message clean up for IEEE802.16m (sections 16.2.3.14)

Inuk Jung, Kiseon Ryu, Ronny Yongho Kim, Jin Sam Kwak

LG Electronics, Inc.

- Introduction

This contribution includes proposed text to amend some errors in the AAI_SCN-RSP message to support correct carrier based scanning.

- Rationale/Motivation of modifications

In general, the structure of this message is flaw and needs correct re-structuring. This proposal provides such suggested restructuring. In addition the physical carrier index is always required to be included in the AAI_SCN-RSP message, whether the target ABS is present in AAI_NBR-ADV or not. This is to specifically indicate which carrier to scan of the target ABS. Hence, the related AAI_SCN-RSP message shall be reformatted accordingly.

- References

[1] IEEE P802.16m/D6, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”

- Proposed AWD Text Changes

Remedy

On the 80216m_D6, page 124, line 1, add proposed text in table 695 of section 16.2.3.14 as follows:

--------------------------------------------------------Text Start -----------------------------------------------------

16.2.3.14 AAI_SCN-RSP

Table 695—AAI_SCN-RSP parameters

| M/O | Attributes / Array of attributes |  |  |  |  |  |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| O | Scan Iteration |  |  |  |  |  |  | 6 | The requested number of iterating scanning interval by an AMS. | Present only when Scan Duration > 0 |
| O | N_Recommended_ABS_Index |  |  |  |  |  |  | 6 | This is the number of neighbor ABS [0 .. 63] to be scanned, which are included in AAI_NBR-ADV message. When an AMS receives AAI_SCN-RSP message from ABS in response to AAI_SCN-REQ message, the AMS shall check whether Configuration Change Count stored by the AMS is the same as one included in AAI_SCN-RSP message sent by the ABS. If an AMS detects mismatch of Configuration Change Counts, it may retransmit AAI_SCN-REQ message to the ABS. | Present if ABS uses index to identify an ABSs to be scanned |
| O | Configuration Change Count for AAI_NBR-ADV |  |  |  |  |  |  | 8 | The value of Configuration Change Count in AAI_NBR-ADV message used for neighbor ABS index references | Present if N_Recommended_ABS_Index > 0 |
| O | nbrBsIndexArr [0..63] |  |  | Neighbor ABS index |  |  |  | Variable ( 8 x N) | ABS index corresponds to the position of ABS to be scanned in AAI_NBR-ADV message | Present if N_Recommended_ABS_Index > 0 |
| O |  |  |  | N_Recommended_Carrier_Index |  |  |  | Variable ( 6 x N) | Number of carriers to be scanned at each neighbor ABS [0..63]. | Present when N_Recommended_ABS_Index > 0 |
| O |  |  |  | carrierIndexArr[0..63] |  | Recommended_Carrier_Index |  | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI_Global-Config message | Present if N_Recommended_Carrier_Index > 0 |
| O | N_Recommended_ABS_Full |  |  |  |  |  |  | 6 | Number of neighbor ABS [0..63] to be scanned, which are using full 48-bit BSID. | Present only when Scan Duration > 0 and AAI_SCN-RSP is sent in response to AAI_SCN-REQ with N_Recommended_ABS_Full_indication>0 |
| O | fullBsIdArr [0..63] | Recommended ABS ID |  |  |  |  |  | Variable ( 48 x N) | ABS ID of the ABSs to be scanned | Present if N_Recommended_ABS _Full > 0 |
| O |  | N_Recommended_Carrier_Index |  |  |  |  |  | Variable ( 6 x N) | Number of carriers to be scanned at each neighbor ABS [0..63]. | Present when N_Recommended_ABS_Full > 0 |
| O |  | carrierIndexArr[0..63] |  |  | Recommended_Carrier_Index |  |  | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighbor ABS. As defined in AAI_Global-Config message | Present if N_Recommended_Carrier_Index > 0 |
| O |  |  |  |  | SA Preamble Index |  |  | Variable ( 10 x N) | SA Preamble Index to be scanned | Present if N_Recommended_Carrier_Index > 0 |
| O |  |  |  |  | CP length |  |  | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this recommended carrier index |
| O | N_Recommended_SA_Preamble_Index |  |  |  |  |  |  | 4 | Number of SA preambles to be scanned [0..15] | Present if ABS requests AMS to scan SA preambles |
| O | preambleIndexArr[0..15] |  | SA Preamble Index |  |  |  |  | Variable ( 10 x N) | SA Preamble Index to be scanned | Present if N_Recommended_SA_Preamble_Index > 0 |
| O | preambleIndexArr[0..15] |  | Physical carrier index |  |  |  |  | 6 | As defined in AAI_Global-Config message | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this SA preamble index |
| O | preambleIndexArr[0..15] |  | Channel bandwidth of recommended ABS ID |  |  |  |  | 8 | Channel bandwidth of recommended ABS ID in units of 125 KHz. Field length is 1 byte. | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this SA preamble index |
| O | preambleIndexArr[0..15] |  | CP length |  |  |  |  | 2 | CP length of the recommended target ABS 00: 1/8 01: 1/16 10: 1/4 | Shall be included when neighbor ABS is not included in AAI_NBR-ADV message and SCN-RSP is not sent as response to SCN-REQ which included this SA preamble index |
| O | preambleIndexArr[0..15] |  | N_Recommended_Carrier_Index |  |  |  |  | Variable ( 6 x N) | Number of carriers to be scanned at each neighbor ABS [0..63]. | Present when N_Recommended_ABS_Index > 0 or N_Recommended_ABS_Full > 0 |
| O | nbrAbsArr[0..63] |  | carrierIndexArr[0..63] |  |  |  | Recommended_Carrier_Index | Variable ( 6 x N x M) | Recommended physical carrier index for scan at each neighbor ABS. | Present if N_Recommended_Carrier_Index > 0 |
|  | … |  | … |  |  |  |  | … | … | … |

--------------------------------------------------------Text End -----------------------------------------------------