| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed changes to the Update of SFH information during ARS operation mode (D6 - 16.6.2.16.2) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Jin Sam Kwak LG Electronics | Voice : +82-31-450-1876 E-mail: heejeong.cho@lge.com |
| Re: | Sponsor Ballot on IEEE p802.16m D6 |  |
| Abstract | This contribution proposes the Update of SFH information during ARS operation mode for clarification. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed changes to the Update of SFH information during ARS operation mode (D6 - 16.6.2.16.2)

Heejeong Cho, Kiseon Ryu, Youngsoo Yuk, Jin Sam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the update of SFH information during ARS operation mode in the IEEE 802.16m DRAFT Amendment [1]. Because AMS should be aware of when essential system information sent via an RS_ESI message is applied, Super-Frame Number Action was included in the RS_ESI message in the IEEE P802.16m/D5. However, the field has gone missing in the IEEE P802.16m/D6.

- References

[1] IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

[Remedy #1 : Adopt the following modification in page 825, line 62, section 16.6.2.16.2]

16.6.2.16.2 Update of SFH information during ARS operation mode

Table 976—RS_ESI message format

| Name | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH information bitmap | 3 | If Bit #0 (LSB) is 0b1, S-SFH SP 1 is included. If Bit #1 is 0b1, S-SFH SP 2 is included. If Bit #2 (MSB) is 0b1, S-SFH SP 3 is included. |
| Super-Frame Number Action | 8 | LSBs of the super-frame number when the S-SFH information is applied |