| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Modification for DL PHY structure for E-MBS support (16.3.5) |  |
| Date Submitted | 2010-07-06 |  |
| Source(s) | Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting ITRI | E-mail: ythsieh@itri.org.tw |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m |  |
| Abstract | This contribution proposes renew text for E-MBS support. |  |
| Purpose | Discussion and adoption by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Modification for DL PHY structure for E-MBS support (16.3.5)

Chia-Lung Tsai, Yu-Tao Hsieh, Pang-An Ting

ITRI

Introduction

Two errors and some typos in Section 16.3.5.6 of [1] have been identified. In this contribution, we provide text modifications to correct these errors and typos.

Proposed text

Black text: text in the subclause 16.3.5.6 of P802.16m/D6

Red Text: delete text

Blue text: new text

Modify the text of section 16.3.5.6, on page 496, line 34 as follows,:

---------------------------------------------------------Start of the Text--------------------------------------------------------

The mapping between SLRUE-MBS index and the PRU index shall be done as follows:

If FP0 is used, only the subands from FP0 shall be assigned for E-MBS and the SLRUE-MBS index is deter-

mined by

In case that FP0 is not available ( e.g., reuse-3 only ), the number of PRUs for E-MBS in each frequency par-

tition, FPi (i>0) is derived as

Where q = KSB,E-MBS-FPCT * floor(KSB,E-MBS/FPCT), 1<=m<FPCT.

For each frequency partition i ( i>0 ), the total number of PRUs for E-MBS up-to and including that partition may be calculated as

Then, the SLRUE-MBS index is given by

where the mapping from the SLRUFPi to the PRU indices is as specified in 16.3.5.3.5. The subbands index

for E-MBS is determined by

---------------------------------------------------------End of the Text---------------------------------------------------------

References

[1] IEEE P802.16m/D6, “Air Interface for Broadband Wireless Access Systems - Advanced Air Interface”, May 2010.