| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up AAI_SBC-REQ/RSP attributes |  |
| Date Submitted | 2010-07-xx |  |
| Source(s) | Shaocheng Wang Joey Chou Intel Youngkyo Baek Samsung Electronics | E-mail: shaocheng.wang@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_SBC-REQ/RSP issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clean up AAI_SBC-REQ/RSP attributes

- Joey Chou, Shaocheng Wang

- Intel

- Introduction

This contribution is intended to clean up AAI_SBC-REQ/RSP attributes formatting ONLY.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.4 AAI_SBC-REQ

Table 682—AAI_SBC-REQ message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CAPABILITY_INDEX | 85 | It refers to the "Capability Class" that the AMS can support. Value: 0~31 |  |
| M | DEVICE_CLASS | 5 | It refers to the "Device Class" that the AMS can support. Value: 0~31 |  |
| O | Long TTI for UL | 1 | If Bit#0 =1,it supports |  |
| O | Long TTI for DL | 1 | If Bit#0 =1,it supports |  |
| O | UL sounding | 1 | If Bit#0 =1,it supports |  |
| O | DL resource metric for FFR | 1 | If Bit#0 =1,it supports |  |
| O | Max. Number of streams for SU-MIMO in DL MIMO | 3 | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |  |
| O | Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1 | 1/2(1bits; The number that is higher by 1 than this field) |  |
| O | DL MIMO mode | 3 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |  |
| O | feedback support for DL | 9 | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1 supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports |  |
| O | Number of Tx antenna of MS | 2 | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |  |
| O | Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | 1/2/3/4(2bits; The number that is higher by 1 than this field) |  |
| O | Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 2 | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |  |
| O | UL MIMO mode | 5 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |  |
| O | Modulation scheme | 2 | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#20=1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#51=1, UL 64 QAM supports |  |
| O | UL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |  |
| O | DL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |  |
| O | AMS DL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |  |
| O | AMS UL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMScan process. |  |
| O | FFT size(2048/1024/512) | 3 | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |  |
| O | Authorization policy support | 1 | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |  |
| O | Inter-RAT Operation Mode | 2 | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved |  |
| O | Supported Inter-RAT type | 8 | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |  |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |  |

Table 683—AMS Capabilities to be transmitted to ABS

| Nego parameters | Value |
| --- | --- |
| Long TTI for UL | If Bit#0 =1,it supports |
| Long TTI for DL | If Bit#0 =1,it supports |
| UL sounding | If Bit#0 =1,it supports |
| DL resource metric for FFR | If Bit#0 =1,it supports |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |
| feedback support for DL | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1 supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |
| Inter-RAT Operation Mode | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved |
| Supported Inter-RAT type | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |
| MIH Capability Supported | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.3.5 AAI_SBC-RSP

Table 684—AAI_SBC-RSP message Field Descriptions

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | CAPABILITY_INDEX | 85 | It refers to the "Capability Class" that the AMS can support. Value: 0~31 |  |
| M | DEVICE_CLASS | 5 | It refers to the "Device Class" that the AMS can support. Value: 0~31 |  |
| O | Long TTI for UL | 1 | If Bit#0 =1,it supports |  |
| O | Long TTI for DL | 1 | If Bit#0 =1,it supports |  |
| O | UL sounding | 1 | If Bit#0 =1,it supports |  |
| O | DL FFR | 1 | If Bit#0 =1,it supports |  |
| O | Max. Number of streams for SU-MIMO in DL MIMO | 3 | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |  |
| O | Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1 | 1/2(1bits; The number that is higher by 1 than this field) |  |
| O | DL MIMO mode | 3 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |  |
| O | feedback support for DL | 9 | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1 supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports |  |
| O | Number of Tx antenna of MS | 2 | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |  |
| O | Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 2 | 1/2/3/4(2bits; The number that is higher by 1 than this field) |  |
| O | Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 2 | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |  |
| O | UL MIMO mode | 5 | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |  |
| O | Modulation scheme | 2 | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#20=1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#51=1, UL 64 QAM supports |  |
| O | UL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |  |
| O | DL HARQ buffering capability | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |  |
| O | AMS DL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |  |
| O | AMS UL processing capability per sub-frame | 7 | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |  |
| O | FFT size(2048/1024/512) | 3 | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |  |
| O | Authorization policy support | 1 | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |  |
| O | Inter-RAT Operation Mode | 2 | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved |  |
| O | Supported Inter-RAT type | 8 | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |  |
| O | MIH Capability Supported | 1 | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |  |

Table 685—AMS Capabilities to be confirmed by ABS

| Nego parameters | Value |
| --- | --- |
| Long TTI for UL | If Bit#0 =1,it supports |
| Long TTI for DL | If Bit#0 =1,it supports |
| UL sounding | If Bit#0 =1,it supports |
| DL resource metric for FFR | If Bit#0 =1,it supports |
| Max. Number of streams for SU-MIMO in DL MIMO | 1/2/3/4/5/6/7/8(3bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in DL MIMO | 1/2(1bits; The number that is higher by 1 than this field) |
| DL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports If Bit#5=1, mode 5 supports |
| feedback support for DL | If Bit#0 =1, differential mode supports If Bit#1 =1, MIMO feedback mode 0 supports If Bit#2 =1, MIMO feedback mode 1 supports If Bit#3=1, MIMO feedback mode 2 supports If Bit#4 =1, MIMO feedback mode 3 supports If Bit#5 =1, MIMO feedback mode 4 supports If Bit#6 =1, MIMO feedback mode 5 supports If Bit#7 =1, MIMO feedback mode 6 supports If Bit#8 =1, MIMO feedback mode 7 supports |
| Number of Tx antenna of MS | 1/2/4(2bits; The number that is higher by 1 than this field and 3 is unavailable) |
| Max. Number of streams for SU-MIMO in UL MIMO(1/2/3/4) | 1/2/3/4(2bits; The number that is higher by 1 than this field) |
| Max. Number of streams for MU-MIMO in MS point of view in UL MIMO(1/2/3) | 1/2/3(2bits; The number that is higher by 1 than this field and 4 is unavailable) |
| UL MIMO mode | If Bit#0 =1, mode0 supports If Bit#1 =1, mode1 supports If Bit#2 =1, mode 2 supports If Bit#3 =1, mode 3 supports If Bit#4 =1, mode 4 supports |
| Modulation scheme | If Bit#0 =1, DL QPSK supports If Bit#1 =1, DL 16QAM supports If Bit#2 =1, DL 64 QAM supports If Bit#3 =1, UL QPSK supports If Bit#4 =1, UL 16QAM supports If Bit#5=1, UL 64 QAM supports |
| UL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field is the amount of information bits in 4800 bytes units the AMS can buffer in the UL. |
| DL HARQ buffering capability | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL HARQ information bits per frame in units of 4800 bytes, at which the aimed combining gain or better is obtained in the benchmark scenario, as defined in paragraph 16.2.14.2.1.3. |
| AMS DL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated DL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| AMS UL processing capability per sub-frame | Bit 0-6: The number that is higher by 1 than this field, is the steady amount of aggregated UL data information bits per sub-frame in units of 600 bytes that the AMS can process. |
| FFT size(2048/1024/512) | If Bit#0 =1, FFT 2048 supports If Bit#1 =1, FFT 1024 supports If Bit#2 =1, FFT 512 supports |
| Authorization policy support | If Bit #0=0, No authorization; If Bit #0=1, EAP-based authorization is supported |
| Inter-RAT Operation Mode | 0b00: single radio mode operation for inter RAT handover 0b01: multi radio mode operation for inter RAT handover 0b10 - 0b11 : reserved |
| Supported Inter-RAT type | 1 indicates support, 0 indicates not support: bit #0: 802.11 bit #1: GERAN(GSM/GPRS/EGPRS) bit #2: UTRAN bit #3: E-UTRAN bit #4: CDMA 2000 bit #5-7: Reserved, set to zero |
| MIH Capability Supported | If bit #0=1, the capability of IEEE 802.21 Media Independent Handover Services supports. |

------------------------------------------------- End of proposed text II--------------------------------------------------