| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for LBS support in Idle mode |  |
| Date Submitted | 2010-07-08 |  |
| Source(s) | Jia Lin, Bin Chen, Junxian Mo, Ronald Mao, Phillip Barber Huawei Technologies | E-mail: linjia@huawei.com binchen@huawei.com |
| Re: | Call for SB on “ P802.16m/D6”: Target topic: “16.2.18.2.1” |  |
| Abstract | This contribution proposes amendment text describing trigger function in multi-carrier scenario in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for LBS support in idle mode

Jia Lin, Bin Chen, Junxian Mo, Ronald Mao, Phillip Barber

- Huawei Technologies

- Introduction

- It is described in IEEE8021.16m SDD document about LBS support in idle mode. Following are contents:

- The system should be able to locate the mobile when in idle state. The ABS may use paging or other network initiated multicast signaling to initiate a location process on the AMS.

- The AMS in idle mode can receive a paging message which may include identifier of the AMS and indicator for LBS measurement method. AMS should perform network re-entry and LBS measurement with attached ABS and neighbor ABSs. When AMS gets LBS measurement parameters, AMS may report them as location information to attached ABS.

- LBS measurement and report may be performed in idle mode. In this case, we propose that action for LBS measurement should be added into paging message in 16m.

- References

- IEEE 802.16m-09/0034r2  “IEEE 802.16m System Description Document (SDD)”.

- IEEE P802.16m/D6, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D6

========================== Start of Proposed Text ==========================

[Note to Editor: Add the following contents in section 16.2.18.2.1 and Table 708]

16.2.18.2.1 Broadcast paging message

A Paging message includes identification of the AMSs to be notified of pending DL traffic, LBS measurement and location update.

The Paging message also includes an action code directing each AMS notified via the inclusion of its identifier

as appropriate:

•0b00: Perform network reentry

•0b01: Perform ranging for location update

•0b10: Perform LBS measurement

•0b11: Reserve

16.2.3.22 AAI_PAG-ADV (paging advertisement) Message

Table 708—AAI_PAG-ADV message

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| M | Paged MS Info[1..32] | Action Code | 12 | Used to indicate the purpose of the AAI_PAG-ADV message 0b00: Perform network reentry 0b01: Perform ranging for location update 0b10: Perform LBS measurement 0b11: Reserve | Shall be present unless Num_AMSs is 0 |

========================== End of Proposed Text ===========================