| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | SFH mismatch in HO procedure |  |
| Date Submitted | 2010-7-8 |  |
| Source(s) | Guanghui Fan, Bin Chen, Jia lin Huawei Technologies | E-mail: fanguanghui@huawei.com chenbin@huawei.com linjia@huawei.com |
| Re: | Call for SB on “ P802.16m/D6”: |  |
| Abstract | This contribution proposes about SFH mismatch in HO procedure in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

SFH mismatch in HO procedure in IEEE 802.16m

Guanghui Fan，Bin Chen, Jia Lin

Huawei Technologies

Introduction

1) Some remedies are proposed to the paragraph in D6 Page 282 Line 55.

2) If the ABS finds that an AMS cannot perform HO due to mismatched system information, we propose that the ABS may send another AAI_HO-CMD which includes up-to-date SFH information for each T-ABS.

3) The service flow which is rejected in the target ABS shall be indicated in the AAI_HO-CMD message. Such fields for this purpose are proposed to be added in the message.

Text proposal for inclusion in the P802.16m/D6

========================== Start of Proposed Text ==============================

[Editor’s Note 1 : modify the text as following in page 282 line 55]

For ABS initiated HO, the AMS may detect an SFH mismatch by referring to the S-SFH change count included in the AAI_HO-CMD message. The AMS should not select a T-ABS with mismatched SFH information. If the AMS does not have the latest SFH for any of the T-ABSs included in AAI_HO-CMD, it should cancel the HO by sending AAI_HO-IND with HO event code 0b011, the ABS should cancel the HO. When the serving ABS receives the AAI_HO-IND with HO event code 0b011, it may send another AAI_HO-CMD message which includes all up-to-date delta SFH information of each recommended T-ABS. If pre-allocated at target ABS, the serving ABS shall include an STID to be used at target ABS in the AAI_HO-CMD message. The pre-allocated STID shall be used in the target ABS by the AMS to communicate with the target ABS. The FIDs which are used to distinguish different connections are not updated during the handover procedure. If the network decides that certain service flow will not exist at the target ABS, this shall also be indicated in the AAI_HO-CMD message.

[Editor’s Note 2 : Insert the following field description in “16.2.3.11 AAI_HO-CMD” in page 108]

Table 691—AAI_HO-CMD parameters

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | Rejected FID indicator | 1 | If set to 0: No FID is rejected at the target ABS If set to 1: one or more FID(s) are rejected at the target ABS | Shall be included for each target ABS when Mode=0b00 |
| O | Rejected FID bitmap | 16 | Any bit in the bitmap set to 1 indicates the related FID is rejected at the target ABS. The n-th Bit refers to the FID n. | Shall be included when the rejected FID indicator is set to 1 |

========================== End of Proposed Text ==============================