| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Change of Service Specific Scheduling Control Header |  |
| Date Submitted | 2010-7-8 |  |
| Source(s) | Guanghui Fan, Bin Chen, Jia Lin Huawei Technologies | E-mail: fanguanghui@huawei.com binchen@huawei.com linjia@huawei.com |
| Re: | Call for SB on “ P802.16m/D6”: |  |
| Abstract | This contribution proposes about Changes of Service Specific Scheduling Control Header in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change of Service Specific Scheduling Control Header in IEEE 802.16m

Guanghui Fan, Bin Chen, Jia Lin

Huawei Technologies

Introduction

Remedy 1. Both the GPI and Grant size parameters of aGP service may be changed by Service Specific Scheduling Control Header, but the message field doesn't include the field used to change the Grant size.

Remedy 2. Per the sentence “ABS may adjust the start time of adaption of new QoS parameters.”  in page 336 line 34, the Service Specific Scheduling Control Header should include such a field for this purpose.

Remedy 3. It seems that the Service Specific Scheduling Control Header is not used for bandwidth request, but only for changing or acknowledging the scheduling parameters.

Text proposal for inclusion in the P802.16m/D6

========================== Start of Proposed Text ==============================

[Editor’s Note 1 : change the following fields of Service Specific Scheduling Control Header in Page 53 ]

16.2.2.1.3.3 Service Specific Scheduling Control Header

Service Specific Scheduling Control Header is sent either by AMS through dedicated UL resource or by ABS, which needs to change or acknowledge the scheduling parameters of its service flow. If the Service Specific Scheduling Control Header is sent from AMS, BR type shall be included to indicate whether BR type is increment or aggregate, and it indicates Adaptation Request when it is sent for aGP service flow. The ABS may acknowledge or modify the QoS parameters for aGP service flow by sending the Service Specific Scheduling Control Header in downlink, which indicates Adaptation Response. Service Specific Scheduling Control header format is defined in Table 659. If SCID change indicator is set to 1, the ABS shall send an acknowledgement to confirm the change of sleep mode configuration. If the ABS accepts the change of sleep mode configuration, the ABS shall send a MAEH as acknowledgement. The ABS may modify the AMS's sleep cycle setting switching request. In this case, the ABS shall reply with SCH including a different SCID to the AMS.

Table 659—Service Specific Scheduling Control Header Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BR type | 1 | Indicates whether the requested bandwidth is incremental or aggregate. 0: aggregate 1: incremental |
| BR Size | 19 | Bandwidth request size in bytes. |
| BR FID | 4 | The FID for which UL bandwidth is requested. The related FID for which this message is intended |
| if (scheduling type == aGP Service) { |  |  |
| QoS parameter change indicator | 1 2 | QoS parameter change indicator 0: no change in QoS parameter 1: having changes in QoS parameter 00: no change in QoS parameter 01: GPI parameter changed 10: Grant Size parameter changed 11: Both GPI and Grant Size parameters changed |
| SCID change indicator | 1 |  |
| Adaption Start time offset indicator | 1 | Adaption Start time offset indicator 0: Adaption Start time offset value included 1: Adaption Start time offset value not included |
| If (Adaption Start time indicator = 1) { |  |  |
| Adaption Start time offset | 6 | The relative frame offset value indicates the start time of adaption of the new QoS parameters |
| } |  |  |
| if (QoS parameter change indicator bit 0 = 1){ |  |  |
| Running Grant Polling Interval (GPI) | 6 | Indicating new Grant Size (frames) to use for future allocation |
| } |  |  |
| if (QoS parameter change indicator bit 1= 1 ){ |  |  |
| Running Grant Size | 6 | Indicating new Grant Size (frames) to use for future allocation |
| } |  |  |
| }else{ if (QoS parameter change indicator = 00){ |  |  |
| QoS parameter set switch |  | 0: primary QoS parameter set. 1: secondary QoS parameter set |
| } |  |  |

========================== End of Proposed Text ==============================