| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clean up AAI_NBR-ADV attributes |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Joey Chou Xiangying Yang Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to fix AAI_NBR-ADV issues |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Clean up AAI_NBR-ADV attributes

- Joey Chou, Xiangying Yang

- Intel

- Introduction

The AAI_NBR-ADV message as defined in Table 692 does not reflect the text in section 16.2.3.12.

To allow AAI_NBR-ADV fragmentation while providing flexibility for AMS HO operation without requiring acquisition of the whole AAI_NBR-ADV message, ABS always provides the total number of deployment types and total number of recommended T-ABS for each type. Each AAI_NBR-ADV fragment has corresponding indexes for each deployment type and each neighbor ABS. ABSs with identical type are listed in the AAI_NBR-ADV message in descending order of their cell coverage.

For example, the text extracted from section 16.2.3.12, indicates that ABS always provides the total number of deployment types and total number of recommended T-ABS for each type. But, the table does not contain the total number of deployment types and total number of recommended T-ABS for each type.

Moreover, the attributes to be included in neighbor ABS array or carrier per neighbor BS array are indicated in the conditions (e.g. Shall be included for each neighbor, BS Shall be included for each carrier of each neighbor BS) that don’t not consistent to the table format.

This contribution is intended to fix the issues described above.

- Proposed text

II.1 Proposed text 1

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.3.12 AAI_NBR-ADV

ABS determines and indicates the system configuration information included for each deployment type and their corresponding broadcast frequency.

To allow AAI_NBR-ADV fragmentation while providing flexibility for AMS HO operation without requiring acquisition of the whole AAI_NBR-ADV message, ABS always provides the total number of deployment cell types, and total number of segments recommended T-ABS for each type, and the total number of neighbor ABS per segment. Each AAI_NBR-ADV fragment is indicated by AAI_NBR-ADV Segment Index has corresponding indexes for each deployment type and each neighbor ABS. ABSs with identical type are listed in the AAI_NBR-ADV message in descending order of their cell coverage.

Each AAI_NBR-ADV message carries

•AAI_NBR-ADV change count

•number of total cell types

•segment information for this AAI_NBR-ADV message

•system information of ABSs from one or more cell types, which is of variable size.

•Starting ABS Index: Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment.

For each cell type, shared configuration parameters are carried as follows

•Prefix of BSID

•SA-Preamble Index range

•Physical carrier range

The Prefix of BSID, SA-Preamble Index range and Physical carrier range are optional fields, which can be used when the serving ABS chooses not to broadcast configuration information for each individual ABS within the cell type.

Within each cell type, if serving ABS chooses to broadcast configuration information for each individual ABS instead of specifying SA-Preamble Index range and Physical carrier range, a list of ABSs are provided and the following parameters are carried for each ABS

•48bit BS-ID

•ABS SA-Preamble Index

•indication whether full system information or partial information is carried for this ABS, which includes

—SFH information

—ESI information

—Physical carrier index (6 bits, refer to the "physical carrier index" defined in AAI_Global-Config)

—Number of MAC protocol versions supported (4 bits)

—list of supported MAC protocol versions (4 bits per entry)

—Paging carrier indication (1bit, refer to specify if a carrier is a paging carrier or not)

where for ABS of macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type); and for Wireless-MAN-OFDMA reference system, only 48-bit BS-ID and Preamble index are included in AAI_NBR-ADV.

•SFH_encoding_format: Based on the present system information, the list of ABSs shall be categorized into three groups. The categorization is indicated by SFH_encoding_format parameter

—0b00: full Subpacket information

—0b01: delta encoding with reference to the information of current carrier which transmits this AAI_NBR-ADV message

—0b10: no SFH included

—0b11: delta encoding with reference to the information of the preceding carrier

Dynamic SFH information such as MSB of superframe number shall not be transmitted in the AAI_NBR-ADV message.

The parameters NBR-ADV offset and NBR-ADV interval indicate the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS. The parameters shall be included in the AAI_NBR-ADV message when ABS indicates the existence of neighbor WirelessMAN-OFDMA BS or LZone of ABS.

NBR-ADV offset: The MOB_NBR-ADV to be scheduled after the frame n+NBR-ADV offset when the AAI_NBR-ADV message is sent in the frame n. Unit: Frame.

NBR-ADV interval: The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame.

Table 692— AAI_NBR-ADV

| M/O | Attributes/Array of attributes | Size (Bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Change Count | 3 | NBR-ADV Change Count | N.A. |
| M | Total number of cell type | 3 | The total number of cell type | N.A. |
| M | Cell type | 3 | Cell type in this message 0b000: macro 0b001: micro 0b010: macro hotzone 0b011: femto 0b100: relay 0b101-0b111: reserved | N.A. |
| M | Total Number of AAI_NBR-ADV Segments | 4 | Total number of segments of AAI_NBR-ADV for this cell type | N.A. |
| M | AAI_NBR-ADV Segment Index | 4 | Indicates current segment index of this message in the specific cell type | N.A. |
| M | BS number M | 8 | Total number of BSs to be included in this AAI_NBR-ADV segment | N.A. |
| M | Starting ABS Index | 8 | Starting ABS Index is the index offset from the last ABS of the previous AAI_NBR-ADV segment. If this is the first AAI_NBR-ADV segment, the Starting ABS Index will be 0. Hence, each AAI_NBR-ADV segment has one Index which corresponds to the first ABS in that AAI_NBR-ADV segment. The order of the ABS within a segment serves as an offset from the 'Starting ABS index'. The sum of the offset and the 'Starting ABS index' serves as the ABS index. | N.A. |
| O | NBR ABS per segment array [1..255] | Variable | The list of neighbor ABS in a segment | N.A. |
| O | List of PHY carrier indices | 6x64 (maximum 64 phy indices) | Refer to the physical carrier index in AAI_Global-Config message | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range start | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range end | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| M | A) BSID | 48 | - | Shall be included for each neighbor BS |
| M | B) MAC protocol versions | 8 | MAC protocol version of the BS Consistent with REV.2 definition, with new MAC protocol version 10 defined for 16m. | Shall be included for each neighbor BS |
| M | C) CP time | 2 | CP time of the BS 00: 1/8 01: 1/16 10: 1/4 | Shall be included for each neighbor BS |
| M | Number of carriers | 3 | Number of carriers of the BS | Shall be included for each neighbor BS |
| O | D) Carrier info per ABS array [1..7] | Variable | The list of carrier information per ABS |  |
| M | D.1) SA-PREAMBLE index | 10 | - | Shall be included for each carrier of each neighbor BS |
| M | D.2) A-PREAMBLE transmit power | 8 | - | Shall be included for each carrier of each neighbor BS |
| M | D.3) Physical carrier index | 6 | Refer to the physical carrier index in AAI_Global-Config message | Shall be included for each carrier of each neighbor BS |
| M | D.4) S-SFH change count | 4 | Indicates the value of S-SFH change count of this neighbor ABS | Shall be included for each carrier of each neighbor BS |
| M | E) SFH_encoding_format | 2 | 0b00: full Subpkt information 0b01: delta encoding with reference to the information of current carrier which transmits this AAI_NBR-ADV message. 0b10: no SFH included 0b11: delta encoding with reference to the information of the preceding carrier For macrocell ABS, the SFH_encoding_format shall be either 0b00 or 0b01 or 0b11. | Shall be included for each neighbor BS |
| O | F) Control_bitmap | 4 | Each bit maps to one SFH subpacket Bit[0]: SFH SP1 Bit[1]: SFH SP2 Bit[2]: SFH SP3 Bit[3]: reserved | Shall be included for each neighbor ABS present when SFH_encoding_format is 0b00. |
| O | G) SFH Subpkt 1 | Variable depending on FFT size | Full information of superframe header subpkt 1 | Shall be included for each neighbor ABS present if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #0 is set to 1 |
| O | H) SFH Subpkt 2 | Variable depending on FFT size | Full information of superframe header subpkt 2 | Shall be included for each neighbor ABS present if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #1 is set to 1 |
| O | I) SFH Subpkt 3 | variab | Full information of superframe header subpkt 3 | Shall be included for each neighbor ABS present if SFH_encoding_format is set to 0b00, and Control_bitmap Bit #2 is set to 1 |
| O | J) Delta information | Var. | Delta encoding, w.r.t. the reference carrier the current carrier transmitting this message if SFH_encoding format = 01, the preceding carrier if SFH_encoding_format=11) | Shall be included for each neighbor ABS present if SFH_encoding_format is set to 0b01 or 0b11 |
| O | List of PHY carrier indices | 6x64 (maximum 64 phy indices) | Refer to the physical carrier index in AAI_Global-Config message | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range start | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | IDCell range end | 10 |  | For ABS type whose system info are not included in AAI_NBR-ADV |
| O | Neighbor-specific trigger | Var. | Trigger definition (See Table 774) that applies only to the designated target ABS | (may not be needed if we have already got cell type specific trigger in AAI_SCD-ADV) May be included for each neighbor BS |
| O | NBR-ADV offset | 4 | The frame number offset the MOB_NBR-ADV is sent from the start point of frame counting. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV. |
| O | NBR-ADV interval: | 4 | The interval of MOB_NBR-ADV is sent in LZone. Unit: Frame. | It shall be included when the existence of neighbor R1 BS or LZone of ABS are indicated in AAI_NBR-ADV. |

------------------------------------------------- End of proposed text I --------------------------------------------------