| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control message with CMAC protection |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Joey Chou Xiangying Yang Muthaiah Venkatachal Elad Levy Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to add CMAC support to MAC control message |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- MAC control message with CMAC protection

- Joey Chou,

- Xiangying Yang,

- Muthaiah Venkatachal

- Intel

- Introduction

This contribution proposes a method to include CMAC tuple in a MAC control message that is encoded in ASN.1 format.

- Proposed text

------------------------------------------------- Start of proposed text I--------------------------------------------------

II.1 Proposed text 1

16.2.2.2.3 MAC Control Extended Header (MCEH)

The MCEH is used on the control connection. When message fragments belonging to two different control messages are being sent, the transmitter shall assign different Control Connection Channel ID (CCC ID)s to the MCEH of each MAC PDU. If a MAC control message is to be protected by CMAC tuple, CMAC Indicator (CMACI) bit shall be set to “1” to incicate the presence of CMAC tuple. The MCEH format is defined in Table 670.

Table 670: MCEH Format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| MCEH () { |  |  |
| Type | 4 | Extended header type = 0b0010 (MCEH Type) |
| EC | 1 | Encryption Control indicator 0 = Payload is not encrypted 1 = Payload is encrypted |
| Control Connection Channel ID (CCC ID) | 1 | Channel ID to identify separate fragmentation / reassembly state machines 0: channel 1 1: channel 2 |
| SN Indicator | 1 | 0 = no FC and sequence number 1= FC and sequence number are followed |
| CMAC Indicator | 1 | 0 = no CMAC tuple present 1 = CMAC tuple present in the MAC PDU or MPDU fragment |
| If (SN Indicator = 10) { |  |  |
| Reserved | 1 | For byte alignment |
| else { |  |  |
| Polling | 1 | 0 = no acknowledgement required 1 = acknowledge required upon receiving the MAC message |
| FC | 2 | Fragmentation control (see Table 669) |
| SN | 6 | Payload sequence number, SN is maintained per CCC ID, the ‘SN’ value increments by one (modulo 64) sequentially. |
| Reserved | 7 | For byte alignment |
| } |  |  |
| } |  |  |

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

[Editor Note: Add a new paragraph as show in the following:]

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.4.8 MAC Control Messages with CMAC Protection

Some MAC control messages contain CMAC tuple to enable the receiver to check the authenticity and the integrity of the messages. If a CMAC tuple is included in a MAC control message, it shall be the last attribute in the message, and its location shall be byte aligned. Since MAC control message will be encoded using ASN.1 PER method with byte unaligned option, there is no guarantee that CMAC tuple can be byte aligned if CMAC tuple is defined in the ASN.1 syntax.

Figure 396 shows an unfragmented MAC control message with CMAC tuple. ASN.1 PER will encode the MAC control message in a binary format that includes padding bits to insure byte alignment. The ASN.1 message, including ASN.1 binary data and padding bits (if any) are sent to the AES-CMAC algorithm to generate the 128 bit CMAC value that are truncated to 64 bits, since, only the 1st 64 bits of the CMAC value are included in a MAC message.

A MAC control message consists of AGMH, MCEH, ASN.1 message body, and CMAC tuple that consists of PMK_SN, CMAC_PN, and CMAC value. A 4 bit reserved is added, since CMAC_PN should be byte aligned. CMACI (CMAC Indicator) in MCEH shall be set tp “1” to indicate a CMAC tuple is appended to the end of the MAC PDU, after the ASN.1 message. The length field in AGMH has to be adjusted accordingly, when CMAC tuple is included in a MAC control message.

Figure 396—Unfragmented MAC Control Message with CMAC tuple

Figure 397 illustrates a fragmented MAC control message with CMAC tuple. It shows that CMAC tuple shall be sent in the last MAC control message fragment.

Figure 397—Fragmented MAC Control Message with CMAC tuple

------------------------------------------------- Start of proposed text II --------------------------------------------------