| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC control message with CMAC protection |  |
| Date Submitted | 2010-08-17 |  |
| Source(s) | Joey Chou Xiangying Yang Muthaiah Venkatachal Elad Levy Intel | E-mail: joey.chou@intel.com |
| Re: | TGm AWD: |  |
| Abstract | This contribution proposes text to add CMAC support to MAC control message |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- MAC control message with CMAC protection

- Joey Chou,

- Xiangying Yang,

- Muthaiah Venkatachal

- Elad Levy

- Intel

- Introduction

In 802.16-2009, CMAC tuple is defined in TLV format in the MAC management message. The receiver needs to parse the message to find out if a message is CMAC protected by checking the CMAC TLV. Since the CMAC tuple is always the last attribute in the message, the receiver can easily locate the part of message required for CMAC verification by excluding the CMAC TLV.

In 802.16m, the MAC control messages are defined in ASN.1 format using PER encoding with byte unaligned option. If the CMAC tuple is included in the ASN.1 message, the receiver will not be able to locate the part of message to be authenticated by AES-CMAC algorithm. If the CMAC tuple is defined outside the ASN.1 message, such as in extended headers, it has the following issues:

- CMAC may require additional attributes (e.g. AK-Count) that can add overheads and complexity to extended headers.

- Since CMAC is only required in a few messages (i.e. AAI-RNG-REQ, AAI-PKM-REQ, AAI-PKM-RSP), exposing CMAC to the MPDU structure will cause uncessary complication. Like 802.16-2009, it should be kept inside the MAC control message payload.

This contribution proposes a method similar to 802.16-2009 that parses the MAC control message prior to CMAC verification to resolve the above issues, and simplify the design.

- Proposed text

II.1 Proposed text 1

[Editor Note: at Page 263, Line 35, add a new paragraph as show in the following:]

------------------------------------------------- Start of proposed text I--------------------------------------------------

16.2.4.8 MAC Control Messages with CMAC Protection

CMAC tuple is included in some MAC control messages to enable the receiver to check the authenticity and the integrity of the messages. Figure X1 describes the construction of a MPDU with CMAC protection. For MAC control messages that need to be protected by CMAC tuple, a CMACI (CMAC Indicator) flag is defined in the message. When CMACI is set to “1”, it indicates the CMAC tuple is present. Otherwise, it is not present.

If a MAC control message has CMACI set to “0” or does not have CMACI, its ASN.1 binary data and padding bit (if needed for byte alignment), shall be the MPDU payload that has the size M = N bytes. If a MAC control message has CMAC set to “1”, its ASN.1 binary data and padding bits are sent to the AES-CMAC algorithm to generate the 128 bit CMAC value. A 12 byte CMAC buffer is used to store the CMAC tuple that consists of the following fields:

- PMK_SN (4 bits) – The sequence number of PMK used to derive the current AK.

- Reserved (4 bits) – Added for byte alignment.

- CMAC_PN (24 bits) – Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink.

- CMAC value (64 bits) – The most significant 64 bits of the 128 bits CMAC value.

The MPDU payload shall contain the ASN.1 binary data, padding bits (if any), and the CMAC buffer, and have the size M = N + 12 bytes.

Figure X1—MPDU contruction with CMAC protection

Figure X2 illustrates the construction of a MPDU with fragmented MAC control message that includes CMAC tuple. It shows that CMAC tuple shall be sent in the last MAC control message fragment.

Figure X2—MPDU construction of Fragmented MAC Control Message with CMAC Protection

Figure X3 describes the procedure of receiving a MAC control message that is PER coded, and has the size of M bytes. It first decodes the message to generate the plain ASN.1 message that contains the ASN.1 attributes needed for any kinds of CMAC processing. If the message type is AAI-RNG-REQ, AAI-PKM-REQ, or AAI-PKM-RSP, and CMAPI equals “1”, then it will send the first M – 12 bytes of the PER encoded message to CMAC verification. If the verification result is passes, it sends the plain ASN.1 message to applications. Otherwise, the message is discarded.

Figure X3—Procedure of receiving a MAC Control message

------------------------------------------------- End of proposed text I --------------------------------------------------

II.2 Proposed text 2

[Editor Note: at Page 87, Line 32 change as the following:]

------------------------------------------------- Start of proposed text II--------------------------------------------------

16.2.3.1 AAI-RNG-REQ

Table 679: AAI-RNG-REQ

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| O | Message type SN CCC ID | PMK_SN | 4 | The sequence number of PMK used to derive the current AK. | It shall be included when the AMS is attempting to perform Network Reentry, Secure Location Update, or HO, or a reentry if the AMS has a CMAC tuple necessary to expedite security authentication |
|  |  | CMAC_PN | 24 | Current CMAC_PN_U for uplink or current CMAC_PN_D for downlink. |  |
|  |  | CMAC Value | 64 | The CMAC value derived from the whole message. |  |
| O | Initial Offset for uplink power control (OffsetIInitial) |  | 5 | The bit size represents power level ranging from -15dB (0x00) to 26dB (0x1F) | The value is determined by AMS after successful initial ranging process |
| O | CMACI |  | 1 | CMAC indicator 0: CMAC tuple is not present 1: a 12 byte CMAC tuple is appended to the end of the message | Present if needed |

------------------------------------------------- End of proposed text II--------------------------------------------------

II.3 Proposed text 3

[Editor Note: at Page 195, Line 56, change as the following:]

------------------------------------------------- Start of proposed text III--------------------------------------------------

16.2.3.43 Privacy key MAC Control messages (AAI-PKM-REQ/AAI-PKM-RSP)

Table 723: AAI-PMK-REQ

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | PMK_SN | 4 | New PMK sequence number for PKM v3 message code = 4 Current PMK sequence number for PKM v3 message codes except 4 | Present whenPKM v3 message code = 1,4,6 or 8 |
| O | CMACdigest | 88 |  | Present whenPKM v3 message code = 1,4,6 or 8 |
| O | A) CMAC_PN_U | 24 | PN used to avoid UL replay attack on the control connection | Present whenPKM v3 message code = 1,4,6 or 8 |
| O | B) CMAC Value | 64 | CMAC value calculated using CMAC_KEY_U based on new PMK SN for PKM v3 message code = 4 CMAC value calculated using CMAC_KEY_U based on new PMK SN for PKM v3 message code except 4 | Present whenPKM v3 message code = 1,4,6 or 8 |
| O | CMACI | 1 | CMAC indicator 0: CMAC tuple is not present 1: A 12 byte CMAC tuple is appended to the end of the message | Present whenPKM v3 message code = 1,4,6 or 8 |

Table 724: AAI-PMK-RSP

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| O | PMK_SN | 4 | New PMK sequence number for PKM v3 message code = 4 Current PMK sequence number for PKM v3 message codes except 4 | Present whenPKM v3 message code = 1,4,6 or 8 |
| O | CMACdigest | 88 |  | Present whenPKM v3 message code = 3, 5, 7 or 8 |
| O | A) CMAC_PN_D | 24 | PN used to avoid DL replay attack on the control connection | Present whenPKM v3 message code = 3, 5, 7 or 8 |
| O | B) CMAC Value | 64 | CMAC value calculated using CMAC_KEY_D based on new PMK SN for PKM v3 message code = 3 or 5 CMAC value calculated using CMAC_KEY_D based on new PMK SN for PKM v3 message code except 3 and 5 | Present whenPKM v3 message code = 3, 5, 7 or 8 |
| O | CMACI | 1 | CMAC indicator 0: CMAC tuple is not present 1: A 12 byte CMAC tuple is appended to the end of the message | Present whenPKM v3 message code = 3, 5, 7 or 8 |

------------------------------------------------- End of proposed text III--------------------------------------------------

II.4 Proposed text 4

[Editor Note: Page 932, Line 48, change as the following:]

------------------------------------------------- Start of proposed text IV--------------------------------------------------

Annex R.2 MAC Control Message Definitions

CMAC ::= SEQUENCE {

pmkSN                          CPMKSN,

cmacPN                         CMACPN,

cmacValue                      CMACValue}

CMACI ::= ENUMERATED {

cmacNotPresent,

cmacPresent}

AAI-RNG-REQ ::= SEQUENCE {

amsidHashValue                 MACAddress OPTIONAL,

macVersion                     MACVersion OPTIONAL,

macAddress                     MACAddress OPTIONAL,

rangingPurpose                 RangingPurpose,

servingBsid                    BSID OPTIONAL,

crid                           CRID OPTIONAL,

previousSTID                   STID OPTIONAL,

previousBasicCID               CID OPTIONAL,

pagingControllerID             PCID OPTIONAL,

deregistrationID               DID OPTIONAL,

pagingGroupID                  PGID OPTIONAL,

pagingCycle                    PgCycle OPTIONAL,

pagingOffset                   PgOffset OPTIONAL,

pagingCycleChange              PgCycle OPTIONAL,

pagingCarrierUpdate            INTEGER(0..15) OPTIONAL,

akCount                        AKCount OPTIONAL,

amsMobility                    ENUMERATED {slow, medium, fast}

OPTIONAL,

csgIdList                      SEQUENCE (SIZE(0..15)) OF CSGID

OPTIONAL,

fidList                        SEQUENCE (SIZE(0..15)) OF FidInfo

OPTIONAL,

smsMessage                     SMS OPTIONAL,

cmac                           CMAC OPTIONAL,

-- The bit size represents power level ranging from -15dB (0x00) to

-- 26dB (0x1F)

-- The value is determined by AMS after successful initial ranging

-- process.

initialOffsetUlpc              INTEGER(0..31) OPTIONAL,

cmacIndicator                  CMACI OPTIONAL,

...

}

------------------------------------------------- End of proposed text IV--------------------------------------------------

II.5 Proposed text 5

[Editor Note: att page 948, line 33, change as the following:]

------------------------------------------------- Start of proposed text V--------------------------------------------------

Annex R.2 MAC Control Message Definitions

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

-- Security Messages

-- *-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-*-

PKMID ::= INTEGER (0..255)

PMKSN ::= INTEGER (0..255)

AKID ::= BIT STRING (SIZE(64))

SAID ::= INTEGER (0..255)

KeyLifetime ::= INTEGER (0..4294967295)

CounterTEK ::= INTEGER (0..65535)

EKS ::= INTEGER (0..3)

Nonce ::= BIT STRING (SIZE(64))

PKM-ReauthRequest ::= SEQUENCE {

cmacIndicator                  CMACI OPTIONAL

pmkSN                          PMKSN,

cmacDigest                     CMACDigest}

PKM-EAPTransfer ::= SEQUENCE {

eapPayload                     OCTET STRING (SIZE(1..1400))}

PKM-KeyAgreementMsg1 ::= SEQUENCE {

nonceABS                       Nonce,

pmkSN                          PMKSN,

akID                           AKID,

keyLifetime                    KeyLifetime,

cmacDigest                     CMACDigest

cmacIndicator                  CMACI OPTIONAL}

PKM-KeyAgreementMsg2 ::= SEQUENCE {

nonceABS                       Nonce,

nonceAMS                       Nonce,

akID                           AKID,

pmkSN                          PMKSN,

securityNegoParameters         SecurityNegotiationPara,

cmacDigest                     CMACDigest

cmacIndicator                  CMACI OPTIONAL}

PKM-KeyAgreementMsg3 ::= SEQUENCE {

nonceABS                       Nonce,

nonceAMS                       Nonce,

pmkSN                          PMKSN,

supportingSAs                  SupportingSAs,

securityNegoParameters         SecurityNegotiationPara,

cmacDigest                     CMACDigest

cmacIndicator                  CMACI OPTIONAL}

PKM-TEKRequest ::= SEQUENCE {

said                           SAID,

pmkSN                          PMKSN,

tekRefreshFlag                 ENUMERATED {

secondTEKUpdate,

firstTEKUpdate},

cmacDigest                     CMACDigest

cmacIndicator                  CMACI OPTIONAL}

PKM-TEKReply ::= SEQUENCE {

said                           SAID,

pmkSN                          PMKSN,

counterTEK                     CounterTEK,

eks                            EKS,

cmacDigest                     CMACDigest

cmacIndicator                  CMACI OPTIONAL}

PKM-TEKInvalid ::= SEQUENCE {

said                           SAID,

pmkSN                          PMKSN,

cmacDigest                     CMACDigest

cmacIndicator                  CMACI OPTIONAL}

CMACDigest ::= SEQUENCE {

cmacPN                         BIT STRING (SIZE(24)),

cmacValue                      BIT STRING (SIZE(64))}

SecurityNegotiationPara ::= SEQUENCE {

sizeOfICV                      ENUMERATED {

thirtyTwoBits,

sixtyFourBits},

windowSize                     INTEGER (0..65535)}

SupportingSAs ::= BIT STRING {

nullSASupported (0),

said1Supported (1),

said2Supported (2)} (SIZE(3))

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Privacy Key Management Request

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-PKM-REQ ::= SEQUENCE {

pkmid                          PKMID,

pkmMessage                     CHOICE {

reauthRequest                PKM-ReauthRequest,

eapTransfer                  PKM-EAPTransfer,

keyAgreementMsg2             PKM-KeyAgreementMsg2,

tekRequest                   PKM-TEKRequest,

tekInvalid                   PKM-TEKInvalid,

...

},

...

}

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

-- Privacy Key Management Response

-- +-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-+-

AAI-PKM-RSP ::= SEQUENCE {

pkmid                          PKMID,

pkmMessage                     CHOICE {

eapTransfer                  PKM-EAPTransfer,

keyAgreementMsg1             PKM-KeyAgreementMsg1,

keyAgreementMsg3             PKM-KeyAgreementMsg3,

tekReply                     PKM-TEKReply,

tekInvalid                   PKM-TEKInvalid,

...

},

...

}

------------------------------------------------- End of proposed text V--------------------------------------------------