| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Group resource allocation in 16m |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Bin Chen, Guanghui Fan, Junxian Mo, George Cummings Huawei | E-mail: binchen@huawei.com |
| Re: | Call for SB on “ P802.16m/D6”: |  |
| Abstract | This contribution analyses the tech problem of group resource allocation in D6, and then provide some solutions to this problem, and finally propose the comments to revise the text in the D6 specification. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Sponsor Ballot. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Group Resource Allocation in 16m

Bin Chen, Guanghui Fan, Junxian Mo, George Cummings

Huawei Technologies

- Introduction

In the specification of D6 [1], a tech problem is that the 4 burst sizes and their corresponding resource size are associated to the group (which could be found in the table 752—AAI_GRP-CFG. This makes the resource allocation in GRA A-MAP IE be not secure, since the burst size and corresponding resource size of the allocated radio resource for a dedicated flow are known by all the members in the group but not only the intended AMS, so that the MCS is known and traffic in the radio resource can be easily detected, saved and traced by other AMSs that is not the intended receiver of the traffic.

In this contribution, we would propose to associate the 4 burst sizes to each individual FID to avoid these disadvantages, and keep the 8 resource sizes associated to the group, so that there will be no overhead added by this solution.

- References

[1] IEEE P802.16m/D6

- Proposed text change in the P802.16m/D6

------------------------------------------ Begin Proposed Text Change 1-------------------------------------------

[Editor’s Note 2 : change the text in page 593, from line 26 to 34]

Resource Allocation Bitmap: The resource allocation bitmap uses 5 bits per flow to signal the HARQ burst size and the resource size for the AMS's allocation in that subframe. The first 2 bits signal the HARQ burst size and the next 3 bits signal the resource size. The 2-bit codes for burst sizes associated with the FID and the 3-bit resource sizes associated with the group are based on the information in the Group Configuration MAC control message

------------------------------------------ Begin Proposed Text Change 1-------------------------------------------

------------------------------------------ Begin Proposed Text Change 2-------------------------------------------

[Editor’s Note 3 : change the text in page 325, from line 32 to 41]

The third bitmap is the Resource Allocation bitmap which uses 2 bits per flow to signal HARQ burst size and 3 bits per flow to signal the Resource Size for the scheduled flow in the AAI subframe or extended AAI subframe that are scheduled in the frame. The resource size refers to the number of LRUs allocated to the flow. The resource size supported in GRA is limited to 16 LRUs. Each group supports eight resource sizes for each burst size supported in the group. The set of resource sizes for each burst size belong to the range [1,16] LRUs. The set of HARQ burst sizes and resource sizes supported in the group is signaled in the Group Configuration MAC control message. For groups corresponding to all MIMO mode sets except UL(0b11), the intended AMS calculates the starting location of resources of its flow with User Bitmap Index i in the AAI subframe as follows.

Ri=R0+j=1iLj

Where R0 is the resource offset of the group as signalled in the DL/UL Group Resource Allocation A-MAP IE and Lj is the resource size in LRUs of the flow in the group whose user bitmap index is j. If the flow with User Bitmap Index j is not indicated as present in the IE of User Bitmap in Group_Resource_Allocation_A-MAP_IE, Lj=0.

------------------------------------------ End Proposed Text Change 2-------------------------------------------

------------------------------------------ Begin Proposed Text Change 3-------------------------------------------

[Editor’s Note 4 : add below text in blue to page 211 line 48, and delete the text in page 213, from line 7 to 28, Table 752—AAI_GRP-CFG]

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| M | Message type | 8 |  |  |
|  | …. |  |  |  |
| M | FID | 4 | Identifies the flow that is added to or deleted from GRA group |  |
| O | Burst&Resource-Info [1..4] |  |  | Present if b == 1 |
| O | A) Burst size i | 5 | ith burst size of the 4 burst sizes supported for the FID | Present if b == 1 |
|  | …. |  |  |  |
| O | I) Resource size inclusion bitmap | 16 | Bitmap to signal which 8 resource sizes are supported in the group out of the range of [1,16] LRUs supported for GRA. If nth bit in the bitmap is set to 1, it signals that a resource size of n LRUs is supported in the group. | Present if f == 1 |

------------------------------------------ End Proposed Text Change 3-------------------------------------------