| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Dynamic resource partitioning for efficient multi-mode support |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Mark Cudak, Fred Vook, Bill Hillery, Eugene Visotsky, Anup Talukdar Motorola | Mark.Cudak@motorola.com |
| Re: | Category: P802.16m/D6 comments for IEEE 802.16m Sponsor Ballot Area: Chapter 16.3.3.7 |  |
| Abstract | IEEE 802.16m promises an exciting evolution path to today’s IEEE 802.16e network operators offering a system that simultaneously provides significantly higher spectral efficiency and protects the service provider’s investment in IEEE 802.16e devices. The Frame Configuration Index (FCI) is the current mechanism employed by the standard to partition the resources between IEEE 802.16m and IEEE 802.16e devices. The FCI mechanism falls short of delivering on the promise of backwards compatibility as it too rigidly partitions the resources between legacy and advanced devices while also being too slow to adapt to changing demand. This contribution proposes to eliminate the reliance on FCI index to balance resource between 16e and 16m and instead rely on blind detection of the A-MAP to identify whether the current subframe supports 16m or 16e. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Dynamic resource partitioning for efficient multi-mode support

Mark Cudak, Fred Vook, Bill Hillery,

Eugene Visotsky, Anup Talukdar

Motorola Inc

- Introduction

IEEE 802.16m promises an exciting evolution path to today’s IEEE 802.16e network operators offering a system that simultaneously provides significantly higher spectral efficiency and protects the service provider’s investment in IEEE 802.16e devices.  In addition, IEEE 802.16m promises lower latency for IEEE 802.16m devices while maintaining the low latency enjoyed by the existing IEEE 802.16e devices.  These promises are critical to the success of IEEE 802.16m standard differentiating the standard from other potential 16e air-interface evolution options.

The Frame Configuration Index (FCI) is the current mechanism employed by the standard to partition the resources between IEEE 802.16m and IEEE 802.16e devices.  Specifically, the FCI defines how many subframes are allocated to IEEE 802.16e and how many subframes are allocated to IEEE 802.16m. The FCI mechanism falls short of delivering on the promise of backwards compatibility as it too rigidly partitions the resources between legacy and advanced devices while also being too slow to adapt to changing demand.

- The FCI mechanism is too rigid in hard coding the subframes as being 16m or 16e.  This strict partitioning is unnecessary and redundant.  For IEEE 802.16e devices, the FCI is irrelevant as 16e devices will determine the location of the 16e resources from DL-MAP.  The DL-MAP tells each 16e device precisely where zones begin and end as well as the location of their allocated resources.  IEEE 802.16m devices can just as easily determine the location of the 16m resources without the aid of the FCI.  For example, the 16m device can blindly attempt to detect the presence of the IEEE 802.16m A-MAP; when an A-MAP is present, the 16m device will identify the PRUs allocated to it, and when the A-MAP is not present the 16m device can ignore the subframe.

- The FCI mechanism is slow to adapt as it is tied to the S-SFH update procedure.  The S-SFH update procedure must hold the current configuration for at least 160 ms before the system can rebalance between 16m and 16e.   Conceivably, 16e data arriving at the BS may be delayed 160 ms before the air interface can be reconfigured and the data may be scheduled.  Alternatively, the air interface could  maintain an artificially even balance between 16e and 16m resources in all frames to avoid the delay; however, this would most certainly reduce system spectral efficiency and the peak achievable throughput rate.  Neither of these solutions delivers on the promise of IEEE 802.16m

This contribution proposes to eliminate the reliance on FCI mechanism to balance resource between 16e and 16m and instead rely on blind detection of the A-MAP to identify whether the current subframe supports 16m or 16e.

- Proposed Text Changes in P802.16m/D6

[Remedy 1: Redefine the FCI to only identify the cyclic prefix length, duplex mode, TDD split, subframe types and frame offset.  Columns “DL Length” and “UL length” will be deleted.  Without these two columns, several rows will appear redundant.  These redundant rows are deleted. 	Make the following changes to section 16.3.3.7]

16.3.3.7 Set of frame configurations

Table 807, Table 808, and Table 809 show sets of the frame configurations and indexing for 5/10/20MHz, 8.75MHz, and 7MHz, respectively. Note that per each combination of bandwidth and CP length, frame configuration information is carried by frame configuration index in S-SFH SP1 IE in Table 840.

The frame structure supporting the WirelessMAN-OFDMA frame defined in 16.3.3.5.1, shall be configured with one of the following configuration numbers (i.e. 'No.' in the tables): 11 to 22 13, 17, 32 and 36 in Table 807 for 5/10/20MHz channel bandwidths; 9 to 14  9 and 11 in Table 808 for 8.75MHz channel bandwidth; 7 to 8 in Table 809 for 7MHz channel bandwidth. In this case, DL offset and DL length denote Frame_Offset and the number of DL AAI subframes dedicated to Advanced Air Interface operations, respectively. UL length denotes the number of UL AAI subframes dedicated to Advanced Air Interface operations. The DL offset specifies the position of the 16m preamble with respect to the 16e preamble.  Table 807 for 5/10/20 MHz and Table 808 for 8.75 MHz support multiple DL offsets for given TDD split.

When supporting WirelessMAN-OFDMA, for the case when UL length is less than total number of UL AAI subframes in a frame, the UL TDM mode defined in 16.3.3.5.1 is applied. For the case when UL length is the same as the total number of UL AAI subframes in a frame, the UL FDM mode defined in 16.3.3.5.1 is applied.

Table 807—Frame Configuration and Indexing (5/10/20MHz channel bandwidth)

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| 1 | 5/10/20 | 1/16 | 0 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 2 | 5/10/20 | 1/16 | 1 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 3 | 5/10/20 | 1/16 | 2 | TDD | 4:4 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 4 | 5/10/20 | 1/16 | 3 | TDD | 3:5 | N/A | N/A | N/A | DL Type1 | DL Type2 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type2 | 82.853 / 60 |
| 5 | 5/10/20 | 1/16 | 4 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 | N/A |
| 6 | 5/10/20 | 1/8 | 0 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | 105.714 / 60 |
| 7 | 5/10/20 | 1/8 | 1 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | 105.714 / 60 |
| 8 | 5/10/20 | 1/8 | 2 | TDD | 4:4 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | 105.714 / 60 |
| 9 | 5/10/20 | 1/8 | 3 | TDD | 3:5 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 | 105.714 / 60 |
| 10 | 5/10/20 | 1/8 | 4 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | N/A |
| 11 | 5/10/20 | 1/8 | 5 | TDD | 5:3 | 3 | 2 | 3 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | Not used | Not Used | Not Used | 105.714 / 60 |
| 12 | 5/10/20 | 1/8 | 6 | TDD | 5:3 | 3 | 2 | 1 | DL Type1 | DL Type1 | Not Used | Not Used | UL Type1 | Not used | Not Used | Not Used | 105.714 / 60 |
| 13 | 5/10/20 | 1/8 | 7 | TDD | 5:3 | 2 | 3 | 3 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 14 | 5/10/20 | 1/8 | 8 | TDD | 5:3 | 2 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 15 | 5/10/20 | 1/8 | 9 | TDD | 5:3 | 2 | 2 | 3 | DL Type1 | DL Type1 | Not used | UL Type1 | UL Type1 | UL Type1 | Not used | Not Used | 105.714 / 60 |
| 16 | 5/10/20 | 1/8 | 10 | TDD | 5:3 | 2 | 2 | 1 | DL Type1 | DL Type1 | Not used | Not used | Not Used | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 17 | 5/10/20 | 1/8 | 11 | TDD | 5:3 | 1 | 4 | 3 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 18 | 5/10/20 | 1/8 | 12 | TDD | 5:3 | 1 | 4 | 2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 19 | 5/10/20 | 1/8 | 13 | TDD | 5:3 | 1 | 3 | 3 | DL Type1 | DL Type1 | DL Type1 | Not used | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 20 | 5/10/20 | 1/8 | 14 | TDD | 5:3 | 1 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not used | Not used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 21 | 5/10/20 | 1/8 | 15 | TDD | 5:3 | 1 | 2 | 3 | DL Type1 | DL Type1 | Not used | Not used | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 22 | 5/10/20 | 1/8 | 16 | TDD | 5:3 | 1 | 2 | 1 | DL Type1 | DL Type1 | Not used | Not used | Not used | Not used | UL Type1 | Not Used | 105.714 / 60 |
| 23 | 5/10/20 | 1/8 | 17 | TDD | 6:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type1 | 208.571 / 60 |
| 24 | 5/10/20 | 1/8 | 18 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type3 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | 208.571 / 60 |
| 25 | 5/10/20 | 1/8 | 19 | TDD | 5:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type3 | DL Type3 | DL Type3 | UL Type1 | UL Type1 | UL Type1 | 311.428 / 60 |
| 26 | 5/10/20 | 1/8 | 20 | TDD | 5:3 | 2 | 1 | 3 | DL Type2 | Not Used | Not Used | UL Type1 | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 27 | 5/10/20 | 1/8 | 21 | TDD | 5:3 | 1 | 1 | 3 | DL Type2 | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 28 | 5/10/20 | 1/8 | 22 | TDD | 6:2 | 1 | 1 | 2 | DL Type2 | Not Used | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 29 | 5/10/20 | 1/8 | 23 | TDD | 6:2 | 1 | 2 | 2 | DL Type1 | DL Type1 | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 30 | 5/10/20 | 1/8 | 24 | TDD | 6:2 | 1 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 31 | 5/10/20 | 1/8 | 25 | TDD | 6:2 | 1 | 4 | 2 | DL Type1 | DL Type1 | DL Type1‘ | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 32 | 5/10/20 | 1/8 | 26 | TDD | 6:2 | 1 | 5 | 2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not Used | 105.714 / 60 |
| 33 | 5/10/20 | 1/8 | 27 | TDD | 6:2 | 2 | 1 | 2 | DL Type2 | Not Used | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 34 | 5/10/20 | 1/8 | 28 | TDD | 6:2 | 2 | 2 | 2 | DL Type1 | DL Type1 | Not Used | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 35 | 5/10/20 | 1/8 | 29 | TDD | 6:2 | 2 | 3 | 2 | DL Type1 | DL Type1 | DL Type1 | Not Used | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 36 | 5/10/20 | 1/8 | 30 | TDD | 6:2 | 2 | 4 | 2 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not Used | Not Used | 105.714 / 60 |
| 37 | 5/10/20 | 1/4 | 0 | TDD | 5:2 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 |  | 139.988 / 60 |
| 38 | 5/10/20 | 1/4 | 1 | TDD | 4:3 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 |  | 139.988 / 60 |
| 39 | 5/10/20 | 1/4 | 2 | TDD | 3:4 | N/A | N/A | N/A | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 |  | 139.988 / 60 |
| 40 | 5/10/20 | 1/4 | 3 | FDD | N/A | N/A | N/A | N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |

Table 808—Frame Configuration and Indexing (8.75MHz channel bandwidth)

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| 1 | 8.75 | 1/16 | 0 | TDD | 5:2 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type2 |  | 138.4/ 74.4 |
| 2 | 8.75 | 1/16 | 1 | TDD | 4:3 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type2 |  | 138.4/ 74.4 |
| 3 | 8.75 | 1/16 | 2 | TDD | 3:4 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type2 |  | 138.4/ 74.4 |
| 4 | 8.75 | 1/16 | 3 | FDD | N/A | N/A | - N/A | - N/A | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type2 |  | N/A |
| 5 | 8.75 | 1/8 | 0 | TDD | 5:2 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 |  | 87.2/ 74.4 |
| 6 | 8.75 | 1/8 | 1 | TDD | 4:3 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 |  | 87.2/ 74.4 |
| 7 | 8.75 | 1/8 | 2 | TDD | 3:4 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 | UL Type1 |  | 87.2/ 74.4 |
| 8 | 8.75 | 1/8 | 3 | FDD | N/A | N/A | - N/A | - N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 | D/U Type1 |  | N/A |
| 9 | 8.75 | 1/8 | 4 | TDD | 5:2 | 3 | - 2 | - 2 | DL Type1 | DL Type1 | UL Type1 | UL Type4 | Not Used | Not Used | Not Used |  | 87.2/ 74.4 |
| - 10 | - 8.75 | - 1/8 | - 5 | - TDD | - 5:2 | - 3 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not Used | - UL Type1 | - Not Used | - Not Used | - Not Used |  | - 87.2/ 74.4 |
| 11 | 8.75 | 1/8 | 6 | TDD | 5:2 | 2 | - 3 | - 2 | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type4 | Not Used | Not Used |  | 87.2/ 74.4 |
| - 12 | - 8.75 | - 1/8 | - 7 | - TDD | - 5:2 | - 2 | - 3 | - 1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not Used | - UL Type1 | - Not Used | - Not Used |  | - 87.2/ 74.4 |
| - 13 | - 8.75 | - 1/8 | - 8 | - TDD | - 5:2 | - 2 | - 2 | - 2 | - DL Type1 | - DL Type1 | - Not Used | - UL Type1 | - UL Type4 | - Not Used | - Not Used |  | - 87.2/ 74.4 |
| - 14 | - 8.75 | - 1/8 | - 9 | - TDD | - 5:2 | - 2 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not Used | - Not Used | - UL Type1 | - Not Used | - Not Used |  | - 87.2/ 74.4 |
| - 15 | - 8.75 | - 1/8 | - 10 | - TDD | - 5:2 | - 2 | - 1 | - 2 | - DL Type2 | - Not Used | - Not Used | - UL Type1 | - UL Type4 | - Not Used | - Not Used |  | - 87.2/ 74.4 |
| 16 | 8.75 | 1/4 | 0 | TDD | 4:2 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | DL Type1 | DL Type1 | UL Type1 | UL Type1 |  |  | 189.6/74.4 |
| 17 | 8.75 | 1/4 | 1 | TDD | 3:3 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | DL Type1 | UL Type1 | UL Type1 | UL Type1 |  |  | 189.6/74.4 |
| 18 | 8.75 | 1/4 | 2 | TDD | 2:4 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | UL Type1 | UL Type1 | UL Type1 | UL Type1 |  |  | 189.6/74.4 |
| 19 | 8.75 | 1/4 | 3 | FDD | N/A | N/A | - N/A | - N/A | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type2 |  |  | N/A |

Tab Table 809—Frame Configuration and Indexing (7MHz channel bandwidth)

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - Subframe provision |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL - offset | - DL length | - UL - length | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| 1 | 7 | 1/16 | 0 | TDD | 4:2 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 |  |  | 180/ 60 |
| 2 | 7 | 1/16 | 1 | TDD | 3:3 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | DL Type3 | UL Type1 | UL Type1 | UL Type1 |  |  | 180/ 60 |
| 3 | 7 | 1/16 | 2 | FDD | N/A | N/A | - N/A | - N/A | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 | D/U Type1 |  |  | N/A |
| 4 | 7 | 1/8 | 0 | TDD | 3:2 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | DL Type2 | UL Type1 | UL Type2 |  |  |  | 188/ 60 |
| 5 | 7 | 1/8 | 1 | TDD | 2:3 | N/A | - N/A | - N/A | DL Type1 | DL Type2 | UL Type1 | UL Type2 | UL Type2 |  |  |  | 188/ 60 |
| 6 | 7 | 1/8 | 2 | FDD | N/A | N/A | - N/A | - N/A | D/U Type1 | D/U Type2 | D/U Type2 | D/U Type2 | D/U Type2 |  |  |  | N/A |
| 7 | 7 | 1/8 | 3 | TDD | 3:2 | 1 | - 2 | - 2 | DL Type1 | DL Type1 | UL Type1 | UL Type1 | Not Used |  |  |  | 188/ 60 |
| - 8 | - 7 | - 1/8 | - 4 | - TDD | - 3:2 | - 1 | - 2 | - 1 | - DL Type1 | - DL Type1 | - Not Used | - UL Type1 | - Not Used |  |  |  | - 188/ 60 |
| - 9 | - 7 | - 1/8 | - 5 | - TDD | - 3:2 | - 1 | - 1 | - 2 | - DL Type2 | - Not Used | - UL Type1 | - UL Type1 | - Not Used |  |  |  | - 188/ 60 |
| 10 | 7 | 1/4 | 0 | TDD | 3:2 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | DL Type1 | UL Type1 | UL Type1 |  |  |  | 140/60 |
| 11 | 7 | 1/4 | 1 | TDD | 2:3 | N/A | - N/A | - N/A | DL Type1 | DL Type1 | UL Type1 | UL Type1 | UL Type1 |  |  |  | 140/60 |
| 12 | 7 | 1/4 | 2 | FDD | N/A | N/A | - N/A | - N/A | D/U Type1 | D/U Type1 | D/U Type2 | D/U Type1 | D/U Type1 |  |  |  | N/A |