| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on AMS Ranging in Femtocells (16.4.7.4) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Pei-Kai Liao MediaTek Inc. | pk.liao@mediatek.com |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D6) |  |
| Abstract | This contribution proposes clarification on S-RCH Parameters in S-SFH SP1 IE |  |
| Purpose | Propose to be discussed and adopted by TGm for the 802.16m Amendment |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on AMS Ranging in Femtocells (16.4.7.4)

Pei-Kai Liao

MediaTek Inc.

- Introduction

In Session #67, transmission timing offset of S-RCH is added into S-SFH SP1 IE for AMS to operate initial/handover ranging in a femtocell.  However, there is no explanation regarding to this in the section of “Support of Femto ABS” so it may cause confusion about AMS ranging operation in a femtocell.  The proposed text can clarify the confusion.

- Text Proposal

[Modify the following text in IEEE P802.16m/D6]

-------------------------------------------------------- Start of the Text ------------------------------------------------

16.4.7.4 Ranging

16.4.7.4.1 Ranging channel configuration

The ranging channel for synchronized AMSsS-RCH is used for initial ranging, handover ranging and periodic ranging of a femto cell. BW REQ is done in exactly the same manner as in a regular Macro cell.

16.4.7.4.2 Ranging channel transmission

When the Femto ABS and the macro ABS operate on the same frequency, AMS shall advance the transmission timing of S-RCH based on the S-RCH transmission timing offset broadcasted by Femto ABS in S-SFH SP1 to conduct uplink initial/handover ranging in a femtocell.

--------------------------------------------------------- End of the Text ------------------------------------------------