| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of CoRe Version Indication (section 16.3.11.4.2) |  |
| Date Submitted | 2010-06-28 |  |
| Source(s) | Jin Xu, Xianwei Gong and Bo Sun ZTE Corporation | sun.bo1@zte.com.cn xu.jin7@zte.com.cn |
| Re: | Comments on IEEE P802.16m/D6 for IEEE 802.16 WG SB_16m Target topic: 16.13.11.4.2 |  |
| Abstract | This contribution provides the proposed text for 16.3.11.4.2 Constellation rearrangement |  |
| Purpose | To be discussed and adopted into P802.16m/D6 by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of CoRe Version Indication (section 16.3.11.4.2)

Jin Xu, Xianwei Gong, Bo Sun

ZTE Corporation

- Introduction

This contribution proposes text change on the current CoRe(Constellation Rearrangement) indication (Section 16.3.11.4.2) in P802.16m/D6 [1] to avoid ambiguities and easy for understanding.

- References

[1] IEEE P802.16m/D6, “Draft Amendment to IEEE standard for local and metropolitan area networks: Air Interface for Fixed and Mobile Broadband Wireless Access Systems,” May. 2010.

[2] IEEE C802.16m-10/0250r2, “Clarification of Constellation Rearrangement (section 16.3.11.4.2)” Mar. 2010.

- Text proposal for inclusion in P802.16m/D6

[Modify the text line 1 in page 781 ~ the text line 37 in page 781 as follows]

-------------------------------------------------------- Text Start --------------------------------------------------------

For kth FEC block, NCTC,k coded bits are mapped onto NQAM QAM symbols applying CoRe mapping rule in Table 940 and 941. In Table 941, a pair of two QAM symbols uses the same value of CRV(Constellation Rearrangement Version) when the number of MIMO stream is larger than 1. For a QAM symbol index j, an ‘Even symbol’ has its index j mod 2 =0 and an ‘Odd symbol’ has its index j mod 2 =1 where j = 0, 1, 2, …, NQAM,k -1 n = 0, 1, 2, …, NQAM,k -1. The QAM symbol index j represents its transmit order.

In DL HARQ operation, CRV (Constellation Rearrangement Version) is signaled for each subpacket to an AMS as a starting value for CRV. For data initially transmitted using persistent allocated resources, the value of the CRV indicated by DL Persistent Allocation A-MAP IE shall be zero. In UL HARQ operation, the information on CRVs is implicitly known at AMS and ABS and the starting value for CRV of the subpacket i shall be (i⋅NCTC,k)NFBBuffer,kmod 2.

In DL and UL HARQ operation, the value of CRV is determined assuming Nshift,i is 0 for all subpacket i while the coded bits are mapped on to the QAM symbols in their transmit order considering Nshift,i.

The CRV shall be changed to the other version of CoRe in DL and UL HARQ operation whenever the transmitted bits wraparound at the end of circular buffer per SPID.

Let CRVj,k be the value of CRV for jth QAM symbol for the kth FEC block. Based on Nshift,i =0, the The value CRVj,k is calculated by Equation (314).		.

-------------------------------------------------------- Text End --------------------------------------------------------