| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Change for Feedback Polling A-MAP IE (16.3.6.5.2.4.11) |  |
| Date Submitted | 2010-07-12 |  |
| Source(s) | Wookbong Lee LG Electronics | wookbong.lee@lge.com |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Change for Feedback Polling A-MAP IE (16.3.6.5.2.4.11)

Wookbong Lee

LG Electronics

- Introduction

Some of description for period in Feedback Polling A-MAP IE section is wrong.

For example, MFM = 1 only supports STC rate 1/2. MFM = 2 does not feedback Stream index but STC rate.

Also, for some MFM, there is no description for p = 0 and q > 0.

- References

[1] Draft Amendment, P802.16m/D6

Remedy : In line 51 of page 603 to line 28 of page 604, modify tests as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

For MFM 0 or 1: When p > 0, q = 0, the wideband CQI and wideband STC rate shall be transmitted using short feedback period. When p = 0, q > 0, the wideband CQI and wideband STC rate shall be transmitted using long feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 0 or 1.

For MFM 1: When p > 0, q = 0, the wideband CQI shall be transmitted using short feedback period. When p = 0, q > 0, the wideband CQI and shall be transmitted using long feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 1.

For MFM 2 or 5: When p > 0, q = 0, the Best_subbands_index, subband CQI and STC rate Stream index shall be transmitted using short feedback period. When p = 0, q > 0, the Best_subbands_index, subband CQI and STC rate shall be transmitted using long feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 2 or 5.

For MFM 3: when p > 0, q = 0, the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the base codebook shall be transmitted using short period. When p = 0, q > 0, only the correlation matrix shall be transmitted using long feedback period. When p > 0, q > 0 , the Best_subbands_index, STC_Rate, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 5: When p > 0, q = 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using short feedback period. When p = 0, q > 0, the Best_subbands_index, subband CQI and Stream index shall be transmitted using long feedback period. p > 0 and q > 0 shall not be set at the same time with MFM 5.

For MFM 6: When p > 0, q > 0, the Best_subbands_index, subband CQI and subband PMI from the transformed codebook shall be transmitted using short period, while the correlation matrix shall be transmitted using long period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period. The ABS shall not set p > 0 and q = 0 simultaneously.

For MFM 4: when p > 0 and q = 0, the wideband CQI, STC rate and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI and STC rate shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

For MFM 7: when p > 0 and q = 0, the wideband CQI and PMI from the base codebook shall be transmitted using short feedback period. When p > 0 and q > 0, the wideband CQI shall be transmitted using short feedback period, and the correlation matrix shall be transmitted using long feedback period. When p = 0 and q > 0, only the correlation matrix shall be transmitted using long feedback period.

-------------------------------------------------Text Change END -----------------------------------------------------