| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Change for UL Sounding Command A-MAP IE (16.3.6.5.2.4.6) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Wookbong Lee LG Electronics | wookbong.lee@lge.com |
| Re: | Comment on IEEE P802.16m/D6 for Sponsor Ballot |  |
| Abstract |  |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D7 Draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Change for UL Sounding Command A-MAP IE (16.3.6.5.2.4.6)

Wookbong Lee

LG Electronics

- Introduction

It is not defined where "Multiplexing type" is transmitted. It is transmitted via AAI_SCD message with name "SoundingMultiplexingType."

Also, there is something wrong in sounding related information indication part in AAI_SCD message.

- References

[1] Draft Amendment, P802.16m/D6

Remedy #1: Modify table 857 (line 37 of page 583) as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| … | … | … |
| If (Multiplexing type == 0) { |  | Multiplexing type is transmitted in AAI_SCD message. |
| … | … | … |

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #2: Modify table 716 (line 24-32 of page 160) as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

| M/O | Attributes / Array of attributes |  | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- | --- |
| … | … |  | … | … | … |
| O | SoundingMultiplexingType | decima-tionVal-ueD | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) | Sounding sequence: Presents when Sounding Multiplexing Type is set to Decimation_ValueD |
|  |  | maxCy-clicShift-IndexP | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) |  |
| … | … |  | … | … | … |

| M/O | Attributes / Array of attributes | Size (bits) | Value / Note | Conditions |
| --- | --- | --- | --- | --- |
| … | … | … | … | … |
| O | multiplexingType | 1 | 0b0: decimation separation 0b1: cyclic shift separation | Presents when Uplink AAI subframes for sounding in S-SFH SP1 is not set to 0b000 |
| O | decimationValueD | 3 | decValueD0 (4), decValueD1 (6), decValueD2 (8), decValueD3 (9), decValueD4 (12), decValueD5 (16), decValueD6 (18), decValueD7 (36) | Presents when MultiplexingType is equal to 0b0. |
| O | maxCyclicShiftIndexP | 3 | csIndexP0 (4), csIndexP1 (6), csIndexP2 (8), csIndexP3 (9), csIndexP4 (12), csIndexP5 (16), csIndexP6(18), csIndexP7(36) | Presents when MultiplexingType is equal to 0b1. |
| … | … | … | … | … |

-------------------------------------------------Text Change END -----------------------------------------------------

Remedy #3: In line 22-44 of page 877, modify ASN.1 code as follows:

-------------------------------------------------Text Change Start -----------------------------------------------------

multiplexingType			INTEGER (0..1) OPTIONAL,

soundingMultiplexingType	CHOICE {

decimationValueD 		ENUMERATED {

decValueD0 (6),

decValueD1 (8),

decValueD2 (9),

decValueD3 (12),

decValueD4 (16),

decValueD5 (18),

decValueD6 (36)} OPTIONAL,

maxCyclicShiftIndexP		ENUMERATED {

csIndexP0 (4),

csIndexP1 (6),

csIndexP2 (8),

csIndexP3 (9),

csIndexP4 (12),

csIndexP5 (16),

csIndexP6 (18),

csIndexP7 (36)} OPTIONAL,

},

-------------------------------------------------Text Change END -----------------------------------------------------