| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ Reset Changes (16.2.13.5.4) |  |
| Date Submitted | 2010-07-09 |  |
| Source(s) | Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son Samsung Electronics | anilag@samsung.com, yb.chang@samsung.com, *<http://standards.ieee.org/faqs/affiliationFAQ.html |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D6). Topic: ARQ (Section 16.2.13.5.4) |  |
| Abstract | This contribution proposes changes to ARQ Reset procedure |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D7. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Copyright Policy | The contributor is familiar with the IEEE-SA Copyright Policy <http://standards.ieee.org/IPR/copyrightpolicy.html>. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ Reset

Anil Agiwal, Youngbin Chang, Rakesh Taori, Jungje Son

Samsung Electronics

# Introduction

In the ARQ reset procedure defined in D6 specification [1], ARQ receiver can receive the ARQ packets which were transmitted by ARQ transmitter before the transmission of ARQ reset because of HARQ. For example consider the sequence:

- Transmitter sends ARQ packets

- Transmitter initiates ARQ reset and disables transmission of ARQ packets

- Transmitter sends ARQ reset

The ARQ packets in the ongoing HARQ transmissions at the time of sending ARQ reset message, can be received by the receiver after receiving the ARQ reset message and after it has reset its windows. This will lead to discarding of new packets as duplicates once the new ARQ packet transmissions are initiated by the transmitter. In some cases it can also lead to combination of two SDU fragments, one from old transmission and one for new transmission leading to delivery of incorrect SDU to upper layer.

# Proposed Solution

To avoid the above problem, during the ARQ reset, the ARQ windows at the transmitter and receiver should start from SN, which is not part of the transmitter’s ARQ window. This way all the new transmissions will be having different SNs with respect to older transmissions. The older transmissions received after ARQ reset will get discarded as out of window at the receiver.

# Proposed text

[Change 1: Modify the text on page 359, section 16.2.13.5.4, lines 4 -11 as follows]

16.2.13.5.4 ARQ Reset procedure

The ARQ reset procedures defined in figures 50 xxx and yyy 51(see 6.3.4.6.2) shall be supported by both the ARQ receiver and ARQ transmitter. Each side (receiver or transmitter) may initiate an ARQ reset procedure when ARQ Synchronization loss (see <<15.3.8.7.5>> 16.2.13.5.5) happens or for any other implementation specific reason. When ARQ reset error happens during the ARQ reset procedure, the ABS or AMS may reinitialize its MAC.

[Change 2: Add Figures xxx and Figures yyy as shown below after 16.2.13.5.4]

Figure xxx Transmitter Initiated ARQ Reset

# Figure yyy Receiver Initiated ARQ Reset

[Change 3: Modify the section 16.2.3.45, lines 23-54 as follows]

16.2.3.45 AAI_ARQ-Reset message

The transmitter or receiver may send this message. The message is used in a dialog to reset the parent connection’s ARQ transmitter and receiver state machines. The detail ARQ reset procedure is described in the section 16.2.13.5.4 <<15.3.8.7.4>>.

Table 741 – AAI_ARQ-Reset message format

| Syntax | Size (bits) | Note |
| --- | --- | --- |
| AAI_ARQ-Reset_Message_Format() { |  |  |
| MAC control Message type = 28 |  |  |
| DL/UL indicator | 1 | 0b0 = Downlink 0b1 = Uplink |
| Flow ID | 4 | Corresponding connection ID to perform ARQ reset procedure. |
| Type | 2 | 0b00 = Original message from Initiator 0b01 = Acknowledgement from Responder 0b10 = Confirmation from Initiator 0b11 = Reserved |
| SN | 10 | This field shall be included when the ARQ transmitter sends an ARQ reset message with type 0b00 or 0b01. ARQ transmitter sets this field to (ARQ_TX_WINDOW_START_SN + ARQ_WINDOW_SIZE) mod (ARQ_SN_MODULUS). |
| Reserved | 17 | Shall be set to zero |
| } |  |  |

# Reference

[1] IEEE P802.16m/D6, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”