| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Handling change in Femto LDM parameters for the IEEE 802.16m/D6 Amendment (16.4.10.2) |  |
| Date Submitted | 2010-07-03 |  |
| Source(s) | Anshuman Nigam, Ying Li, Jung Je Son Samsung Electronics Andreas Maeder, Linghang Fang NEC Europe | Email: anshun@samsung.com, yli2@sta.samsung.com jungje.son@samsung.com Andreas.Maeder@neclab.eu Linghang.Fan@EU.NEC.COM |
| Re: | Sponsor Ballot on the Draft Amendment (IEEE P802.16m/D6) |  |
| Abstract | This contribution provides the proposed text for broadcasting LDM parameters in AAI_NBR-ADV |  |
| Purpose | To be discussed and adopted into P802.16m/D6 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Handling change in Femto LDM parameters (16.4.10.2)

Anshuman Nigam, Ying Li, Jung Je Son

Samsung Electronics

Andreas Maeder, Linghang Fang

NEC Europe

- Introduction

As per D6 the LDM parameters are provided to the AMS on network entry either in AAI_REG-RSP or can be pre-provisioned. However the LDM parameters can change though not often. There exists no simple means to intimate the change of LDM parameters to the AMS. The AMS may be paged to do forced network entry for receiving the new LDM parameters but this is not practical as all the AMSs need to wake up to receive the same set of information. Hence there exists a need to provide the new LDM parameters to the AMS in an efficient manner.

We propose to broadcast the LDM parameters in AAI_NBR-ADV message. However since the LDM parameters will change rarely, we further propose to broadcast them only when they are changed. The broadcast of the new parameters can be restricted to certain duration of time which is judged suitable enough by the network to ensure reception of the new parameters by all the AMSs in the network. The time for example can be few repetitions of the longest paging cycle in the network.

- References

[1] IEEE P802.16m/D6, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, June 2010.

- Text proposal

Instruction to editor;

Black text: existing D6 text

Red and strike out text: for deletion

Blue and underline text: for addition

[Remedy 1:]

[Modify line 11 onwards on the page 798, under subsection 16.4.10.2, as follows:]

============================== Start of Proposed Text =================================

There may be one or more default LDM patterns in a Femto ABS deployment. The Default LDM pattern(s) parameters can be pre-provisioned or unicasted to the AMS during initial network entry with the Femto ABS in the AAI_REG-RSP message. The LDM parameters can be broadcasted in AAI_NBR-ADV message, by the ABS when they are changed, for certain duration of time as decided by the network.

============================== End of Proposed Text =================================

[Remedy 2:]

[Apend in table 692 on the page 117 , under subsection 16.2.3.12, as follows:]

============================== Start of Proposed Text =================================

| M/O | Attributes/Array of Attributes | Size (bits) | Value/Note | Conditions |
| --- | --- | --- | --- | --- |
| O | LDM Parameter - AI | 4 | The new AI for LDM | It shall be included when the LDM parameters are changed. It will be broadcasted for a certain duration of time as decided by the network |
| O | LDM Parameter - UAI | 8 | The new UAI for LDM | It shall be included when the LDM parameters are changed. It will be broadcasted for a certain duration of time as decided by the network |
| O | LDM Parameter - SFO | 9 | The new SFO for LDM | It shall be included when the LDM parameters are changed. It will be broadcasted for a certain duration of time as decided by the network |

============================== End of Proposed Text =================================