| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of NSCF Operation (16.2.17) |  |
| Date Submitted | 2010-07-14 |  |
| Source(s) | Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon ETRI | jscha@etri.re.kr, *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE Sponsor Ballot on P802.16m/D6 |  |
| Abstract | This contribution clarifies the NSCF Operation. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of NSCF Operation (16.2.17)

Jaesun Cha, Soojung Jung, Eunkyung Kim, Sungcheol Chang, Hyun Lee, Chulsik Yoon

ETRI

# Introduction

In this contribution, we propose to remove NSCF from SCH and to modify the definition of NSCF included in AAI_SLP-RSP message. The rationale for this proposal is as follows;

## Usage of SCH and AAI_SLP-RSP

The current draft defines two different methods for changing NSCF value; SCH and AAI_SLP-RSP message. However, there is no clear condition for when SCH or AAI_SLP-RSP message is used for NSCF change. If we remove NSCF from SCH, the usage of SCH and AAI_SLP-RSP can be more clear. Because SCH does not contain any parameters related with Sleep Cycle setting, it will be used when ABS wants to control the AMS behavior but does not need any changes of Sleep Cycle setting. If any change of Sleep Cycle setting itself is needed, then AAI_SLP-RSP shall be used.

## Efficiency of NSCF

To my understanding, NSCF is introduced to exclude the transmission of any control messages and SMS during Listening Window from the consideration of Sleep Cycle update. We agree with the concept of NSCF. However, the current operation seems to be somewhat inefficient. Let’s consider the following scenario.

- 1st Listening Window (transmission of only user traffic): SCH with NCSF = 0b00

- 2nd Listening Window (transmission of only control traffic): SCH with NSCF = 0b01

- 3rd Listening Window (transmission of only user traffic): SCH with NSCF = 0b00

In this scenario, ABS shall transmit SCH with NCSF every Listening Window based on the traffic pattern during Listening Window. This assumption may be an extreme case. But, the problem is that additional transmission of SCH may be expected when traffic type in the previous Listening Window is different from that in the current Listening Window. If the purpose of NSCF is to exclude the transmission of any control messages and SMS during Listening Window from the consideration of Sleep Cycle update, then we can use another single semi-static rule and such inefficiency can be avoided.

In this contribution, we propose to use the following rule for Sleep Cycle update.

| NCSF = 0 | In this case, user traffic and control message are considered for Sleep Cycle update. Therefore, if there is any traffic during Listening Window, then Sleep Cycle is reset to initial Sleep Cycle. |
| --- | --- |
| NCSF = 1 | In this case, only user traffic is considered for Sleep Cycle update. Therefore, if there are only control messages during Listening Window, then Sleep Cycle is updated according to the normal Sleep Cycle update rule (Equation 176 in the draft) |

The same rule can be applied for the transmission of SMS because SMS is sent and received by AAI_L2-XFER message.

# Text Proposal

---------------------------------------------------------Start of the Text----------------------------------------------------------

[Modify Table 660 as follows]

Table 660 – Sleep Control header format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Sleep Control Header Format () { | ······ | ······ |
| FID | 4 | Flow Identifier. Set to 001. |
| Type | 5 | MAC signaling header type = 0b00011 |
| Length | 3 | Indicates the length of the signaling header in bytes. |
| SCH sub-type | 3 | 0b000 = Listening Window control 0b001 = Resume Sleep Cycle Indication 0b010 = Sleep cycle configuration switch 0b011 = Sleep Cycle control 0b011100 = Multi-Carrier Listening Window control 0b100101~0b111 = reserved |
| Response Indication | 1 | 0: This indicates the request 1: This indicates the response (i.e., acknowledgement) to the request |
| ······ | ······ | ······ |
| else if (SCH sub-type == Sleep Cycle control) { |  |  |
| Next Sleep Cycle Flag (NSCF) | 2 | 0b00 = Reset to Initial Sleep Cycle 0b01 = min (2 x previous sleep cycle, Final Sleep Cycle) 0b10 = Reset to another Initial Sleep Cycle value 0b11 = Reserved |
| if (NSCF == 0b10) { |  |  |
| New Initial Sleep Cycle | 6 | When the current Sleep Cycle is reset, if this value is included, the current Sleep Cycle shall be reset to this value. |
| } |  |  |
| } |  |  |
| ······ | ······ | ······ |

---------------------------------------------------------End of the Text----------------------------------------------------------